3GPP RAN WG4 Meeting #94bis-e		R4-2004682

Electronic meeting, 20 – 30th April 2020

Agenda item:	4.4.3.2

Source:	Apple Inc.

Title:	Clarification for the definition of the UL duty cycle

WI/SI:	NR_newRAT-Core

Release:	Rel-15

Document for:	Decision

# 1	Introduction

During the RAN4#91 meeting a CR was agreed [1] introducing the concept of the "maximum uplink duty cycle" that enables a UE to indicate its preferred maximum uplink duty cycle to meet the exposure compliance requirements. And the corresponding signaling was introduced by RAN WG2 in [2][3]. It is worth noting that RAN WG2 has introduced maximum UL duty cycle signaling in a form of a static UE capability, which means that a UE sets value not only by accounting for its design constraints and limitation, but also assuming certain "reference" behavior. However, as raised during previous RAN WG4 meetings [4], it is not clear which "reference" transmission power is used while reporting maximum UL duty cycle. Thus, after RAN#94-e the following WG was agreed aiming to clarify it further [5].

In this discussion paper we suggest further clarifying maximum UL duty cycle definition.

# 2	Maximum UL duty cycle definition

Referring to agreed RAN WG2 CRs in [2][3], current definition of the maximum UL duty cycle in TS 38.306 looks as follows.

maxUplinkDutyCycle-FR2

Indicates the maximum percentage of symbols during 1s that can be scheduled for uplink transmission so as to ensure compliance with applicable electromagnetic power density exposure requirements provided by regulatory bodies. This field is applicable for all power classes UE in FR2 as specified in TS38.101-2 [3]. Value n15 corresponds to 15%, value n20 corresponds to 20% and so on. If the field is absent or the percentage of uplink symbols transmitted within any 1s evaluation period is larger than maxUplinkDutyCycle-FR2, the UE behaviour is specified in TS38.101-2 [3].

As already mentioned in the Introduction part, it is not clear which reference transmission power is assumed for the UE to estimate its preferred UL duty cycle. It would be logical to assume worst case scenario that the UE maximum transmission power is used, but the definition does not mention it explicitly. Otherwise, it is not clear how a UE can indicate the preferred UL duty cycle, because the actual transmission power obviously depends on the radio conditions and how many RBs the network scheduler allocates. In fact, using maximum transmission power as a reference point of the maximum UL duty cycle definition can simplify network implementation because the network scheduler will know that applying that duty cycle, irrespective of the actual UE transmission power, can avoid P-MPR. At the same time, it can also help network scheduler to optimize its operation when a UE transmits at a lower transmission power. As an example, if a UE indicates 15% maximum UL duty cycle for the maximum transmission power of e.g. 23dBm, then the network scheduler can consider 30% duty cycle if a UE average transmission power stays at the level of 20dBm. It is of course up to the network implementation whether to use always the same duty cycle or scale it accordingly, but a clear definition of the reference transmission power will benefit both UE design and the network scheduler implementation.

As also discussed during the RAN4#94e meeting, it is necessary to clarify how to define "maximum transmission power". Referring to TS 38.101-2, each UE has associated power class, which in turn governs maximum TRP and EIRP. Thus, it seems quite straightforward to use maximum total radiated power as a reference point for the capability definition.

Proposal 1:	Clarify that the UE maximum total radiated power (as governed by its power class) is assumed for the definition of the Rel-15 maximum UL duty cycle.

Finally, as the maximum UL duty definition is captured in RAN WG2 specifications, we suggest sending LS to introduce the corresponding changes to TS 38.306. Improved definition may look as presented below:

maxUplinkDutyCycle-FR2

Indicates the maximum percentage of symbols during 1s that can be scheduled for uplink transmission at the UE maximum total radiated power so as to ensure compliance with applicable electromagnetic power density exposure requirements provided by regulatory bodies. This field is applicable for all power classes UE in FR2 as specified in TS38.101-2 [3]. Value n15 corresponds to 15%, value n20 corresponds to 20% and so on. If the field is absent or the percentage of uplink symbols transmitted within any 1s evaluation period is larger than maxUplinkDutyCycle-FR2, the UE behaviour is specified in TS38.101-2 [3].

Proposal 2:	Send LS to RAN WG2 asking to introduce further clarifications into the definition of maximum UL duty cycle.

If proposed changes are agreeable, we also ask RAN WG4 to clarify whether we need to change the following paragraph from TS 38.101-2, sub-clause 6.2.4. If a UE transmits at the maximum transmission power and the percentage of allocated uplink symbols within any 1s evaluation period exceeds the signalled value, then a UE can apply P-MPR and that is what it will most likely do. At the same time, a UE transmitting at a lower transmission power can be scheduled more frequently and still can meet the regulatory requirements without applying P-MPR. The paragraph below from TS 38.101-2 may give a misleading impression that a UE will always apply P-MPR.

If the field of UE capability maxUplinkDutyCycle-FR2 is present and the percentage of uplink symbols transmitted within any 1 s evaluation period is larger than maxUplinkDutyCycle-FR2, the UE follows the uplink scheduling and can apply P-MPRf,c.

# 3	Conclusions

In this discussion paper we have presented our concerns for the current definition of the maximum UL duty cycle. As it is not clear which reference transmission power is assumed for reporting the maximum cycle value, it is somewhat ambiguous for both UE and network side how it can be used in the most efficient way.  Thus, we suggest clarifying that the UE maximum transmission power is used a reference point.

Proposal 1:	Clarify that the UE maximum total radiated power (as governed by its power class) is assumed for the definition of the Rel-15 maximum UL duty cycle.

Proposal 2:	Send LS to RAN WG2 asking to introduce further clarifications into the definition of maximum UL duty cycle.

# 4	References

- R4-1907454, "CR to TS38.101-2 on FR2 PC3 UE maxUplinkDutyCycle", Nokia, Nokia Shanghai Bell

- R2-1908488, "CR on capability of maxUplinkDutyCycle for FR2", CMCC, OPPO, Apple

- R2-1908489, "CR on capability of maxUplinkDutyCycle for FR2", CMCC, OPPO, Apple

- R4-2000005, "Clarification for the definition of the UL duty cycle", Apple Inc

- R4-2002733, "WF on Max duty cycle clarifications", Apple Inc