3GPP TSG-RAN4 WG4 Meeting # 94b-e	R4-2004764

# Electronic meeting, April 20- 30, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0141 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for modified MPR on NS_201 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | AMPR for NS_201 is modified in TS 38.101 f70 compared with previous version. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding modifed MPR/AMPR bit for NS_201 AMPR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs support the modified AMPR for NS_201 can not indicate to the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR’s revision history: |  |
| --- | --- |

<Start of change>

Annex H (normative): Modified MPR behavior

H.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPRbehavior indicated in the IE RF-Parameters [13] by a UE supporting an MPR or A-MPR modified in a later release of this specification. modifiedMPRbehavior is indicated by an 8-bit bitmap per NR band.

Table H.1-1: Definitions of the bits in the field modifiedMPRbehavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n258 | 0 (leftmost bit) | - AMPR for NS_201 as defined in clausue 6.2.3.2 of 38.101-2 v15.7.0 | - This bit may be set to 1 by a UE supporting n258 |

<End of change>