3GPP TSG-RAN WG4 Meeting #94-e-Bis                               R4-2004818

Electronic Meeting, 20 – 30 April, 2020

Source: 	Skyworks Solutions, Inc.

Title: 	n1 50MHz REFSENS and Band n34 Protection Measurements

Agenda Item:	8.17.1	UE RF (38.101-1)	[NR_n1_BW2-Core]

Document for:	Discussion

# Introduction

At RAN4 #94-e, WF [1] agreed on n1 50MHz REFSENS values and invited companies to present A-MPR results. This contribution presents MSD measurements that confirm the validity of the 50MHz n1 REFSENS levels and a limited Power Amplifier (PA) back-off levels required to protect band 34. Measurements could not be completed prior to deadline and will be uploaded as soon as possible.

# Experimental Measurements

## Power Amplifier Calibration and Test Waveforms

A band 1 power amplifier (PA) is calibrated to meet 1 dB MPR using a 100 RB QPSK DFT-s-OFDM SCS 15kHz with RBs allocated at the channel edge.

- Image and Local Oscillator (LO) rejection is set to -28 dBc;

- Counter-IM3 (C-IM3) set to -60 dBc;

- Counter-IM5 (C-IM5) set to -70 dBc.

## MSD Measurement Results

Tx noise in Rx band power level measurements are performed with a QPSK DFT-s-OFDM SCS 15 kHz, 128RB waveform for which the allocated Resource Blocks (RBs) are located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth.

The measured TX noise level in RX band are -48.4 dBm.

# MSD and Reference Sensitivity Estimation

We assume:

- 50 dB Tx to RX and Tx to Antenna duplexer rejection in Rx band, as a worst case assumption;

- 10 dB Primary antenna to diversity antenna isolation;

- 4 dB Post PA loss;

- 4 dB Antenna to LNA front-end losses;

- 3 dB Antenna diversity gain;

- -1 dB SNR; and

- Previously agreed rules to compute correlated and uncorrelated MRC contributions.

Figure 1 below shows summarizes the MSD analysis and shows that the n1 MSD levels for 50MHz operation is approximately 0.5dB.

Figure 1: n1 MSD analysis for 50 MHz CBW

Considering the less than 0.5 dB MSD levels, we confirm the agreed REFSENS levels for 50MHz in WF [1].

Observation 1:

Less than 0.5 dB desensitization is measured for n1 operation and 50MHz channel bandwidth, thereby confirming values proposed in [1].

# Band 34 Protection PA Back-off Levels

For band 34 protection, we assume a worst case filter rejection of 11dB.

Measurements are performed with the following RF parameters:

A band 1 power amplifier (PA) is calibrated to meet 1 dB MPR using a 100 RB QPSK DFT-s-OFDM SCS 15kHz with RBs allocated at channel edge.

- Image and Local Oscillator (LO) rejection is set to -28 dBc;

- Counter-IM3 (C-IM3) set to -60 dBc;

- Counter-IM5 (C-IM5) set to -70 dBc.

Measurement results can be found in Table 1 below.

Table 1: Measured PC3 back-off levels for band 34 protection at 50MHz CBW.

| RB length | RB start | PC3 back-off [dB] |
| --- | --- | --- |
| 1 | 0 | [tbc] |
| 1 | 6 | [tbc] |
| 1 | 18 | [tbc] |
| 1 | 30 | [tbc] |
| 1 | 42 | [tbc] |
| 1 | 54 | [tbc] |
| 1 | 66 | [tbc] |
| 1 | 78 | [tbc] |
| 1 | 201 | [tbc] |
| 1 | 218 | [tbc] |
| 1 | 235 | [tbc] |
| 1 | 252 | [tbc] |
| 1 | 269 | [tbc] |
| 270 | 0 | [tbc] |
| 200 | 0 | [tbc] |
| 150 | 0 | [tbc] |
| 100 | 0 | [tbc] |
| 50 | 0 | [tbc] |
| 200 | 70 | [tbc] |
| 150 | 120 | [tbc] |
| 100 | 170 | [tbc] |
| 50 | 220 | [tbc] |

The intention here is to provide reference measurements points to calibrate simulation results. The highest observed raw back-off level is approximately xx dB for PC3 operation in n1 50MHz CBW.

# Conclusion

In this contribution we confirm no MSD is needed for n1 operation at 50MHz Channel Bandwidth. We also provide raw PA back-off level measurements for power class 3 operation to protect band 34 for n1 50MHz CBW.

Observation 1:

Less than 0.5 dB desensitization is measured for n1 operation and 50MHz channel bandwidth, thereby confirming values proposed in [1].

# References

[1]		R4-2002845, WF on UE RF requirements for adding 50 MHz channel BW to band n1, Huawei.