3GPP TSG-RAN WG4 Meeting #94bis-e		            R4-2004915

Electrical Meeting, 20th April – 30th April, 2020

Agenda item:	6.17.2.1.3

Source:	Samsung

Title:	Simulation results for UE demodulation requirements under HST single tap

Document for:	Discussion

# Introduction

During RAN4#93 meeting, way forward [1] for NR Rel-16 UE demodulation was approved.

In this contribution, we provide the updated simulation results for HST -single tap channels based on the assumptions agreed in RAN4#93 found in [1] for further alignment.

# Common test setup

Table1: Simulation assumption for HST-single tap channel

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 1×2; 1×4 (baseline) 2×2; 2×4 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | DMRS 1+1+1 |  |
| TDD pattern | 7D1S2U, S: 6D 4G 4U |  |
| MCS | MCS 13; MCS 17 based on 64QAM table |  |
| Propagation condition | HST-single tap |  |
| TRS periodicity | 10ms, 2slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds=300m, Dmin=2m |  |
| Rank | Rank =1 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | - Option 1: 1250Hz - Option 2: 875Hz | 1667Hz |
| Testing metric | SNR @70% of maximum throughput |  |

# Simulation results

Note: the following simulation results are based on the sample changed HST-single tap channel.

## FDD Simulation results

Figure 3-1:FDD Simulation results for NR PDSCH under HST single tap for 1x2 and 1x4 antenna configuation

As indicated in figures, the maximum throughput can be not achieved with maximum doppler value configured 1250Hz.  Meanwhile,  based on BS side , 1740Hz is agreed with requirement for 15KHz SCS.  In that sense, we slightly prefer to align with BS side.

Observation 1: The maximum throughput can not be achieved with Doppler value with 1250Hz

Proposal 1: Define the requirement wth 870Hz as requirement of single tap with 15KHz.

## TDD Simulation results

Figure 3-2:TDD Simulation results for NR PDSCH under HST- single tap with 1x2 and 1x4

## Summary of @ 70% of max throughput

Table 1: Ideal and impairment simulation results for NR PDSCH under HST- single tap channel with 1x2 and 1x4

| Case Number | CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | MCS | SNR（dB）@70% Max TP (ideal) | SNR（dB）@70% Max TP (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10MHz/15kHz | 1250 | 1Tx 2Rx ULA Low | 13 | 3.9 | 6.4 |
| 2 |  |  |  | 17 | 7.7 | 10.2 |
| 3 |  |  | 1Tx 4Rx ULA Low | 13 | 0.8 | 3.3 |
| 4 |  |  |  | 17 | 4.7 | 7.2 |
| 5 |  | 875 | 1Tx 2Rx ULA Low | 13 | 3.6 | 6.1 |
| 6 |  |  |  | 17 | 7.2 | 7.7 |
| 7 |  |  | 1Tx 4Rx ULA Low | 13 | 0.6 | 3.1 |
| 8 |  |  |  | 17 | 4.1 | 6.6 |
| 9 | 40MHz/30kHz | 1667 | 1Tx 2Rx ULA Low | 13 | 3.5 | 6.0 |
| 10 |  |  |  | 17 | 6.6 | 9.1 |
| 11 |  |  | 1Tx 4Rx ULA Low | 13 | 0.6 | 3.1 |
| 12 |  |  |  | 17 | 3.7 | 6.2 |

# Conclusion

In this contribution we provide simulation results for HST- single tap channels.

Observation 1: The maximum throughput can not be achieved with Doppler value with 1250Hz

Proposal 1: Define the requirement wth 870Hz as requirement of single tap with 15KHz.

# Reference

- R4-1915926 , Draft WF on UE demodulation for NR HST, RAN4#93, CMCC

- R4-1912808, WF on UE demodulation for NR HST, RAN4#92Bis, CMCC

- R4-1913374, Simulation results for UE demodulation requirements under HST single tap, Samsung