3GPP TSG-RAN WG4 Meeting #94bis-e                                                                                     R4-2004917

Electrical Meeting, 20th April – 30th April, 2020

Source:	Samsung

Title:	Simulation results for UE demodulation requirements for TDD with special slot

Agenda item:			6.17.2.1.4

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the test parameters for requirement of HST single tap, HST SFN and multi-path fading channel was discussed [1]. RAN4 introduced the NR HST requirements for both FDD and TDD. Regarding the scheduling in TDD special slots, the following options are considered as follows:

- Scheduling in TDD special slot for multi-path fading - Option 1: For PDSCH with TDD configuration, do not schedule data in the special slot in order to achieve maximum throughput - Option 2: Do not schedule data on special slot in HST fading scenario and also check same issue for HST-SFN scenario - Option 3: Align with normal PDSCH assumption in Rel-16 - Option 4: Further simulation is needed - Option 5: Further check this issue for multi-path fading channel HST-SFN scenario

In this contribution, the simulation results are provided for investigating the impact of TDD special slot.

# 2. Common test setup

Table 1: Simulation assumption for multi-path fading channel

| Parameter | Value |  |
| --- | --- | --- |
|  | TDD 30KHz SCS |  |
| Antenna configuration | 2×2; |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | DMRS 1+1+1 |  |
| TDD pattern | 7D1S2U, S: 6D 4G 4U | 7D3U, S: 6D 4G 4U |
| MCS | MCS 4; MCS 13; MCS 17 based on 64QAM table |  |
| Propagation condition | TDL-C 300ns |  |
| TRS periodicity | 10ms, 2slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Rank | Rank = 1 for TDLC and Rank = 2 for HST-SFN |  |
| BW | 40MHz |  |
| Maximum Doppler shift | 1200Hz for TDLC and 1667Hz for HST-SFN |  |

Table 2: Simulation assumption for HST-SFN

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 2×2; 2×4 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | DMRS 1+1+1 |  |
| TDD pattern | 7D1S2U, S: 6D 4G 4U |  |
| MCS | MCS 13 based on 64QAM table |  |
| Propagation condition | HST-SFN |  |
| TRS periodicity | 10ms, 2slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds=700m, Dmin=150m |  |
| Rank | Rank = 2 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | - Option 1: 870Hz - Option 2: 851Hz | - Option 1: 1500Hz - Option 2: 1667Hz |
| Testing metric | SNR @70% of maximum throughput |  |

# 3. Simulation results

In this subsection, the simulation results are provided with investigation the impact on special slot in TDD.

3. 1 Multi-path fading channel

Figure 3-1:     Simulation results for NR PDSCH TDD under multi-path fading TDLC channel

3. 2 HST SFN

Figure 3-2:     Simulation results for NR PDSCH TDD under HST-SFN channel

As observation in Figure 1 and Figure 2, the performance difference between with and without scheduling in TDD special slot is minor. In case of special slot scheduling, only 1 DMRS symbol is available with 6G4S4U in special slot for TDD pattern 7D1S2U, the performance will be suffered degradation with insufficient channel estimation performance. From the target SNR with 70%TP perspective, the results show there is no significant different for fading channel for two different scheduling schemes.

Regarding HST SFN scenario, the performance can be improved about 0.5dB without scheduling TDD special slot for targeting SNR with 70% TP with large value of Doppler.

Observation 1: No obvious difference for multi-path fading channel between with and without scheduling TDD special slot is observed.

Observation 2: Without scheduling TDD special slot, the performance can be improved around 0.5dB for HST-SFN scenario with large value of Doppler

Based on the initial investigation, considering the performance difference is minor. Meanwhile, in order to provide the comparable requirement with Rel-15 UE for fading scenario and HST single tap scenario, we slightly prefer to keep align Rel-15 UE demodulation simulation, with scheduling TDD special slot.

Proposal 1: In order to provide the comparable requirement with Rel-15 UE demodulation requirement for fading scenario, keep align with Rel-15 UE demodulation assumption to schedule TDD special slot.

# 4. Conclusion

In this contribution, the simulation results are provided to investigate the performance impact on scheduling in TDD special slot for multi-path fading and HST-SFN scenario.

Observation 1: No obvious difference for multi-path fading channel between with and without scheduling TDD special slot is observed.

Observation 2: Without scheduling TDD special slot, the performance can be improved around 0.5dB for HST-SFN scenario with large value of Doppler

Proposal 1:  In order to provide the comparable requirement with Rel-15 UE demodulation requirement for fading scenario, keep align with Rel-15 UE demodulation assumption to schedule TDD special slot.

# 4. Reference

[1] R4-2002418, WF on Rel-16 NR HST UE demodulation. CMCC