3GPP TSG-RAN WG4 Meeting #94bis-e              	R4-2004919

Electronic meeting, 20th April– 30th April, 2020

Agenda item:	6.9.1.3

Source:	Samsung

Title:	View on BS demodulation requirement for URLLC in NR Rel-16

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, the general issue and test setup for test feasibility and performance requirements for URLLS are further discussed and the related agreements are captured in WF [1] and WF [2], separately

In this contribution, the view on remain issue of URLLC performance requirements is presented.

# 2	Discussion and initial results

## 2.1	URLLC test methodology

TDD pattern

In Rel-15 NR demodulation requirement, RAN4 has defined the TDD pattern for FR1 with 3D1S1U for 15KHz SCS, 7D1S2U for 30KHz SCS. The requirement are applicable for different TDD patterns.  In that sense, we prefer to reuse the TDD pattern defined in Rel-15 for URLLC test.

Proposal 1: Reusing the TDD pattern 3D1S1U for 15 KHz SCS and 7D1S2U for 30 KHz SCS for URLLC test.

PT-RS and DM-RS configuration for PUSCH

With regarding the DM-RS configuration, the following options are considered:

- Option 1: 1+0 - Option 2: 1+1 - Option 3: 1+0 and 1+1 with applicability rule

As agreed, the PUSCH low BLER high confidence requirements is introduced with static channel. Since there is no fading, either configured 1 DMRS or 2 DMRS can meet the requirement. In order to provide the comparable performance with Rel-15 BS demodulation requirement, we slightly prefer to introduce 2 DMRS configuration for PUSCH low BLER confidence test to align existing PUSCH requirement for eMBB scenario.

Proposal 2:  Introduce 2 DMRS configuration for PUSCH low BLER high confidence test

Bandwidth for PUSCH

With regarding the Bandwidth for PUSCH, the following options are considered:

- Option 1: 5MHz for 15K SCS, 10MHz for 30K SCS - Option 2: 10MHz for 15K SCS, 40MHz for 30K SCS - Option 3: 5/10/20 MHz for 15 KHz SCS, 10/20/40/100 MHz for 30 KHz SCS, applicability rule to ensure just one test - Option 4: option 1+ option 2 - Other options not precluded.

Based on the high reliability definition, the BLER should be lower than 99.999% for one transmission with 32 byte, to meet 1ms delay. In that sense, the URLLC should target with small data scenario.  We do not think larger channel bandwidth is the typical scenario. Either option 1 or option 2 can meet with small data requirement.

Considering RAN4 has already defined the applicability rule, introducing the minimum channel bandwidth per each SCS is preferred.

Regarding with number of RB, full bandwidth is preferred

Proposal 3: Introduce 5MHz for 15K SCS, 10MHz for 30K SCS with full bandwidth for PUSCH with lower BLER high reliability test.

Test requirement

In eMBB scenario, we have similar discussed whether considering additional margin for PUSCH FR2 without considering the impact of phase noise. Eventually, RAN4 agreed this additional margin can be considered as in the part of IM, depended on implementation. In that sense, we slightly prefer to apply the similar approach.

Proposal 4: Introduce additional margin X with part of IM implicitly for URLLC test requirement, not to specify the value into conformance and core spec.

## 2.2	Requirements for high reliability with higher BLER

PUSCH aggregation level

With regarding the PUSCH aggregation level, the following options are considered:

- Option 1: 2 - Option 2: 4 - Option 3: 2,4 - Option 4: 8

In order to improve reliability, PUSCH based on slot repetitions is the important feature for URLLC. Based on RAN1 design, the number of repetitions can be supported if UE is configured with pusch-AggregationFactor as follows, where the supported aggregation factor is {2, 4, and 8}

Considering the number of HARQ transmission is agreed with 4, based on the following RV configuration for repetitions

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

As indicated with the configuration, the different RV can be covered with 2 PUSCH aggregable level with HARQ.

Additionally, due to the unavailable UL slots with TDD, the transmission delay will be increasing in order to complete one transmission with large number of PUSCH aggregation level. When the channel condition is worse, the process delay will be more serious with HARQ transmission. Therefore, we prefer to only introduce the requirement with 2 slots aggregation to reduce the test complexity and transmission delay.

Proposal 5: Only introduce the requirement for high reliability with higher BLER with 2 slots aggregation level.

DFT-s-OFDM requirement

In Rel-15 NR BS demod, both PUSCH requirement with CP-OFDM and DFT-s-OFDM waveform are introduced. In terms of performance requirement, the difference is minor. Therefore, we prefer to focus on the high reliability with high BLER requirement with CP-OFDM waveform firstly

Proposal 6: Deprioritize the high reliability requirement with DFT-s-OFDM waveform.

## 2.3	Requirements for low latency

In the previous meeting, it is agreed to introduce PUSCH demodulation requirements to verify the support of PUSCH mapping type B with non-slot configured with fewer symbols than Rel-15.

For Rel-15 NR, The performance requirement for both slot based and not slot based PUSCH with type A and type B have been defined. To verify the latency feature, mini slot based transmission can be regarded to one solution to meeting latency targeting, such as 2, 4 or 7 symbols.

With regarding the symbol length, the following options are considered:

- Option 1: 4os - Option 2: 2os - Option 3: 7os - Option 4: 2os and 7os

Generally, all the options can meet latency requirement. In terms of requirement, we think RAN4 should focus on the typical scenario with possible network scheduling.

With 2OS symbol length, only 1 symbol is available for PUSCH data transmission based on Rel-15 BS demod simulation assumption. As agreed, the RAN4 will introduce the URLLC requirement with SCS/BW as 15 KHz/10MHz and 30 KHz/40MHz. The payload with 1 symbol is smaller than typical URLLC scenario, at last 32 bytes.

With 7OS symbol length, in current Rel-15 BS demod requirement, RAN4 has already defined with 10 symbols requirement with type B. In terms for performance, we do not think the performance will be too much different. To reduce the test effort, we slightly prefer to define the requirement with less than 7 OS.

Meanwhile, in current Rel-16 URLLC feature with mini-slot repetition, different options based on 4 OS mini-slot repetition to investigate the scheme with supporting dynamic switch between mini-slot repetition and multi-segments as follow:

Observation 1:  Mini-slot repetition with 4OS is the typical scenario in RAN1 discussion to supporting dynamic switch between mini-slot repetition and multi-segments

Therefore, we prefer to align with RAN1 Rel-16 URLLC discussion with 4 OS.

Proposal 7:  Non-slot scheduling with 4 symbols can be considered for the lower latency requirement.

DFT-s-OFDM

In Rel-15 NR BS demod, both PUSCH requirement with CP-OFDM and DFT-s-OFDM waveform were introduced. In terms of performance requirement, the different is minor. Since the motivation is to define the low latency requirement, from the complexity and implementation respective, additional FFT operation will result in the increasing of process delay. Therefore, we prefer to focus on the latency requirement with CP-OFDM waveform firstly.

Proposal 8: Deprioritize the latency requirement with DFT-s-OFDM waveform for NR URLLC

Test metric

With regarding the test metric, the following options are considered:

- Option 1: 70% throughput - Option 2: 10% BLER (=90% throughput)

As test metric, it is agreed that test metric of delay is not feasible for low latency test in RAN4. And low latency related features are feasible to be tested under test metrics other than delay, such as through and/or BLER.  Meanwhile, no combined performance requirements will be defined by meeting both reliability and low latency requirements. Since low BLER metric is already used for requirement of high reliability. We prefer to reuse normal PUSCH test metric with 70% TP to compact on the test.

Proposal 9: Reuse the normal PUSCH test metric with 70% throughput for URLLC latency requirement

F2 requirement

Normally, with 30KHz SCS and higher, mini-slot HARQ based retransmission can fulfill latency target. From the demodulation requirement perspective, the performance different with FR1 and FR2 is minor as observed in Rel-15 NR BS demodulation. To reduce the test effort, we slightly prefer to only define requirement with FR1. Meanwhile, if RAN4 agreed to introduce the FR2 requirement for URLLC low latency, we prefer to define the test applicability for FR1 and FR2 requirement, only one of them chosen to test.

Proposal 10: Deprioritize FR2 requirement for URLLC low latency requirement, if RAN4 agreed to introduce FR2 requirement for URLLC low latency, the test applicability for FR1 and FR2 requirement should be defined with only one of them chosen to test with BS declaration.

## 2.4	Initial results

In this subsection, the initial simulation results are provided for investigating URLLC requirement with high reliability and low latency.

| Parameter | value |
| --- | --- |
|  | FR1 |
| Transform precoding | Disabled |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) |
| DMRS type | type 1 |
| Number of DMRS symbols | 1(0), 1+1(0,4) |
| symbols length | 2, 4, 7 |
| Start symbol index | 0 |
| Time domain resource allocation type | type B |
| Frequency domain resource | Full RB allocation of the applicable BW |
| MCS index | 5 with Table 3 |
| Carrier frequency (GHz) | 4 |
| Propagation channel | AWGN/ TDLB100-400 |
| SCS and BW | 15KHz/10MHz, 5MHz, 30KHz/40 MHz |
| Aggregation level | 1, 2, 4,8 |
| Timing offset | 0 |
| Frequency offset | 0 |
| PTRS | Without |
| Code block group | Disabled |
| Frequency hopping | Disabled |
| Limited buffer rate matching | Disabled |
| Number of HARQ transmission | 4 |

Ultra-lower BLER test for 10^-5

| Test case | SCS,BW | MCS(new table) | Aggregation level | Tx, Rx | Number DMRS | channel | SNR for 10^-5 BLER |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case1 | 15KHz, 5MHz | 5 | 1 | 1T2R | 1(2) | AWGN | -6.6 |
| Case2 | 15KHz, 10MHz | 5 | 1 | 1T2R | 1(2) | AWGN | -7.28 |
| Case3 | 15KHz, 5MHz | 5 | 1 | 1T2R | 1+1 (2,11) | AWGN | -7.58 |
| Case4 | 15KHz, 10MHz | 5 | 1 | 1T2R | 1+1 (2,11) | AWGN | -8.06 |
| Case5 | 15KHz, 5MHz | 5 | 2 | 1T2R | 1(2) | AWGN | -8.82 |
| Case6 | 15KHz, 10MHz | 5 | 2 | 1T2R | 1(2) | AWGN | -9.74 |
| Case7 | 15KHz, 5MHz | 5 | 2 | 1T2R | 1+1 (2,11) | AWGN | -10.36 |
| Case8 | 15KHz, 10MHz | 5 | 2 | 1T2R | 1+1 (2,11) | AWGN | -10.26 |

As indicated, with 2 DMRS configuration, the performance can be expected about 1dB improvement. And around 2dB improvement can be expected with 2 aggregation level.

High reliability with higher BLER

Figure 1 BLER performance with different aggregation level for 15 KHz and 10 MHz

Figure 2 BLER performance with different aggregation level for 30 KHz and 40 MHz

As observed in Figure 1 and Figure 2, PUSCH aggregation level with 2 combined HARQ transmission can achieve the similar performance with 8 PUSCH aggregation level without combined HARQ transmission, and better than 4 aggregation level.

Observation 2: With 2 PUSCH aggregation level combined HARQ transmission can achieve the comparable performance with 8 PUSCH aggregation level without combined HARQ transmission, and better than 4 aggregation level

Low latency requirement

Observation 3: From the target SNR value with 70% TP perspective, there is no significant difference with configured 2, 4 and 7 OS for PUSCH mini-slot transmission

# 3 Conclusion

In this contribution, the initial results are provided to analyze the URLLC with high reliability requirement.

Proposal 1: Reusing the TDD pattern 3D1S1U for 15 KHz SCS and 7D1S2U for 30 KHz SCS for URLLC test.

Proposal 2:  Introduce 2 DMRS configuration for PUSCH low BLER high confidence test

Proposal 3: Introduce 5MHz for 15K SCS, 10MHz for 30K SCS with full bandwidth for PUSCH with lower BLER high reliability test.

Proposal 4: Introduce additional margin X with part of IM implicitly for URLLC test requirement, not to specify the value into conformance and core spec.

Proposal 5: Only introduce the requirement for high reliability with higher BLER with 2 slots aggregation level.

Proposal 6: Deprioritize the high reliability requirement with DFT-s-OFDM waveform.

Observation 1:  Mini-slot repetition with 4OS is the typical scenario in RAN1 discussion to supporting dynamic switch between mini-slot repetition and multi-segments

Proposal 7:  Non-slot scheduling with 4 symbols can be considered for the lower latency requirement.

Proposal 8: Deprioritize the latency requirement with DFT-s-OFDM waveform for NR URLLC

Proposal 9: Reuse the normal PUSCH test metric with 70% throughput for URLLC latency requirement

Proposal 10: Deprioritize FR2 requirement for URLLC low latency requirement, if RAN4 agreed to introduce FR2 requirement for URLLC low latency, the test applicability for FR1 and FR2 requirement should be defined with only one of them chosen to test with BS declaration.

Observation 2: With 2 PUSCH aggregation level combined HARQ transmission can achieve the comparable performance with 8 PUSCH aggregation level without combined HARQ transmission, and better than 4 aggregation level.

Observation 3: From the target SNR value with 70%TP perspective, there is no significant difference with configured 2, 4 and 7 OS for PUSCH mini-slot transmission

# References

R4-2002422, Way Forward on parameters and test methodology for URLLC ultra-low BLER test, Ericsson

R4-2002429, Way Forward on NR BS  URLLC performance requirements, Huawei