3GPP TSG-RAN WG4 Meeting #94bis-e                                                                    R4-20004923

Electrical Meeting, 20th April – 30th April, 2020

Source:	Samsung

Title:	View on performance requirements for Rel-16 DMRS enhancement

Agenda item:			6.11.3.2

Document for:	Approval

- Introduction

Rel-16 NR eMIMO WI is a RAN1 leading WI with below major enhancement in RAN1 area:

- Enhancements on MU-MIMO support

- Enhancements on multi-TRP/panel

- Enhancements on multi-beam operation

  - L1-SINR measurement

  - Beam failure recovery for SCell

  - DL/UL beam indication with reduced latency and overhead

- Enhancement on low PAPR RS

- Enhancement on full Tx power uplink transmission

During the last RAN4 meeting, the general test scope for demodulation requirement of NR eMIMO was discussed and the related agreement was captured in WF [1]. Regarding with enhancement on low PAPR RS, whether to define the related demodulation requirement is still FFS.

In this contribution, the review on performance requirements impact for Low PAPR RS was provided.

# Discussion

Low PAPR RS

In Rel-16, in order to reduce the PAPR for DMRS, DMRS enhancement with new sequence generation was introduced for PDSCH/PUSCH using CP-OFDM waveform as follows:

Compared with Rel-15 DMRS generation formation, the CDM group index λ and n’SCID were introduced with differentiating different DMRS port as follows

With this new sequence, the DMRS PAPR can be reduced to same level as data symbols for all port combination for CP-OFDM with different DMRS sequence for different CDM groups.

In Rel-15 NR PDSCH demodulation requirement, both 2 DMRS ports and 4 DMRS ports are considered for type 1 DMRS configuration. In case of 4 DMRS ports, with changed the sequence generation for two CDM groups, the PAPR can be reduced. The proper UE receiver may need to be verified. While from performance requirements perspective, there is no SNR changed. Therefore, if needed, existing UE performance test cases with 4 ports can be reused or replaced with Rel-16 DMRS sequence configuration without requirements and other test parameters modification.

Observation 1: If needed, existing UE performance test cases with 4 ports can be reused or replaced with Rel-16 DMRS configuration without requirements and other test parameters modification for Rel-16 DMRS functionality verification

Regarding with 2 DMRS ports, the number of CDM group without data equals to 1, which was taken into account for PDSCH requirement.  Based on the exiting DMRS RE mapping method for 2 DMRS ports belongs CDM group (λ=0), we can observe there is no different with Rel-15 and Rel-16 DMRS sequence.

Due to the number of CDM group without data is 1, changing the DMRS RE mapping method for 2 DMRS ports to CDM group (λ=1) is not supported within Rel-15 PDSCH requirement. To verify the UE receiver processing based on Rel-16 DMRS enhancement for 2 DMRS ports, the test parameter of number of CDM group without data and DMRS RE mapping index should be modified.

Rel-16 NR eMIMO introduces the new PDCCH schemes including TCI state scheduling, DMRS port mapping for multi-TRP/multi-panel transmission. To compact the test case, we prefer to verify the Rel-16 DMRS enhancement feature jointly together with test cases designed for multi-TRP/Panel transmission. No individual test case is defined to verify the Rel-16 DMRS enhancement.

Proposal 1: If needed, verify Rel-16 DMRS enhancement feature jointly together with test cases designed for multi-TRP/Panel transmission. No test case is defined to verify the Rel-16 DMRS enhancement individually.

In Rel-15 NR PUSCH demodulation requirement, RAN4 introduced the requirement with 2 ports with DMRS type A, where DMRS port {0, 1} belongs the same CDM group λ=0. Based on the Rel-16 DMRS enhancement sequence generation, the value of cinit is exactly same with calculating by Rel-15 DMRS generation for given DMRS sequence generation as

- NID0=0, nSCID =0

Observation 2: From receiver performance requirements, there is no different for BS processing with Rel-16 DMRS sequence configured.

Meanwhile, the “Number of DM-RS CDM group(s) without data equals to 2” is taken into accounting for specifying the requirements, which means either {0,1} or {2,3} with different CDM groups can be chosen for DMRS RE mapping. Additionally, 3 dB power boosting is added into the DMRS RE with keeping same level with data, considering there is no FDM operation for DMRS REs and data REs allocated in the DMRS symbols.

If DMRS port mapping index is changed as {2, 3}, the value of cinit can be different. While there is no performance requirements difference with new or Rel-15 DMRS from receiver baseband processing perspective, only changing the test parameter for DMRS sequence generation.

Proposal 2: Not to define new performance requirement for PUSCH with CP-OFDM waveform based on Rel-16 DMRS enhancement

# Simulation results

In this section, the initial simulation results are provided. The simulation assumptions for PDSCH and PDSCH are provided to investigate the performance impact for low PAPR RS as indicated the table 1 and table 2, separately.

PDSCH

Figure 1:  Performance comparing between Rel-15 DMRS and Rel-16 DMRS with 2 ports (case 1 and case 2)

Figure 2:  Performance comparing between Rel-15 DMRS and Rel-16 DMRS with 4 ports (case 3 and case 4)

PUSCH

Figure 3:  Performance comparing between Rel-15 DMRS and Rel-16 DMRS with 1 ports

Observation 3:  About 1 dB can be improved with configured number of CDM group as 2.

Observation 4:  From performance requirement perspective, there is no different for PDSCH configured with Rel-15 DMRS or Rel-16 DMRS sequence.

Observation 5:  From the performance requirement perspective,   there is no different for PUSCH with CP-OFDM waveform configured with Rel-15 DMRS or Rel-16 DMRS sequence.

# 3. Conclusion

In this contribution, the view on performance requirements for Rel-16 DMRS enhancement has been analyzed. Meanwhile, the initial results are provided to

Observation 1: If needed, existing UE performance test cases with 4 ports can be reused or replaced with Rel-16 DMRS configuration without requirements and other test parameters modification for Rel-16 DMRS functionality verification

Proposal 1: If needed, verify Rel-16 DMRS enhancement feature jointly together with test cases designed for multi-TRP/Panel transmission. No test case is defined to verify the Rel-16 DMRS enhancement individually.

Proposal 2: Not to define new performance requirement for PUSCH with CP-OFDM waveform based on Rel-16 DMRS enhancement

Observation 2: From receiver performance requirements, there is no different for BS processing with Rel-16 DMRS sequence configured.

Observation 3:  About 1 dB can be improved with configured number of CDM group without data as 2.

Observation 4:  From performance requirement perspective, there is no different for PDSCH configured with Rel-15 DMRS or Rel-16 DMRS sequence.

Observation 5:  From the performance requirement perspective,   there is no different for PUSCH with CP-OFDM waveform configured with Rel-15 DMRS or Rel-16 DMRS sequence.

# 4. Reference

[1] R4-2002419, WF on demodulation and CSI requirement for NR eMIMO. Samsung

# 4. Annex

PDSCH simulation assumption

Table 1:  Test case for PDSCH requirement comparing Rel-15/Rel-16 DMRS enhancement

| Case Number | CHBW/ SCS | MIMO | MCS | Number of layer | DMRS index | Channel Model | Receiver |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10MHz/15kHz | 2Tx 2Rx ULA Low | 16QAM MCS 13 | Layer 2 | {1000, 1001} For rank 2 test Number of PDSCH DMRS CDM group(s) without data =1 Rel-15 DMRS | TDL-A 30ns, 10Hz | MMSE-IRC |
| 2 | 10MHz/15kHz | 2Tx 2Rx ULA Low | 16QAM MCS 13 | Layer 2 | {1002, 1003} For rank 2 test Number of PDSCH DMRS CDM group(s) without data =2 Rel-16 DMRS | TDL-A 30ns, 10Hz | MMSE-IRC |
| 3 | 10MHz/15kHz | 4Tx 4Rx ULA Low | 16QAM MCS 13 | Layer 4 | {1000~1003} For rank 4 test Number of PDSCH DMRS CDM group(s) without data =2 Rel-15 DMRS | TDL-A 30ns, 10Hz | MMSE-IRC |
| 4 | 10MHz/15kHz | 4Tx 4Rx ULA Low | 16QAM MCS 13 | Layer 4 | {1000~1003} For rank 4 test Number of PDSCH DMRS CDM group(s) without data =2 Rel-16 DMRS | TDL-A 30ns, 10Hz | MMSE-IRC |

Table 2:  Simulation assumption for PDSCH requirements with CP-OFDM waveform

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Test 1-1 2 for other tests |
|  | Resource allocation type |  | Test 1-2: Type 1 with start RB = 23, LRBs = 6 Other tests: Type 0 |
|  | RBG size |  | Test 1-2: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
|  | Antenna ports indexes |  | {1000, 1001} or [{1002, 1003}] for Rank 2 tests {1000-1003} for Rank 4 tests |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for Rank 2 tests for case 1 [2 for Rank 2 tests for case 2] |
|  | DMRS sequence generation |  | NID=0, nSCID=0 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | Test 1-5: 10 for CSI-RS resource 1,2,3,4. Other tests: Table 5.2-1. |
|  | CSI-RS offset | Slots | Test 1-5: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. Other tests: Table 5.2-1. |
| Number of HARQ Processes |  |  | 8 for Test 1-4 4 for other tests |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 3:  FRC table for PDSCH simulation

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  | Case1 | Case 2 | Case 3 | Case 4 |
| Channel bandwidth | MHz |  | 10 | 10 | 10 | 10 |
| Subcarrier spacing | kHz |  | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs |  | 52 | 52 | 52 | 52 |
| Number of consecutive PDSCH symbols |  |  | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots |  | 19 | 19 | 19 | 19 |
| MCS table |  |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  |  | 13 | 13 | 13 | 13 |
| Modulation |  |  | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate |  |  | 0.48 | 0.48 | 0.48 | 0.48 |
| Number of MIMO layers |  |  | 2 | 2 | 4 | 4 |
| Number of DMRS rEs |  |  | 12 | 12 | 24 | 24 |
| Overhead for TBS determination |  |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits |  | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits |  | 26120 | 24072 | 48168 | 48168 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits |  | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits |  | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs |  | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | CBs |  | 4 | 4 | 6 | 6 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits |  | N/A | N/A | N/A | N/A |
| For Slots i = 10, 11 | Bits |  | 52416 | 47424 | 94848 | 94848 |
| For Slots i = 1,…, 9, 12, …, 19 | Bits |  | 54912 | 49920 | 99840 | 99840 |
| Max. Throughput averaged over 2 frames | Mbps |  | 24.814 | 22.868 | 45.760 | 45.760 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

PUSCH simulation assumption

Table 4: simulation assumption for PUSCH requirements with CP-OFDM waveform

| Parameter | value |
| --- | --- |
|  | FR1 |
| Transform precoding | CP-OFDM |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) |
| DMRS type | type 1 |
| DMRS port | {0} or {2} |
| DMRS sequence generation | NID=0, nSCID=0 |
| Number of DMRS symbols | 1+1 (2,11) |
| symbols length | 14 |
| Start symbol index | 0 |
| Time domain resource allocation type | type A |
| Frequency domain resource | Full RB allocation of the applicable BW |
| MCS index | MCS =2,16 |
| Carrier frequency (GHz) | 4GHz |
| Propagation channel | MCS 2 TDLB100-400 MCS 16 TDLC300-100 |
| SCS and BW | 15KHz, 10M for CP-OFDM |
| Timing offset | 0 |
| Frequency offset | 0 |
| PTRS | Without PT-RS configuration |
| Code block group | Disabled |
| Frequency hopping | Disabled |
| Limited buffer rate matching | Disabled |
| Number of HARQ transmission | 4 |