3GPP RAN WG4 Meeting #94bis-e                                                                  R4-2004984

# Electronic Meeting, 20 – 30 Apr., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | Rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR of NR V2X measurement accuracy requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add 10.3 V2X measurements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 10.3 V2X measurements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Miss NR V2X measurement accuracy requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

## 10.3	V2X Measurements

### 10.3.1	Introduction

The requirements in this section are applicable for a UE capable of V2X sidelink communication.

The accuracy requirements in this clause are:

-	applicable for AWGN radio propagation conditions,

-	assume independent interference (noise) at each receiver antenna port.

### 10.3.2	Intra-Frequency PSBCH-RSRP Measurement Accuracy Requirements for FR1

#### 10.3.2.1	Absolute PSBCH-RSRP Accuracy

The requirements for absolute accuracy of PSBCH-RSRP in this clause apply to a V2X synchronization source on the same frequency as that of the own V2X UE performing the measurement in FR1.

The accuracy requirements in Table 10.3.2.1-1 are valid under the following conditions:

-	Demodulation reference signals for PSBCH are transmitted from one port.

-	Conditions defined in clause 7.3E of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	PSBCH-RSRP|dBm according to Annex B.4.2 for a corresponding Band are fulfilled.

Table 10.3.2.1-1: Intra-frequency PSBCH-RSRP absolute accuracy for UE capable of V2X sidelink communication

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR V2X operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm/SCSS-SSB |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSS-SSB = 15kHz | SCSS-SSB = 30kHz | SCSS-SSB = 60kHz |  |  |
| 4.5 | 9 | -6 dB | NR_TDD_FR1_A | [-121] | [-118] | [-115] | N/A | -70 |
|  |  |  | NR_TDD_FR1_J | [-116.5] | [-113.5] | [-110.5] | N/A | -70 |
| 8 | 11 | -6 dB | NR_TDD_FR1_A, NR_TDD_FR1_J | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The parameter Ês/Iot is the Ês/Iot of PSBCH-DMRS. |  |  |  |  |  |  |  |  |

#### 10.3.2.2	Relative PSBCH-RSRP Accuracy

The relative accuracy of PSBCH-RSRP is defined as the PSBCH-RSRP measured from one V2X synchronization source compared to the PSBCH-RSRP measured from another V2X synchronization source on the same frequency in FR1.

The accuracy requirements in Table 10.3.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3E of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	PSBCH-RSRP1,2|dBm according to Annex B.4.2 for a corresponding Band.

Table 10.3.2.2-1: PSBCH-RSRP Intra frequency relative accuracy for UE capable of V2X sidelink communication

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 2,5 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR V2X operating band groups Note 3 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm/SCSS-SSB |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSS-SSB = 15kHz | SCSS-SSB = 30kHz | SCSS-SSB = 60kHz |  |  |
| 2 | 3 | -3 dB | NR_TDD_FR1_A | [-121] | [-118] | [-115] | N/A | -50 |
|  |  |  | NR_TDD_FR1_J | [-116.5] | [-113.5] | [-110.5] | N/A | -50 |
| 3 | 3 | -6 dB | NR_TDD_FR1_A, NR_TDD_FR1_J | N/A | N/A | N/A | N/A | Note 4 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of SyncRef UEs to which the requirement applies. NOTE 3: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 5: The parameter Ês/Iot is the Ês/Iot of PSBCH-DMRS. |  |  |  |  |  |  |  |  |

### 10.3.3	Intra-Frequency PSSCH-RSRP Measurement Accuracy Requirements for FR1

#### 10.3.3.1	Absolute PSSCH-RSRP Accuracy

The requirements for absolute accuracy of PSSCH-RSRP in this clause apply to a UE performing PSSCH-RSRP measurements on the same frequency as used by operating V2X sidelink communication.

The accuracy requirements in Table 10.3.3.1-1 are valid under the following conditions:

-	Demodulation reference signals for PSSCH are transmitted from one port.

-	Conditions defined in clause 7.3E of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	PSSCH-RSRP|dBm according to Annex B.4.3 for a corresponding Band are fulfilled.

Table 10.3.3.1-1: Intra-frequency PSSCH-RSRP absolute accuracy for UE capable of V2X sidelink communication

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR V2X operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm/SCSPSSCH |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSPSSCH = 15kHz | SCSPSSCH = 30kHz | SCSPSSCH = 60kHz |  |  |
| [4.5[ | [9.0] | [0] dB | NR_TDD_FR1_A | [-121] | [-118] | [-115] | N/A | -70 |
|  |  |  | NR_TDD_FR1_J | [-116.5] | [-113.5] | [-110.5] | N/A | -70 |
| [8.0] | [11.0] | [0] dB | NR_TDD_FR1_A, NR_TDD_FR1_J | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The parameter Ês/Iot is the Ês/Iot of PSSCH-DMRS. |  |  |  |  |  |  |  |  |

### 10.3.4	Intra-Frequency PSCCH-RSRP Measurement Accuracy Requirements for FR1

#### 10.3.4.1	Absolute PSCCH-RSRP Accuracy

The requirements for absolute accuracy of PSCCH-RSRP in this clause apply to a UE performing PSCCH-RSRP measurements on the same frequency as used by operating V2X sidelink communication.

The accuracy requirements in Table 10.3.4.1-1 are valid under the following conditions:

-	Demodulation reference signals for PSCCH are transmitted from one port.

-	Conditions defined in clause 7.3E of TS38.101-1 [18] for reference sensitivity are fulfilled.

-	PSCCH-RSRP|dBm according to Annex B.4.4 for a corresponding Band are fulfilled.

Table 10.3.4.1-1: Intra-frequency PSCCH-RSRP absolute accuracy for UE capable of V2X sidelink communication

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR V2X operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm/SCSPSCCH |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSPSCCH = 15kHz | SCSPSCCH = 30kHz | SCSPSCCH = 60kHz |  |  |
| [4.5[ | [9.0] | [0] dB | NR_TDD_FR1_A | [-121] | [-118] | [-115] | N/A | -70 |
|  |  |  | NR_TDD_FR1_J | [-116.5] | [-113.5] | [-110.5] | N/A | -70 |
| [8.0] | [11.0] | [0] dB | NR_TDD_FR1_A, NR_TDD_FR1_J | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS. |  |  |  |  |  |  |  |  |

### 10.3.5	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1

#### 10.3.5.1	Absolute SL-RSSI Accuracy

The intra-frequency SL-RSSI requirements are specified in Table 10.3.5.1-1. The requirements apply for measurement period of 1slot and for any configured measurement bandwidth larger than 10 RBs, provided that:

-	All symbols duing each RSSI measurement duration are available for RSSI sampling within the same measurement interval.

Table 10.3.5.1-1: Intra-frequency SL-RSSI absolute accuracy

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |
|  |  |  |  | NR V2X operating band groups Note 2 |  | Minimum Io |  |  | Maximum Io |
| dB |  | dB |  |  |  | dBm/SCSSL |  |  | dBm/BWChannel |
|  |  |  |  |  |  | SCSSL = 15kHz | SCSSL = 30kHz | SCSSL = 60kHz |  |
| 2.5 |  | 5.5 |  | NR_TDD_FR1_A |  | [-121] | [-118] | [-115] | -50 |
|  |  |  |  | NR_TDD_FR1_J |  | [-116.5] | [-113.5] | [-110.5] | -50 |
| 4.5 |  | 7.5 |  | Note 3 |  | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |

< END OF CHANGE #1 >