3GPP TSG-RAN WG4 Meeting #94bis-e                                R4-2005059

Online, 20th April-30th April, 2020

Source:	Samsung, KDDI

Title:	TP for TR 37.716-21-21: DC_41_n28-n78

Agenda item:			8.7.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-21 to include DC_41A_n28A-n78A and DC_41C_n28A-n78A according to the request in [1].

# 2. Reference

- RP-200080, Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL), LG Electronics.

# 3. Text Proposal

<Start of Text Proposal>

## 6.x	DC_41_n28-n78

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A_n28A-n78A DC_41C_n28A-n78A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_41A_n28A-n78A | DC_41A_n28A DC_41A_n78A | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_41C_n28A-n78A | DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A | 41 |  | 15 | See CA_41C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.x.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_41A_n28A-n78A with dual uplink on DC_41A_n28A and DC_41A_n78A were calculated and listed in Table 6.x.3-1 and 6.x.3-2 respectively.

Table 6.x.3-1: Harmonic and IMD analysis for DC_41_n28-n78 with UL on DC_41_n28

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1748 | 1987 | 3199 | 3438 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4244 | 4677 | 1000 | 1284 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5695 | 6128 | 3902 | 4186 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 6740 | 7367 | 252 | 581 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8191 | 8818 | 4605 | 4934 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3496 | 3974 | 6398 | 6876 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 122 | 496 | 9236 | 10057 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5308 | 5682 | 10687 | 11508 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2748 | 3271 | 5992 | 6664 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7101 | 7624 | 8894 | 9566 |

Table 6.x.3-2: Harmonic and IMD analysis for DC_41_n28-n78 with UL on DC_41_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 9900 | 11400 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 | 1304 | 5796 | 6490 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1192 | 2080 | 3910 | 5104 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 | 9180 | 9096 | 10290 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3688 | 4770 | 7210 | 8904 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10788 | 11870 | 12396 | 14090 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1220 | 2608 | 11592 | 12980 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 10510 | 12704 | 6184 | 7460 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 | 17890 | 13284 | 14560 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4520 | 6408 | 112 | 1470 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 | 16780 | 14088 | 15670 |

The coexistence studies of Band41 + Band n28 and Band41 + Band n78 will be captured in the constituent fallback modes. The summary of self-interference issues for the 3rd own receive band are listed below.

Based on Table 6.x.3-1 and Table 6.x.3-2:

- 2nd and 4th order IMD generated by dual uplink of Band 41 + Band n28 may also fall into own Rx of band n78.

- 2nd and 5th order IMD generated by dual uplink of Band 41 + Band n78 may also fall into own Rx of band n28.

### 6.x.4	∆TIB and ∆RIB values

For DC_41_n28-n78, the TIB,c and RIB,c values can reuse the values of DC_28A-41A_n78A as given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n28-n78 | 41 | 0.3 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n28-n78 | 41 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 6.x.5	MSD

Table 6.x.5-1 shows the required MSD, come from evaluation results:

Table 6.x.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_41_n28-n78

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n28A-n78A DC_41C_n28A-n78A | 41 | IMD2 | ¦fB41 +fn28¦ | 2580 | 5 | 25 | 2580 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | N/A |
|  | n78 |  |  | 3323 | 10 | 50 | 3323 | 28.21 |
|  | 41 | IMD2 | ¦fB41 -fn78¦ | 2642 | 5 | 25 | 2642 | N/A |
|  | n78 |  |  | 3440 | 10 | 50 | 3440 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 30.82 |
| NOTE 1: This band is subject to IMD4 also which MSD is not specified NOTE 2: This band is subject to IMD5 also which MSD is not specified |  |  |  |  |  |  |  |  |

<End of Text Proposal>