3GPP TSG-RAN WG4 Meeting #94bis-e                                                         R4-2005134

Online, 20th April-30th April, 2020

Source:	Samsung, KDDI, LGE

Title:	TP for TR 37.716-21-11: DC_18-41_n78

Agenda item:			8.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-11 to include DC_18-41_n78 according to the request in [1].

# 2. Reference

- RP-200271, Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.x	DC_18-41_n78

### 5.1.x.1		Operating bands for DC

Table 5.1.x.1-1: Band combination EN-DC (three bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_18-41_n78 | CA_18-41 | n78 | No |

### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_18A-41A_n78A | DC_18A_n78A DC_41A_n78A | CA_18A-41A | n78A |
| DC_18A-41C_n78A | DC_18A_n78A DC_41A_n78A DC_41C_n78A | CA_18A-41C | n78A |

### 5.1.x.3	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, the IMD issues for this band combination are as below list:

-  2nd order IMDs generated by dual uplink of band 18 and band n78 maybe fall into the Rx of band 41.

-  2nd and 5th order IMDs generated by dual uplink of band 41 and band n78 maybe fall into the Rx of band 18.

There is only IMD5 impact to Band 18 considering KDDI’s spectrum, since B18 only was used in Japan.

### 5.1.x.4	∆TIB and ∆RIB values

For DC_18-41_n78, the TIB,c and RIB,c values are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 5.1.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n78 | 18 | 0.3 |
|  | 41 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18-41_n78 | 18 | 0 |
|  | 41 | 0 |
|  | n78 | 0.5 |

### 5.1.x.5	REFSENS requirements

Table 5.1.x.5-1 shows the required MSD levels for the DC configuration.

Table 5.1.x.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| c |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_18A-41A_n78A DC_18A-41C_n78A | 18 | 820 | 5 | 25 | 865 | 3.4 | FDD | IMD5 ¦2*fn78-3*fB41¦ |
|  | n78 | 3527.5 | 10 | 50 | 3527.5 | N/A | FDD | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | TDD | N/A |

<End of Text Proposal>