3GPP RAN WG4 Meeting #94bis-e                                                                  R4-2005314

# Electronic Meeting, 20 – 30 Apr., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | Rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR of Annex.B for NR V2X side conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce condtions for NR V2X in B.4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce condtions for NR V2X in B.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Miss condtions for NR V2X |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

B.4	Conditions for V2X

B.4.1	Test parameters for GNSS signals

This clause defines the reference signal power levels of generated salellites for a corresponding GNSS, which will be used in V2X test cases.

Table B.4.1-1: GNSS Referenece Signal Power Parameters

| System | Parameters | Unit | Value |
| --- | --- | --- | --- |
|  | Number of generated satellites per system | - | 6 |
| GPS(1) | Reference signal power level for all satellites | dBm | -128.5 |
| Galileo | Reference signal power level for all satellites | dBm | -127 |
| GLONASS | Reference signal power level for all satellites | dBm | -131 |
| BDS | Reference signal power level for all satellites | dBm | -133 |
| NOTE 1: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. NOTE 2: The DUT UE does not need to support all systems. The DUT UE shall support at least one system and will be test for the supported systems. |  |  |  |

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements

This clause defines the following conditions for PSBCH-RSRP measurement accuracy requirements applicable for a corresponding operating band.

The conditions are defined in Table B.4.2-1 for FR1.

Table B.4.2-1: Conditions for PSBCH-RSRP measurements in FR1

| Parameter | NR V2X operating band groups Note1 | Minimum S-SSB_RP |  |  | S-SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | dBm/SCSS-SSB |  |  |  |  |  | dB |  |
|  |  | SCSS-SSB = 15kHz | SCSS-SSB = 30kHz | SCSS-SSB = 60kHz |  |
|  |  | [NR_TDD_FR1_A] |  | [-127] |  | [-124] |  | [-121] |  | -6 |  |
|  | [NR_TDD_FR1_I] | [-123] | [-120] | [-117] |  |
| NOTE 1: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. |  |  |  |  |  |

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE

This clause defines the S-SSB_RP and S-SSB Ês/Iot applicable for a corresponding operating band.

The conditions for selection/reselection to intra-frequency SyncRef UE are defined in Table B.4.5-1 for FR1.

Table B.4.5-1: V2X synchronization measurements in FR1

| Parameter | NR V2X operating band groups Note1 | Minimum S-SSB_RP |  |  | S-SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | dBm/SCSS-SSB |  |  |  | dB |  |
|  |  | SCSS-SSB = 15kHz | SCSS-SSB = 30kHz | SCSS-SSB = 60kHz |  |
|  | [NR_TDD_FR1_A] | [-121] | [-118] | [-115] | 0 |
|  | [NR_TDD_FR1_I] | [-117] | [-114] | [-111] | 0 |
| NOTE 1: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. |  |  |  |  |  |

B.4.4	[Conditions for L1 SL-RSRP Accuracy Requirements]

This clause defines the following condtions for L1 SL-RSRP measurement accuracy requirements applicable for a corresponding operating band.

The conditions are defined in Table B.4.4-1 for FR1.

Table B.4.4-1: Conditions for L1 SL-RSRP measurements in FR1

| Parameter | NR V2X operating band groups Note1 | Minimum L1 SL-RSRP |  |  | Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/SCS |  |  | dB |
|  |  | SCS= 15kHz | SCS= 30kHz | SCS = 60kHz |  |
|  | [NR_TDD_FR1_A] | [-121] | [-118] | [-115] | [ 0] |
|  | [NR_TDD_FR1_I] | [-117] | [-114] | [-111] |  |
| NOTE 1: NR V2X operating band groups are as defined in Section 3.5 for the corresponding NR operating bands. NOTE 2: The parameter Ês/Iot is the Ês/Iot of PSCCH-DMRS[/PSSCH-DMRS]. NOTE 3: The SCS is for PSCCH/[/PSSCH] |  |  |  |  |  |

< END OF CHANGE #1 >