3GPP TSG-RAN WG4 Meeting #94-e-Bis		R4-2005035

Electronic meeting, 20 – 30 April, 2020

Agenda item:	8.18.2

Source:	CableLabs

Title:	Study of DSS in band 48/n48 channel raster

Document for:	Discussion

# Introduction

RAN#86 meeting approved WID RP-193213 [1] targeting:

- “Channel raster: Confirm that NR channel raster can be aligned with LTE center frequencies.”

Two NR channel raster options, 100 and 300 kHz, were discussed during the RAN4 #94-e meeting but no agreement was achieved [2]. A third and fourth raster option of 15 and 30 kHz are part of the current draft of the standard for band n48 [9]. A WF for further study was proposed [3]. In this contribution, we provide a brief summary of the of DSS in band 48/n48 channel raster challenges and makes appropriate observations and proposals.

Proposals based on this analysis are contained in companion document R4-2003464.

# NR channel raster in band 48/n48

Based on TS 36.101 Section 5.7, LTE channels are defined by center frequency and bandwidth. LTE carrier center frequency must be an integer multiple of 100 kHz channel raster. More details are provided in Section 5.1.

However, NR channels have two groups of parameters that define the synchronization signal (SS/PBCH) resource block grid and separately the relative position of the common resource block grid. These two resource block grids, which may or may not be resource block aligned [14], are defined by a group of parameters, as illustrated in Figure 1, including

- Common Resource Block grid:

  - absoluteFrequencyPointA: serves as a common reference point for resource block (RB) grids and is defined by NR-ARFCN [4] and [5];

  - carrierBandwidth: The width of this carrier in number of PRBs using the subcarrier spacing defined for this carrier [6]. For band n48, 10 MHz nominal channel width, the maximum value is 24 for 30 kHz SCS or 52 for 15 kHz SCS [13];

  - offsettoCarrier: Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the subcarrier spacing for this carrier) [6].

- SS/PBCH Resource Block grid:

  - offsetToPointA: represents the frequency offset between point A and the lowest subcarrier of the lowest RB which has the subcarrier spacing provided by the higher-layer parameter subCarrierSpacingCommon and overlaps with the SS/PBCH block used by the UE for initial cell selection .  offsetToPointA is expressed in 180 kHz steps for FR1. The band n48 range: 0:2199, [4], [6] and [7];

  - kSSB: represents the frequency domain offset between SSB and the overall resource block grid in number of subcarriers. For FR1 the range 0:23 [8];

  - absoluteFrequencySSB: center frequency of SSB/PBCH, defined by global synchronization channel number (GSCN) [9], [10]. Note that absoluteFrequencySSB is not center of the channel. The frequency offset between the first subcarrier of SSB/PBCH and absoluteFrequencySSB is 3.6 MHz (10 RBs).

Note that absoluteFrequencySSB can only take a defined set of NR-ARFCN values identified by GSCN index. GSCN is defined [10] with an increment of 1.2 MHz below 3 GHz and 1.44 MHz above 3 GHz. This increment is also defined as sync raster and governs the position of the SSB.

Figure 1. Parameters that define a NR channel.

Observation 1: NR channels are not defined by center frequency.

Accordingly, we consider the goal of an NR channel raster analysis for band n48 is to determine if NR channels can be defined to fit within the 10 MHz channels identified in the FCC band plan for 3.5 GHz priority access licensing (PAL) seen in Figure 2.

Figure 2. FCC 10MHz Channelized PAL Channel Band Plan [15]

Note that while this diagram is not explicitly labeled on the frequency axis, the only configuration which produces 10 non-overlapping channels of 10 MHz each in the PAL sub-band (3550 – 3560MHz), results in channels defined on 10 MHz boundaries i.e. 3550, 3560, 3570, etc.

Therefore, our analysis goals targeted:

- To evaluate raster options and channel definition parameter sets which produce NR channels that occupy 10 MHz channels with the FCC defined lower and upper edge channel boundaries.

- After identifying such channel configurations, we then evaluate any implied changes to previously agreed 3GPP specification content and/or impact on spectrum usage efficiency.

## ΔF_Global

As currently defined in TS 38.101-1 Table 5.4.2.1-1 the Δf_global for frequencies above 3 GHz is 15 kHz. As a result, all the channel definition parameters which are defined as NR-ARFCN values must be specified as a valid NR-ARFCN values based on the band specific subset of integers defined in Section 5.4.2.3 and Table 5.4.2.3.-1.

### 30 kHz Channel Raster

The currently defined 30 kHz channel raster produces a band specific subset of integer NR-ARFCN values deductible  from the Δf_global Nref set of values.

Additionally, [12] allows +/-10 kHz offsets for ∆FRaster.

Further details on the calculations checking that the NR 10 MHz, channel edges, channel center frequency are provided in Appendix 5.2. Two subcases were analysed: Point A aligned with the Low Channel Edge and PointA below the Low Channel Edge. Both cases indicate tht PRB alignment with the channel cengter (same as in the LTE case).

Observation 1: The proposed 30 kHz channel raster for band n48 aligns with the Δf_global 15 kHz global raster defined for frequencies above 3 GHz and also with the LTE channel raster.

Observation 2: The 30kHz channel raster may require  +/- 10 kHz channel alignment offset

It is observed that the total Ch BW in this case is 10.055 MHz. This is caused by the difference between the offsetToPointA (expressed in multiples of 360 kHz – e.g. 720 kHz for the case above). In order to get a perfect LTE and NR PRB alignment, the offsetToCarrier definition may require further analysis (to match the lower guard band size).

In order to avoid defining non-standard Channel BW definitions, the offsetToCarrier should equal Low guard band (e.g. 665 kHz for this case).

Observation 3: The offsetToCarrier would require further analysis.

### 15 kHz Channel Raster

The currently defined 15 kHz channel raster produces a band specific subset of integer NR-ARFCN values deductible  from the Δf_global Nref set of values.

Additionally, [12] allows +/-5 kHz offsets for ∆FRaster.

It is observed that the total Ch BW in this case is 10.055 MHz. This is caused by the difference between the offsetToPointA (expressed in multiples of 360 kHz – e.g. 720 kHz for the case above).

In order to avoid defining non-standard Channel BW definitions, the offsetToCarrier should equal Low guard band (e.g. 312.5 kHz for this case.)

Without repeating the same analysis as presented in the above Table, we conclude that the 15 kHz also aligns with the LTE chanel raster.

Observation4: The 15 kHz channel raster for band n48 aligns with the Δf_global 15 kHz global raster defined for frequencies above 3 GHz and also with the LTE channel raster.

Observation 5: The 15 kHz channel raster may require  +/- 5 kHz channel alignment offset

### 100 kHz Channel Raster

The proposed 100 kHz channel raster does not produce raster frequencies that can be described as a band specific subset of the Nref values described by the Δf_global raster. This is because 100 kHz is not an integer multiple of the Δf_global 15 kHz. 7This observation is further supported by noting that all band specific channel rasters for bands above 3 GHz in FR1 use either 15 kHz or 30 kHz. All bands with a 100 kHz channel raster bands are below 3GHz where the Δf_global is 5 kHz which is a divisor of 100 kHz.

### 300 kHz Channel Raster

The proposed 300 kHz channel raster produces a band specific subset of NR-ARFCN integers deductible from the Δf_global Nref set of values.

Observation 7: Since 300kHz is a multiple of both 15 and 30 khz channel rasters, this case may become an implementation of either one or both of the 15 or 30 kHz channel rasters.

## GSCN Analysis

We observe that, in the frequency range above 3 GHz:

- The 1.44 MHz GSCN sync raster defines the set of frequencies used to define the center of the SSB.

- These frequencies must fall on a valid NR-ARFCN value to allow signaling per the definition of absoluteFrequencySSB given above.

In addition to this requirement:

- We assessed whether the resulting 7.2 MHz wide SSB, as defined by a valid GSCN index and NR-ARFCN value,

- Subsequently, the SSB would be fully contained within the FCC PAL channel definitions per above. Any NR channel to be used in band n48 uses a SSB and that SSB must fit within the defined lower and upper channel edges.

- In some cases, the SSB may be close to either the lower or upper channel edges. If so, we also checked that that either the lower or upper SSB edges break into minimum guard band [16].

### 30 kHz Channel Raster

The currently defined GSCN index values, and associated SSB center frequencies, can all be described as valid NR-ARFCN values on the band specific (15 kHz and) 30 kHz channel raster for band n48.

Furthermore, there is at least one valid GSCN index which can be defined using the 30 kHz NR-ARFCN values  producing an SSB fully contained within the target channel bounds for FCC PAL channels, including guard band requirements where necessary [Table 1].

In this case, the The GSCN frequency step (1.44 MHz) is an integer multiple of 30 kHz channel raster

### 100 kHz Channel Raster

The currently defined GSCN Index values and the associated SSB center frequencies, cannot be all described as valid NR-ARFCN values on the proposed band specific channel raster of 100 kHz for band n48 due to the following:.

- 100 kHz is not a multiple of the ΔFGlobal=15 kHz (f>3 GHz)

- The GSCN frequency step (1.44 MHz) is not a integer multiple of 100kHz channel raster

Observation 8: The proposed 100 kHz channel raster for band n48 is not a multiple of the ΔFGlobal=15 kHz (f>3 GHz) and it is not an integer submultiple of the GSCN 1.44MHz step, therefore it can’t be PRB aligned with for all LTE band 48 channels.

# Conclusions

All observations and proposals are comprised in our companion paper R4-2003464.

From this analysis, we conclude that:

1: 15 kHz Channel Raster

- It requires no changes to the actual standard.

- It supports PRB alignment with a coexistence LTE channel

- Requires further study to see if any of the issues found with other raster options are mitigated by using 15 kHz.

2: 30 kHz Channel Raster

- It requires no changes to the actual standard.

- It supports PRB alignment with a coexistence LTE channel

3: 100 kHz Channel Raster

- It breaks the NR release 15/16 specifications (e.g. ARFCN and GSCN compliance).

4: 300 kHz Channel Raster

- It breaks the NR release 15/16 specifications (e.g. GSCN compliance).

5. offsetToCarrier would require further analysis in order to equal the low Guard Band size.

# References

RP-193213, “LTE/NR spectrum sharing in band 48/n48 frequency range,” Apple Inc.

R4-2002902, “Email discussion summary for RAN4#94e_#35_NR_n48_LTE_48_coex,” Apple Inc.

R4-2002854, “WF on LTE/NR spectrum sharing in band 48/n48,” Apple, Charter Communications, Comcast and CableLabs.

TS 38.211 Section 4.4.4.1

TS 38.101-1 Section 5.4.2.1

TS 38.331 Section 6.3.2

TS38.331 section #6.3.2 FrequencyInfoDL-SIB

TS38.331 section #6.3.2 MIB field descriptions

TS 38.101-1 Section 5.4.3

TS 38.101-1, Tables 5.4.3.1-1 and 5.4.3.3-1

TS 38.101-1 Section 5.4.2

TS38.101-1 Section 5.4.1.1

TS38.101-1 Section 5.3.2

TS38.211 Section 7.4.3.1

https://www.fcc.gov/wireless/bureau-divisions/mobility-division/35-ghz-band/35-ghz-band-overview

TS 38.101-1 Section 5.3.3.

# Appendix

## LTE Channel Definition Parameters

LTE channels are fully defined by the center frequency (E-ARFCN) and the dl_Bandwidth.

Example

- FDL = FDL-Low + 0.1 × (NDL – Noffs-DL)

- for band 48:

- FDL = 3550, NDL = 55240:56739 (E-ARFCN)

- dl-Bandwidth = {n6, n15, n25, n50, n75, n100} (as signaled by MIB)

- ChBW = 1.4, 3, 5, 10, 15, 20 [MHz]

Figure 3. LTE channel definition parameters.

## Details on 30 kHz channel raster estimates

Table 5. Point A, center channel frequency and high channel edge estimates (PointA aligned with the Low Channel Edge)

Table 3 Point A, center channel frequency and high channel edge estimates (PointA below the Low Channel Edge)

Where: PointA frequency is the lower edge of the Ch BW, PRB HI is the high end of the 24 PRBs and Hi Channel is the frequency of the higher edge of the CH BW