3GPP TSG RAN WG4 Meeting #94-e-BIS	R4-2005060

Online, 20th – 30th April 2020

Agenda item:	8.7.2

Source:	LG Electronics

Title:	MSD results for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-16

Document for:	Approval

1	Introduction

Then new EN-DC of x DL Band LTE (x=1, 2, 3, 4) and 2 DL Band NR were approved in RAN#86[1]. In this paper we propose the required MSD levels for the new EN-DC bands combinations base on summary paper [2].

2	MSD analysis

In rel-16 EN-DC of x DL Band LTE (x=1, 2, 3, 4) and 2 DL Band NR basket WI, RAN4 also consider shared antenna RF architectures for NSA UE in sub-6GHz as LTE system. So we consider shared antenna RF architecture for general NSA DC UE to derive MSD levels.

For the MSD analysis of these 3DL/2UL EN-DC NR UE, we assume the parameters and attenuation levels based on current UE RF FE components as shown in Table 1 and 2.

Table 1 show the RF component isolation parameters to derive MSD level at sub-6GHz.

Table 1: UE RF Front-end component parameters

| UE ref. architecture Component | Triplexer-Diplexer Architecture w/ single ant. |  |  |  | Cascaded Diplexer Architecture w/ single ant. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DC_28A_n40A-n78A, DC_3A_n28A-n77A DC_8A_n28A-n77A, DC_42A_n28A-n77A DC_2A_n38A-n78A, DC_66A_n38A-n78A DC_18A_n3A-n77A, DC_28A_n3A-n77A DC_41A_n3A-n77A, DC_41A_n28A-n77A DC_20A_n41A-n78A, DC_1A_n75A-n78A DC_3A_n75A-n78A |  |  |  | DC_7A_n8A-n40A |  |  |  |
|  | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| Ant. Switch | 112 | 68 | 55 | 55 | 112 | 68 | 55 | 55 |
| Triplexer | 110 | 72 | 55 | 52 |  |  |  |  |
| Quadplexer |  |  |  |  | 112 | 72 | 55 | 52 |
| Diplexer | 115 | 87 | 55 | 55 | 115 | 87 | 55 | 55 |
| Duplexer | 100 | 75 | 55 | 53 | 100 | 75 | 55 | 53 |
| PA Forward | 28.0 | 32 | 30 | 28 | 28.0 | 32 | 30 | 28 |
| PA Reversed | 40 | 30.5 | 30 | 30 | 40 | 30.5 | 30 | 30 |
| LNA | 10 | 0 | 0 | -10 | 10 | 0 | 0 | -10 |

Table 2 show the isolation levels according to the RF component.

Table 2: UE RF Front-end component isolation parameters

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 60 | PCB isolation (PA forward mixing) |
| Triplexer | 20 | High/low band isolation |
| Diplexer | 25 | High/low band isolation |
| PA (out) to PA (out) | 60 | L-H/H-L cross-band |
| PA (out) to PA (out) | 50 | H-H cross-band |
| LNA (in) to PA (out) | 60 | L-H/H-L cross-band |
| LNA (in) to PA (out) | 50 | H-H cross-band |
| Duplexer | 50 | Tx band rejection at Rx band |

Based on these assumptions, we proposed the MSD levels as below. For the DC_1A_n41A-n78A, DC_3A_n41A-n78A and DC_20A_n41A-n78A, the proposed MSD levels will be merged with the results from interested companies as average manner to derive MSD requirements.

Table 3: Proposed MSD test configuration and results by IMD problems

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n8A-n40A | 7 | IMD5 | ¦2*fB7 -3*fn8¦ | 2530 | 5 | 25 | 2650 | 5 | N/A |
|  | n8 |  |  | 905 | 5 | 25 | 950 | 5 |  |
|  | n40 |  |  | 2345 | 5 | 25 | 2345 | 5 | 3.0 |
| DC_28A_n40A-n78A | 28 | IMD3 | ¦2*fB28 +fn40¦ | 733 | 5 | 25 | 788 | 5 | N/A |
|  | n40 |  |  | 2320 | 5 | 25 | 2320 | 5 |  |
|  | n78 |  |  | 3786 | 10 | 50 | 3786 | 10 | 14.4 |
|  | 28 | IMD3 | ¦2*fB28 -fn78¦ | 733 | 5 | 25 | 788 | 5 | N/A |
|  | n78 |  |  | 3786 | 10 | 50 | 3786 | 10 |  |
|  | n40 |  |  | 2320 | 5 | 25 | 2320 | 5 | 15.8 |
| DC_3A_n28A-n77A | 3 | IMD3 | ¦2*fB3 +fn28¦ | 1720 | 5 | 25 | 1815 | 5 | N/A |
|  | n28 |  |  | 733 | 5 | 25 | 788 | 5 |  |
|  | n77 |  |  | 4173 | 10 | 50 | 4173 | 10 | 15.9 |
|  | 3 | IMD3 | ¦2*fB3 -fn77¦ | 1712.5 | 5 | 25 | 1807.5 | 5 | N/A |
|  | n77 |  |  | 4195 | 10 | 50 | 4195 | 10 |  |
|  | n28 |  |  | 715 | 5 | 25 | 770 | 5 | 15.3 |
| DC_8A_n28A-n77A | 8 | IMD4 | ¦3*fB8 +fn28¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 |  |
|  | n77 |  |  | 3473 | 10 | 50 | 3473 | 10 | 10.3 |
|  | 8 | IMD4 | ¦3*fB8 -fn77¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n77 |  |  | 3495 | 10 | 50 | 3495 | 10 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 11.6 |
| DC_42A_n28A-n77A | 42 | IMD2 | ¦fB42 +fn28¦ | 3410 | 5 | 25 | 3410 | 5 | N/A |
|  | n28 |  |  | 733 | 5 | 25 | 788 | 5 |  |
|  | n77 |  |  | 4143 | 10 | 50 | 4143 | 10 | 29.2 |
| DC_2A_n38A-n78A | 2 | IMD3 | ¦fB2 -2*fn38¦ | 1870 | 5 | 25 | 1950 | 5 | N/A |
|  | n38 |  |  | 2610 | 5 | 25 | 2610 | 5 |  |
|  | n78 |  |  | 3350 | 10 | 50 | 3350 | 10 | 14.8 |
| DC_66A_n38A-n78A | 66 | IMD3 | ¦fB66 -2*fn38¦ | 1760 | 5 | 25 | 2160 | 5 | N/A |
|  | n38 |  |  | 2610 | 5 | 25 | 2610 | 5 |  |
|  | n78 |  |  | 3460 | 10 | 50 | 3460 | 10 | 15.0 |
| DC_18A_n3A-n77A | 18 | IMD3 | ¦2*fB18 +fn3¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n3 |  |  | 1770 | 5 | 25 | 1865 | 5 |  |
|  | n77 |  |  | 3410 | 10 | 50 | 3410 | 10 | 16.3 |
|  | 18 | IMD3 | ¦2*fB18 -fn77¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n77 |  |  | 3505 | 10 | 50 | 3505 | 10 |  |
|  | n3 |  |  | 1770 | 5 | 25 | 1865 | 5 | 15.7 |
| DC_28A_n3A-n77A | 28 | IMD3 | ¦fB28 +2*fn3¦ | 733 | 5 | 25 | 788 | 5 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 | 5 |  |
|  | n77 |  |  | 4173 | 10 | 50 | 4173 | 10 | 15.9 |
|  | 28 | IMD3 | ¦2*fB28 -fn77¦ | 735 | 5 | 25 | 790 | 5 | N/A |
|  | n77 |  |  | 3320 | 10 | 50 | 3320 | 10 |  |
|  | n3 |  |  | 1755 | 5 | 25 | 1850 | 5 | 17.0 |
| DC_41A_n3A-n77A DC_41A_n3A-n78A | 41 | IMD3 | ¦2*fB41 -fn3¦ | 2580 | 5 | 25 | 2580 | 5 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 | 5 |  |
|  | n77/n78 |  |  | 3440 | 10 | 50 | 3440 | 10 | 16.8 |
|  | 41 | IMD3 | ¦2*fB41 -fn77¦ | 2620 | 5 | 25 | 2620 | 5 | N/A |
|  | n77/n78 |  |  | 3400 | 10 | 50 | 3400 | 10 |  |
|  | n3 |  |  | 1745 | 5 | 25 | 1840 | 5 | 16.4 |
| DC_41A_n28A-n77A DC_41A_n28A-n78A | 41 | IMD2 | ¦fB41 +fn28¦ | 2580 | 5 | 25 | 2580 | 5 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 |  |
|  | n77/n78 |  |  | 3323 | 10 | 50 | 3323 | 10 | 28.2 |
|  | 41 | IMD2 | ¦fB41 -fn77¦ | 2642 | 5 | 25 | 2642 | 5 | N/A |
|  | n77/n78 |  |  | 3440 | 10 | 50 | 3440 | 10 |  |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 | 30.8 |
| DC_1A_n41A-n78A | 1 | IMD3 | ¦fB1 -2*fn41¦ | 1930 | 5 | 25 | 2120 | 5 | N/A |
|  | n41 |  |  | 2650 | 5 | 25 | 2650 | 5 |  |
|  | n78 |  |  | 3370 | 10 | 50 | 3370 | 10 | 16.8 |
|  | 1 | IMD4 | ¦3*fB1 -fn78¦ | 1970 | 5 | 25 | 2160 | 5 | N/A |
|  | n78 |  |  | 3400 | 10 | 50 | 3400 | 10 |  |
|  | n41 |  |  | 2510 | 5 | 25 | 2510 | 5 | 11.0 |
| DC_3A_n41A-n78A | 3 | IMD3 | ¦2*fn41 –fB3¦ | 1720 | 5 | 25 | 1815 | 5 | N/A |
|  | n41 |  |  | 2580 | 5 | 25 | 2580 | 5 |  |
|  | n78 |  |  | 3440 | 10 | 50 | 3440 | 10 | 16.8 |
| DC_20A_n41A-n78A | 20 | IMD2 | ¦fB20 +fn41¦ | 837 | 5 | 25 | 796 | 5 | N/A |
|  | n41 |  |  | 2650 | 5 | 25 | 2650 | 5 |  |
|  | n78 |  |  | 3487 | 10 | 50 | 3487 | 10 | 27.9 |
|  | 20 | IMD2 | ¦fB20 -fn78¦ | 837 | 5 | 25 | 796 | 5 | N/A |
|  | n78 |  |  | 3370 | 10 | 50 | 3370 | 10 |  |
|  | n41 |  |  | 2533 | 5 | 25 | 2533 | 5 | 28.8 |
| DC_1A_n75A-n78A | 1 | IMD2 | ¦fB1 -fn78¦ | 1930 | 5 | 25 | 2120 | 5 | N/A |
|  | n78 |  |  | 3400 | 10 | 50 | 3400 | 10 |  |
|  | n75 |  |  | - | - | - | 1470 | 5 | 29.0 |
| DC_3A_n75A-n78A | 3 | IMD2 | ¦fB3 -fn78¦ | 1782.5 | 5 | 25 | 1877.5 | 5 | N/A |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 |  |
|  | n75 |  |  | - | - | - | 1514.5 | 5 | 10.0 |

3	Proposed text for TR37.716-21-21:

[Unchanged Parts Skipped]

6.X	DC_7_n8-n40

6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 produced by Band 7 and NR band n8 that impact the reference sensitivity of NR band n40. The required MSD is shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n8A-n40A | 7 | IMD5 | ¦2*fB7 -3*fn8¦ | 2530 | 5 | 25 | 2650 | 5 | N/A |
|  | n8 |  |  | 905 | 5 | 25 | 950 | 5 |  |
|  | n40 |  |  | 2345 | 5 | 25 | 2345 | 5 | 3.0 |

6.X	DC_28_n40-n78

6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD3& IMD4 produced by Band 28 and NR band n40 that impact the reference sensitivity of NR band n78. Also there is IMD3 produced by Band 28 and NR band n78 that impact the reference sensitivity of NR band n40. The required MSDs are shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n40A-n78A | 28 | IMD3 | ¦2*fB28 +fn40¦ | 733 | 5 | 25 | 788 | 5 | N/A |
|  | n40 |  |  | 2320 | 5 | 25 | 2320 | 5 |  |
|  | n78 |  |  | 3786 | 10 | 50 | 3786 | 10 | 14.4 |
|  | 28 | IMD3 | ¦2*fB28 -fn78¦ | 733 | 5 | 25 | 788 | 5 | N/A |
|  | n78 |  |  | 3786 | 10 | 50 | 3786 | 10 |  |
|  | n40 |  |  | 2320 | 5 | 25 | 2320 | 5 | 15.8 |

6.X	DC_3_n28-n77

6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD3, IMD4 & IMD5 produced by Band 3 and NR band n28 that impact the reference sensitivity of NR band n77. Also there is IMD3 produced by Band 3 and NR band n77 that impact the reference sensitivity of NR band n28. The required MSDs are shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n28A-n77A | 3 | IMD3 | ¦2*fB3 +fn28¦ | 1720 | 5 | 25 | 1815 | 5 | N/A |
|  | n28 |  |  | 733 | 5 | 25 | 788 | 5 |  |
|  | n77 |  |  | 4173 | 10 | 50 | 4173 | 10 | 15.9 |
|  | 3 | IMD3 | ¦2*fB3 -fn77¦ | 1712.5 | 5 | 25 | 1807.5 | 5 | N/A |
|  | n77 |  |  | 4195 | 10 | 50 | 4195 | 10 |  |
|  | n28 |  |  | 715 | 5 | 25 | 770 | 5 | 15.3 |

6.X	DC_8_n28-n77

6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 produced by Band 8 and NR band n28 that impact the reference sensitivity of NR band n77. Also there is IMD4 produced by Band 8 and NR band n77 that impact the reference sensitivity of NR band n28. The required MSDs are shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n28A-n77A | 8 | IMD4 | ¦3*fB8 +fn28¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 |  |
|  | n77 |  |  | 3473 | 10 | 50 | 3473 | 10 | 10.3 |
|  | 8 | IMD4 | ¦3*fB8 -fn77¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n77 |  |  | 3495 | 10 | 50 | 3495 | 10 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 11.6 |

6.X	DC_42_n28-n77

6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD2 produced by Band 42 and NR band n28 that impact the reference sensitivity of NR band n77. The required MSD si shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_42A_n28A-n77A | 42 | IMD2 | ¦fB42 +fn28¦ | 3410 | 5 | 25 | 3410 | 5 | N/A |
|  | n28 |  |  | 733 | 5 | 25 | 788 | 5 |  |
|  | n77 |  |  | 4143 | 10 | 50 | 4143 | 10 | 29.2 |

6.X	DC_2_n38-n78

6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 produced by Band 2 and NR band n38 that impact the reference sensitivity of NR band n78. The required MSD si shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n38A-n78A | 2 | IMD3 | ¦fB2 -2*fn38¦ | 1870 | 5 | 25 | 1950 | 5 | N/A |
|  | n38 |  |  | 2610 | 5 | 25 | 2610 | 5 |  |
|  | n78 |  |  | 3350 | 10 | 50 | 3350 | 10 | 14.8 |

6.X	DC_66_n38-n78

6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 produced by Band 66 and NR band n38 that impact the reference sensitivity of NR band n78. The required MSD si shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n38A-n78A | 66 | IMD3 | ¦fB66 -2*fn38¦ | 1760 | 5 | 25 | 2160 | 5 | N/A |
|  | n38 |  |  | 2610 | 5 | 25 | 2610 | 5 |  |
|  | n78 |  |  | 3460 | 10 | 50 | 3460 | 10 | 15.0 |

6.X	DC_18_n3-n77

6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 produced by Band 18 and NR band n3 that impact the reference sensitivity of NR band n77. Also there is IMD3 produced by Band 18 and NR band n77 that impact the reference sensitivity of NR band n3. The required MSDs are shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n3A-n77A | 18 | IMD3 | ¦2*fB18 +fn3¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n3 |  |  | 1770 | 5 | 25 | 1865 | 5 |  |
|  | n77 |  |  | 3410 | 10 | 50 | 3410 | 10 | 16.3 |
|  | 18 | IMD3 | ¦2*fB18 -fn77¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n77 |  |  | 3505 | 10 | 50 | 3505 | 10 |  |
|  | n3 |  |  | 1770 | 5 | 25 | 1865 | 5 | 15.7 |

6.X	DC_28_n3-n77

6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 produced by Band 28 and NR band n3 that impact the reference sensitivity of NR band n77. Also there is IMD3 produced by Band 28 and NR band n77 that impact the reference sensitivity of NR band n3. The required MSDs are shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n3A-n77A | 28 | IMD3 | ¦fB28 +2*fn3¦ | 733 | 5 | 25 | 788 | 5 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 | 5 |  |
|  | n77 |  |  | 4173 | 10 | 50 | 4173 | 10 | 15.9 |
|  | 28 | IMD3 | ¦2*fB28 -fn77¦ | 735 | 5 | 25 | 790 | 5 | N/A |
|  | n77 |  |  | 3320 | 10 | 50 | 3320 | 10 |  |
|  | n3 |  |  | 1755 | 5 | 25 | 1850 | 5 | 17.0 |

6.X	DC_41_n3-n77

6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 produced by Band 41 and NR band n3 that impact the reference sensitivity of NR band n77. Also there is IMD3 produced by Band 41 and NR band n77 that impact the reference sensitivity of NR band n3. The required MSDs are shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n3A-n77A | 41 | IMD3 | ¦2*fB41 -fn3¦ | 2580 | 5 | 25 | 2580 | 5 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 | 5 |  |
|  | n77 |  |  | 3440 | 10 | 50 | 3440 | 10 | 16.8 |
|  | 41 | IMD3 | ¦2*fB41 -fn77¦ | 2620 | 5 | 25 | 2620 | 5 | N/A |
|  | n77 |  |  | 3400 | 10 | 50 | 3400 | 10 |  |
|  | n3 |  |  | 1745 | 5 | 25 | 1840 | 5 | 16.4 |

6.X	DC_41_n3-n78

6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 produced by Band 41 and NR band n3 that impact the reference sensitivity of NR band n78. Also there is IMD3 produced by Band 41 and NR band n78 that impact the reference sensitivity of NR band n3. The required MSDs are shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n3A-n78A | 41 | IMD3 | ¦2*fB41 -fn3¦ | 2580 | 5 | 25 | 2580 | 5 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 | 5 |  |
|  | n78 |  |  | 3440 | 10 | 50 | 3440 | 10 | 16.8 |
|  | 41 | IMD3 | ¦2*fB41 -fn78¦ | 2620 | 5 | 25 | 2620 | 5 | N/A |
|  | n78 |  |  | 3400 | 10 | 50 | 3400 | 10 |  |
|  | n3 |  |  | 1745 | 5 | 25 | 1840 | 5 | 16.4 |

6.X	DC_41_n28-n77

6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD2 produced by Band 41 and NR band n28 that impact the reference sensitivity of NR band n77. Also there is IMD2 produced by Band 41 and NR band n77 that impact the reference sensitivity of NR band n28. The required MSDs are shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n28A-n77A | 41 | IMD2 ¦fB41 +fn28¦ | 2580 | 5 | 25 | 2580 | 5 | N/A |
|  | n28 |  | 743 | 5 | 25 | 798 | 5 | N/A |
|  | n77 |  | 3323 | 10 | 50 | 3323 | 10 | 28.2 |
|  | 41 | IMD2 ¦fB41 -fn77¦ | 2642 | 5 | 25 | 2642 | 5 | N/A |
|  | n77 |  | 3440 | 10 | 50 | 3440 | 10 | N/A |
|  | n28 |  | 743 | 5 | 25 | 798 | 5 | 30.8 |

6.X	DC_41_n28-n78

6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD2 produced by Band 41 and NR band n28 that impact the reference sensitivity of NR band n78. Also there is IMD2 produced by Band 41 and NR band n78 that impact the reference sensitivity of NR band n28. The required MSDs are shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_41A_n28A-n78A | 41 | IMD2 ¦fB41 +fn28¦ | 2580 | 5 | 25 | 2580 | 5 | N/A |
|  | n28 |  | 743 | 5 | 25 | 798 | 5 | N/A |
|  | n78 |  | 3323 | 10 | 50 | 3323 | 10 | 28.2 |
|  | 41 | IMD2 ¦fB41 -fn78¦ | 2642 | 5 | 25 | 2642 | 5 | N/A |
|  | n78 |  | 3440 | 10 | 50 | 3440 | 10 | N/A |
|  | n28 |  | 743 | 5 | 25 | 798 | 5 | 30.8 |

6.X	DC_1_n75-n78

6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD2 produced by Band 1 and NR band n78 that impact the reference sensitivity of the S-DL NR band n75. The required MSD is shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n75A-n78A | 1 | IMD2 | ¦fB1 -fn78¦ | 1930 | 5 | 25 | 2120 | 5 | N/A |
|  | n78 |  |  | 3400 | 10 | 50 | 3400 | 10 |  |
|  | n75 |  |  | - | - | - | 1470 | 5 | 29.0 |

6.X	DC_3_n75-n78

6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD2 produced by Band 3 and NR band n78 that impact the reference sensitivity of the S-DL NR band n75 which was not directely impact to n75, but 2nd IMD sidelobe will be impacted. The required MSD is shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n75A-n78A | 3 | IMD2 | ¦fB3 -fn78¦ | 1782.5 | 5 | 25 | 1877.5 | 5 | N/A |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 |  |
|  | n75 |  |  | - | - | - | 1514.5 | 5 | 10.0 |

[End of changes]