3GPP TSG-RAN WG4 Meeting #94bis-e                                                          R4-2005129

Online, 20th April-30th April, 2020

Source:	Samsung, KDDI

Title:	TP for TR 37.716-11-11: DC_41_n28

Agenda item:			8.3.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-11-11 to include DC_41_n28 according to the request in [1].

# 2. Reference

- RP-200221, Revised WID on EN-DC for 2 bands DL with 2 bands UL (1 LTE band + 1 NR band).

# 3. Text Proposal

<Start of Text Proposal>

### 6.1.x	DC_41_n28

#### 6.1.x.1	Operating bands for DC

Table 6.1.x.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_41_n28 | 41 | n28 | No |

#### 6.1.x.2	Configuration for DC

Table 6.1.x.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_41A_n28A | DC_41A_n28A | 41A | n28A |
| DC_41C_n28A | DC_41A_n28A DC_41C_n28A | CA_41C | n28A |

#### 6.1.x.3	Maximum output power for DC

Table 6.1.x.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_41A_n28A | 23 | +2/-3 |
| DC_41C_n28A | 23 | +2/-3 |

#### 6.1.x.4	Spurious emission and UE co-existence for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.x.4-1

Table 6.1.x.4-1: Harmonic and IMD analysis for DC_41_n28

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 7488 | 8070 |
| Two tone 2nd order IMD products | (fx_low – fy_high) | (fx_high – fy_low) | (fx_low + fy_low) | (fx_high + fy_high) |
| IMD frequency limits (MHz) | 1748 | 1987 | 3199 | 3438 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | (2*fx_low – fy_high) | (2*fx_high – fy_low) |
| IMD frequency limits (MHz) | 1000 | 1284 | 4244 | 4677 |
| Two-tone 3rd order IMD products | (2*fx_low + fy_low) | (2*fx_high + fy_high) | (2*fx_low + fy_low) | (2*fx_high + fy_high) |
| IMD frequency limits (MHz) | 3902 | 4186 | 5695 | 6128 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | (3*fy_low – fx_high) | (3*fy_high – fx_low) |
| IMD frequency limits (MHz) | 581 | 252 | 6740 | 7367 |
| Two-tone 4th order IMD products | (2*fy_low – 2*fx_high) | (2*fy_high – 2*fx_low) | (2*fx_low+ 2*fy_low) | (2*fx_high + 2*fy_high) |
| IMD frequency limits (MHz) | 3496 | 3974 | 6398 | 6876 |
| Two-tone 4th order IMD products | (3*fx_low + fy_low) | (3*fx_high + fy_high) | (3*fy_low + fx_low ) | (3*fy_high +fx_high) |
| IMD frequency limits (MHz) | 4605 | 4934 | 8191 | 8818 |
| Two-tone 4th order IMD products | (2*fx_low + 2*fy_low) | (2*fx_high + 2*fy_high) | (2*fx_low - 2*fy_high) | (2*fx_high - 2*fy_low) |
| IMD frequency limits (MHz) | 6398 | 6876 | 3496 | 3974 |
| Two-tone 5th order IMD products | (4*fy_low – fx_high) | (4*fy_high – fx_low) | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ |
| IMD frequency limits (MHz) | 9236 | 10057 | 122 | 496 |
| Two-tone 5th order IMD products | (3* fy_low-2*fx_high ) | (3*fy_high-2*fx_low) | ¦2*fy_low –3* fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5992 | 6664 | 2748 | 3271 |
| Two-tone 5th order IMD products | (4*fy_low + fx_low ) | (4*fy_high +fx_high) | (4*fx_low + fy_low) | (4*fx_high + fy_high) |
| IMD frequency limits (MHz) | 10687 | 11508 | 5308 | 5682 |
| Two-tone 5th order IMD products | (3*fy_low + 2*fx_low ) | (3*fy_high +2*fx_high) | (3*fx_low +2* fy_low) | (3*fx_high + 2*fy_high) |
| IMD frequency limits (MHz) | 8894 | 9566 | 7101 | 7624 |

Based on above table, there is no IMD falling into their own Rx frequencies.

The Spurious emission requirement for this DC combination is specified in Table  6.1.x.4-2.

Table 6.1.x.4-2: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_41_n28 | E-UTRA Band 4, 10, 12, 13, 14, 17, 18, 19, 20, 26, 27, 29, 39, 42, 43, 50, 51, 52, 65, 66, 71, 73, 85 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9,10 |
|  | E-UTRA Band 2, 3, 5, 8, 24, 25, 30, 31, 34, 40, 44, 48, 70, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |

#### 6.1.x.5	MSD analysis for DC

There is no additional requirement for this band combination.

#### 6.1.x.6	∆TIB and ∆RIB values

For DC_41_n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.1.x.6-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n28 | 41 | 0.3 |
|  | n28 | 0.3 |

Table 6.1.x.6-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n28 | 41 | 0 |
|  | n28 | 0 |

<End of Text Proposal>