3GPP TSG-RAN WG4 Meeting #95-e	R4-2006005

Electronic Meeting, 25 May – 5 June, 2020

Source:	ZTE

Title:	Discussion on RRC procedure delay in RRC release with redirection

Agenda item:	4.9.5

Document for:	Discussion

# 1	Introduction

- In RAN4 94e, the issue of RRC procedure delay in RRC redirection has been brought up with some progress made [1]. Some of the content from the WF is copied below:

- During RAN4 94-e-bis, the agreement was still to remove the reference to 38.331 but no agreement on the value of the delay.

- In this paper we continue to discuss this problem based on previous agreements.

2	The value of TRRC_procedure_delay

As to the value of TRRC_procedure_delay, 110  ms inherited from LTE specification seems unreasonably long, compared to other RRC procedures.

Part of the content in clause 12 of 38.331 [2] is copied below:

12	Processing delay requirements for RRC procedures

The UE performance requirements for RRC procedures are specified in the following tables. The performance requirement is expressed as the time in [ms] from the end of reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation). In case the RRC procedure triggers BWP switching, the RRC procedure delay is the value defined in the following table plus the BWP switching delay defined in TS 38.133 [14], clause 8.6.3.

Table 12.1-1: UE performance requirements for RRC procedures for UEs

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | 10 |  |
| RRC reconfiguration (scell addition/release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration (SCG establishment/ modification/ release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC setup | RRCSetup | RRCSetupComplete | 10 |  |
| RRC Release | RRCRelease |  | NA |  |
| RRC re-establishment | RRCReestablishment | RRCReestablishmentComplete | 10 |  |
| RRC resume | RRCResume | RRCResumeComplete | 6 or 10 | Value=6 applies for a UE supporting reduced CP latency for the case of RRCResume message only including MAC and PHY configuration, and no DRX, SPS, configured grant, CA or MIMO re-configuration will be triggered by this message. Further, the UL grant for transmission of RRCResumeComplete and the data is transmitted over common search space with DCI format 0_0. In this scenario, the RRC procedure delay [ms] can extend beyond the reception of the UL grant, up to 7 ms. For other cases, Value = 10 applies. |
| RRC resume (scell addition) | RRCResume | RRCResumeComplete | 16 |  |
| Initial AS security activation | SecurityModeCommand | SecurityModeComplete/SecurityModeFailure | 5 |  |
| Other procedures |  |  |  |  |
| UE assistance information |  | UEAssistanceInformation | NA |  |
| UE capability transfer | UECapabilityEnquiry | UECapabilityInformation | 80 |  |
| Counter check | CounterCheck | CounterCheckResponse | 5 |  |

According to the table, the RRC procedure delay for most RRC processes are less than 20 ms. This should be a benchmark for RRC message processing delay for RRC release with redirection. We propose to set TRRC_procedure_delay = 20  ms.

TRRC_procedure_delay = 20  ms.

3	Where to specify TRRC_procedure_delay

The options given in the WF [1] are:

Option 1. TRRC_procedure_delay = X ms specified in test cases

- This is similar to what we have now. We only specify the value in the test cases, using no reference to 38.331. This option would first require internal discussion of a reasonable value in RAN4.

Option 2. TRRC_procedure_delay = X ms specified in core requirements and test cases

- This option means to specify TRRC_procedure_delay by a fixed value in both core requirement and test cases, using no reference to 38.331. This option would first require internal discussion of a reasonable value in RAN4.

Adopting Option 2 would suggest a more thorough correction and gives the reader a clear concept of the value of TRRC_procedure_delay in core requirements. As the value of TRRC_procedure_delay doesn’t vary across different tests, it can be specified in core requirements.

Specify TRRC_procedure_delay = 20 ms in core requirements and test cases.

We provide a draft CR [3] capturing the suggested changes in this paper.

4	36.133 related changes

In TS 36.133, for most redirection cases (redirection to UTRAN TDD, GERAN, UTRAN FDD), TRRC_procedure_delay is specified in test cases. However for redirection to NR there is no test case.

Observation 1: For redirection to NR there is no test case.

Thus, TRRC_procedure_delay has to be specified in core requirements. We prepare a draft CR [4] to specify TRRC_procedure_delay = 110 ms (as in other cases) in the core requirement in clause 6.3.2.4 in 36.133. In our view this shouldn’t be controversial at all since RAN4 has spotted a hole so the delay needs to be specified somewhere, and core part for redirection to NR is the only place to specify it since there’s no test cases for such redirection.

Specify TRRC_procedure_delay = 110 ms in clause 6.3.2.4 in 36.133 since there is no test case for redirection to NR. Agree CR [4].

5	Conclusion

Proposal 1: TRRC_procedure_delay = 20  ms.

Proposal 2: Specify TRRC_procedure_delay = 20 ms in core requirements and test cases.

Observation 1: For redirection to NR there is no test case.

Proposal 3: Specify TRRC_procedure_delay = 110 ms in clause 6.3.2.4 in 36.133 since there is no test case for redirection to NR. Agree CR [4].

References

- R4-2002206, WF on TRRC_procedure_delay for requirements of RRC release with redirection, ZTE Corporation

- TS 38.331

- R4-2006002, [CR] RRC release with redirection 38.133 R16, ZTE Corporation

- R4-2006003, [CR] RRC release with redirection 36.133 R15, ZTE Corporation