3GPP TSG-RAN WG4 Meeting #95-e	R4-2006012

Electronic Meeting, 25 May – 5 June, 2020

Source:	ZTE

Title:	On BWP Switch in NR-U

Agenda item:	6.1.5.9

Document for:	Discussion

# 1	Introduction

In RAN4 94-e, one of the open issues remain on BWP switch for NR-U is whether to re-use the DCI and timer-based BWP switching delay to NR-U scenario.

The agreement in WF [1] is copied below:

In RAN4 94-e-bis, the following was captured in WF [2]:

This paper intends to provide our view on this topic.

2	Discussion

As to the end point of UL BWP switch, we think it’s fine to define the end point as when the UE is ready to transmit RACH. This would exclude any delay caused by no UL resources due to failed UL LBT attempt, while is still testable.

The end point of UL BWP switch is when UE is ready to transmit RACH.

During email discussions last meeting, some companies brought up the issue that the old and new BWP should avoid being overlapped. In our view, a more precise statement of this requirement would be that when performing UL BWP switch, the new BWP shouldn’t contain the old BWP since under this case, the UE will much likely to end up with UL LBT failure on the new BWP as well.

When performing UL BWP switch, the new BWP shouldn’t contain the old BWP.

3	Conclusion

- Proposal 1: The end point of UL BWP switch is when UE is ready to transmit RACH.

Proposal 2: When performing UL BWP switch, the new BWP shouldn’t contain the old BWP.

References

- R4-2002335, WF on NR-U RRM Requirements (Part 2)

- R4-2005367