3GPP TSG-RAN WG4 Meeting #95-e	R4-2006023

Electronic Meeting, 25 May – 5 June, 2020

Source:	ZTE

Title:	PBCH payload reading for SSB index identification in NR-U

Agenda item:	6.1.5.11

Document for:	Discussion

# 1	Introduction

- In RAN4 94-e meeting, the topic of PBCH payload reading for SSB index identification in NR-U has been discussed [1]. The group has reached one consensus conditioned on that Q is known to the UE, while couldn’t agree on if Q could be assumed to be known to the UE.

- The options are listed below:

- The discussion continued in RAN4 94-e-bis [2] as Issue 2-5-1:

- In this paper, we tend to discuss this issue and provide our view.

2	Discussion

Before giving our view, we share some RAN1 agreements as background to this problem.

Agreement: (RAN1 #98bis, 1910) For RRM measurements of a neighbor cell in IDLE, INACTIVE and CONNECTED UE states, · Support signaling of a common Q value per frequency by broadcast RRC signaling (SIBx) and/or dedicated RRC signaling (measObjectNR) from the serving cell. · Support signaling from the serving cell of a Q value for a listed neighbour cell. o If Q is signaled for a listed cell, it overrides any common Q value per frequency Agreement: · No further down selection is made for set of Q values, i.e., Q = {1,2,4,8} (resolves FFS of prior agreement on Q values) · UE does not expect to be configured with two Type-0 PDCCH monitoring occasions in a slot with Q=1. · Note: This does not imply a change in Rel-15 behavior for Type-0 PDCCH monitoring in two consecutive slots. · Note: This agreement does not imply any changes to Rel-15 PDCCH monitoring requirements on BD/CCE limits per slot · RAN4 requirements for SSB detection during initial cell search should be defined assuming Q = 8. This assumption does not have any other specification impact. o Include this agreement in an LS to RAN4 asking them to take this into account. The LS should also provide an overview on the definition of Q. Agreement: (RAN1 #99) The Q value signalled in system information for a serving cell is also used for RRM measurements on the serving cell. Agreement: (RAN1 #99) A serving cell can indicate Q via dedicated RRC signaling (e.g., for SCell or SCG addition).

Based on these RAN1 agreements, especially the ones highlighted in yellow, we believe that Q can be assumed to be known to the UE. Even if the value of Q is not signalled to the UE through broadcast RRC signalling, it’s RAN1 agreement that:

RAN4 requirements for SSB detection during initial cell search should be defined assuming Q = 8.

In our view this statement should cover the case we’re discussing now. Even if under some occasions, the UE doesn’t receive the broadcast successfully, it should assume Q = 8. Thus, we propose that Q can be assumed to be always known to the UE.

Q can be assumed to be always known to the UE.

3	Conclusion

- Proposal 1: Q can be assumed to be always known to the UE.

References

- R4-2002278, WF on NR-U RRM requirements (Part 3)

- R4-2005375, WF on NR-U RRM requirements (Part 3)