3GPP TSG-RAN4 Meeting #95-e	R4-2006189

# Online, 25th May - 5th Jun 2020

Agenda item:	4.9.7

Source:	Apple

Title:		On issues of R15 BWP switching delay requirement

Document for:	Discussion

# Introduction

In R15 BWP switching requirement, the clarification on the duration where UE is not required to perform Rx and Tx has some potential issues and uncertainties. Extending to R16 UE specific CBW switching, we also observed the similar risk.

In this contribution, we discuss the potential issues for R15 BWP switching delay requirement and also extend to R16 UE specific CBW switch.

# Potential issue on R15 RRC based BWP switching

In the current TS38.133, the UE behaviour during RRC based BWP switching is defined as,

For RRC-based BWP switch, after the UE receives RRC reconfiguration involving active BWP switching or parameter change of its active BWP, UE shall be able to receive PDSCH/PDCCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs on the first DL or UL slot right after a time duration of TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length slots which begins from the beginning of DL slot n, where DL slot n is the last slot containing the RRC command, and TRRCprocessingDelay is the length of the RRC procedure delay in millisecond as defined in clause 12 in TS 38.331 [2], and TBWPswitchDelayRRC=[6]ms is the time used by the UE to perform BWP switch. The UE is not required to transmit UL signals or receive DL signals during the time defined by TRRCprocessingDelay+TBWPswitchDelayRRC on the cell where RRC-based BWP switch occurs.

The current stating point of the duration where UE is not requirement to transmit UL is from the beginning of slot of RRC signalling. As shown in figure 1, in fact UE needs to transmit ACK/NACK for the RRC signalling of BWP switching and then UE may be not able to receive or transmit any signal. If ACK cannot be received at network side, network may not change to the new BWP for scheduling this UE after slot m in figure 1, and also it will waste network resource to resend RRC signalling to switching BWP for this UE. In order to avoid this incorrect situation, we propose to revise the requirement to make UE transmit the ACK/NACK of associated RRC signalling.

Figure 1. Potential issue of ACK/NACK feedback during RRC based BWP switching

The UE is not required to transmit UL signals or receive DL signals until the first DL or UL slot occurs right after a time duration of (TRRCprocessingDelay+TBWPswitchDelayRRC -THARQ) which is right after UE transmitting HARQ feedback for associated RRC reconfiguration signalling involving active BWP switching or parameter change of its active BWP, as shown in figure 2. THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213.

Figure 2. UE is not required to perform Rx and Tx after the THARQ

Propose 1: Revise the UE behaviour in current RRC based BWP switching requirement as:

The UE is not required to transmit UL signals or receive DL signals until the first DL or UL slot occurs right after a time duration of (TRRCprocessingDelay+TBWPswitchDelayRRC -THARQ) which is right after UE transmitting HARQ feedback for associated RRC reconfiguration signalling involving active BWP switching or parameter change of its active BWP.

where,

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

# Potential issue on R16 UE specific CBW change

Last meeting RAN4 endorsed one draft CR in [2], however, the R16 UE specific CBW switching also has similar issue as RRC based BWP switching, so the UE behaviour in the CR [2] needs to be further revised as below,

### 8.12.2 UE-specific CBW change delay After the UE receives RRC reconfiguration involving offsetToCarrier or carrierBandwidth change on the old CBW, UE shall be able to receive PDSCH/PDCCH on an active DL BWP or transmit PUSCH on an active UL BWP of the new CBW right after a time duration of TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length slots which begins from the beginning of DL slot n, where DL slot n is the last slot containing the RRC command, and TRRCprocessingDelay is the length of the RRC procedure delay in millisecond as defined in clause 12 in TS 38.331 [2], and TCBWchangeDelayRRC=[6]ms is the time used by the UE to perform CBW change. The UE is not required to transmit UL signals or receive DL signals until the first DL or UL slot occurs right after a time duration of (TRRCprocessingDelay+TBWPswitchDelayRRC -THARQ) which is right after UE transmitting HARQ feedback for associated RRC reconfiguration signalling involving offsetToCarrier or carrierBandwidth change. where, THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

Proposal 2: RAN4 to define the duration within which UE is not required to transmit or receive signals for UE specific CBW change in R16.

# Conclusion

In this contribution, we discuss the potential issues for R15 BWP switching delay requirement and also extend to R16 UE specific CBW switch.

Propose 1: Revise the UE behaviour in current RRC based BWP switching requirement as:

The UE is not required to transmit UL signals or receive DL signals until the first DL or UL slot occurs right after a time duration of (TRRCprocessingDelay+TBWPswitchDelayRRC -THARQ) which is right after UE transmitting HARQ feedback for associated RRC reconfiguration signalling involving active BWP switching or parameter change of its active BWP.

where,

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

Proposal 2: RAN4 to define the duration within which UE is not required to transmit or receive signals for UE specific CBW change in R16.

# References

[1] R4-2005525, Way forward on NR UE power saving performance requirements, CATT, RAN4 #94e-bis meeting

[2] R4 -2005352, Draft CR to TS 38.133: RRM requirement for UE-specific CBW change, Intel, RAN4 #94e-bis meeting