3GPP RAN WG4 Meeting #95-e	R4-2006240

# Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0644 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on gNB Rx-Tx time difference measurement report mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2020-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | gNB Rx-Tx time difference measurement report mapping is missing in 38.133 |  |  |  |  |  |  |  |  |
| Summary of change: |  | gNB Rx-Tx time difference measurement report mapping is introduced in 38.133 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | gNB Rx-Tx time difference measurement report mapping is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New chapter and sections: 11, 11.3, 11.3.1, 11.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 11 Measurement Performance Requirements for NR

### 11.3 gNB Rx-Tx time difference measurement

11.3.1 Measurement accuracy requirements

11.3.2 Measurement report mapping

The reporting range of gNB Rx-Tx time difference is defined from - 985024Tc to 985024Tc with resolution Tc*2^k, where k is a configured value from set {0,1,2,3,4,5}.

The mapping of measured quantity is defined in Table 11.3.2-1 to Table 11.3.2-6 respectively. The range in the signalling may be larger than the guaranteed accuracy range.

Table 11.3.2-1: gNB Rx-Tx report mapping, k=0

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| gNB Rx-Tx_000000 | -985024 > gNB Rx-Tx | Tc |
| gNB Rx-Tx_000001 | -985024 gNB Rx-Tx < -985023 | Tc |
| gNB Rx-Tx_000002 | -985023 gNB Rx-Tx < -985022 | Tc |
|  |  | … |
| gNB Rx-Tx_985022 | -3 gNB Rx-Tx -2 | Tc |
| gNB Rx-Tx_985023 | -2 gNB Rx-Tx -1 | Tc |
| gNB Rx-Tx_985024 | -1 gNB Rx-Tx 0 | Tc |
| gNB Rx-Tx_985025 | 0 < gNB Rx-Tx 1 | Tc |
| gNB Rx-Tx_985026 | 1 < gNB Rx-Tx 2 | Tc |
| gNB Rx-Tx_985027 | 2 < gNB Rx-Tx 3 | Tc |
| … | … | … |
| gNB Rx-Tx_1970047 | 985022 < gNB Rx-Tx 985023 | Tc |
| gNB Rx-Tx_1970048 | 985023 < gNB Rx-Tx 985024 | Tc |
| gNB Rx-Tx_1970049 | 985024 < gNB Rx-Tx | Tc |

Table 11.3.2-2: gNB Rx-Tx report mapping, k=1

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| gNB Rx-Tx_000000 | -985024 > gNB Rx-Tx | Tc |
| gNB Rx-Tx_000001 | -985024 gNB Rx-Tx < -985022 | Tc |
| gNB Rx-Tx_000002 | -985022 gNB Rx-Tx < -985020 | Tc |
|  |  | … |
| gNB Rx-Tx_492510 | -6 gNB Rx-Tx -4 | Tc |
| gNB Rx-Tx_492511 | -4 gNB Rx-Tx -2 | Tc |
| gNB Rx-Tx_492512 | -2 gNB Rx-Tx 0 | Tc |
| gNB Rx-Tx_492513 | 0 < gNB Rx-Tx 2 | Tc |
| gNB Rx-Tx_492514 | 2 < gNB Rx-Tx 4 | Tc |
| gNB Rx-Tx_492515 | 4 < gNB Rx-Tx 6 | Tc |
| … | … | … |
| gNB Rx-Tx_985023 | 985020 < gNB Rx-Tx 985022 | Tc |
| gNB Rx-Tx_985024 | 985022 < gNB Rx-Tx 985024 | Tc |
| gNB Rx-Tx_985025 | 985024 < gNB Rx-Tx | Tc |

Table 11.3.2-3: gNB Rx-Tx report mapping, k=2

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| gNB Rx-Tx 000000 | -985024 > gNB Rx-Tx | Tc |
| gNB Rx-Tx_000001 | -985024 gNB Rx-Tx < -985020 | Tc |
| gNB Rx-Tx_000002 | -985020 gNB Rx-Tx < -985016 | Tc |
|  |  | … |
| gNB Rx-Tx_246254 | -12 gNB Rx-Tx -8 | Tc |
| gNB Rx-Tx_246255 | -8 gNB Rx-Tx -4 | Tc |
| gNB Rx-Tx_246256 | -4 gNB Rx-Tx 0 | Tc |
| gNB Rx-Tx_246257 | 0 < gNB Rx-Tx 4 | Tc |
| gNB Rx-Tx_246258 | 4 < gNB Rx-Tx 8 | Tc |
| gNB Rx-Tx_246259 | 8 < gNB Rx-Tx 12 | Tc |
| … | … | … |
| gNB Rx-Tx_492511 | 985016 < gNB Rx-Tx 985020 | Tc |
| gNB Rx-Tx_492512 | 985020 < gNB Rx-Tx 985024 | Tc |
| gNB Rx-Tx_492513 | 985024 < gNB Rx-Tx | Tc |

Table 11.3.2-4: gNB Rx-Tx report mapping, k=3

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| gNB Rx-Tx_000000 | -985024 > gNB Rx-Tx | Tc |
| gNB Rx-Tx_000001 | -985024 gNB Rx-Tx < -985016 | Tc |
| gNB Rx-Tx_000002 | -985016 gNB Rx-Tx < -985008 | Tc |
|  |  | … |
| gNB Rx-Tx_123126 | -24 gNB Rx-Tx -16 | Tc |
| gNB Rx-Tx_123127 | -16 gNB Rx-Tx -8 | Tc |
| gNB Rx-Tx_123128 | -8 gNB Rx-Tx 0 | Tc |
| gNB Rx-Tx_123129 | 0 < gNB Rx-Tx 8 | Tc |
| gNB Rx-Tx_123130 | 8 < gNB Rx-Tx 16 | Tc |
| gNB Rx-Tx_123131 | 16 < gNB Rx-Tx 24 | Tc |
| … | … | … |
| gNB Rx-Tx_246255 | 985008 < gNB Rx-Tx 985016 | Tc |
| gNB Rx-Tx_246256 | 985016 < gNB Rx-Tx 985024 | Tc |
| gNB Rx-Tx_246257 | 985024 < gNB Rx-Tx | Tc |

Table 11.3.2-5: gNB Rx-Tx report mapping, k=4

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| gNB Rx-Tx_000000 | -985024 > gNB Rx-Tx | Tc |
| gNB Rx-Tx_000001 | -985024 gNB Rx-Tx < -985008 | Tc |
| gNB Rx-Tx_000002 | -985008 gNB Rx-Tx < -984992 | Tc |
|  |  | … |
| gNB Rx-Tx_61562 | -48 gNB Rx-Tx -32 | Tc |
| gNB Rx-Tx_61563 | -32 gNB Rx-Tx -16 | Tc |
| gNB Rx-Tx_61564 | -16 gNB Rx-Tx 0 | Tc |
| gNB Rx-Tx_61565 | 0 < gNB Rx-Tx 16 | Tc |
| gNB Rx-Tx_61566 | 16 < gNB Rx-Tx 32 | Tc |
| gNB Rx-Tx_61567 | 32 < gNB Rx-Tx 48 | Tc |
| … | … | … |
| gNB Rx-Tx_123127 | 984992< gNB Rx-Tx 985008 | Tc |
| gNB Rx-Tx_123128 | 985008< gNB Rx-Tx 985024 | Tc |
| gNB Rx-Tx_123129 | 985024 < gNB Rx-Tx | Tc |

Table 11.3.2-6: gNB Rx-Tx report mapping, k=5

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| gNB Rx-Tx_00000 | -985024 > gNB Rx-Tx | Tc |
| gNB Rx-Tx_00001 | -985024 gNB Rx-Tx < -984992 | Tc |
| gNB Rx-Tx_00002 | -984992 gNB Rx-Tx < -984960 | Tc |
|  |  | … |
| gNB Rx-Tx_30780 | -80 gNB Rx-Tx -64 | Tc |
| gNB Rx-Tx_30781 | -64 gNB Rx-Tx -32 | Tc |
| gNB Rx-Tx_30782 | -32 gNB Rx-Tx 0 | Tc |
| gNB Rx-Tx_30783 | 0 < gNB Rx-Tx 32 | Tc |
| gNB Rx-Tx_30784 | 32 < gNB Rx-Tx 64 | Tc |
| gNB Rx-Tx_30785 | 64 < gNB Rx-Tx 80 | Tc |
| … | … | … |
| gNB Rx-Tx_61563 | 984960< gNB Rx-Tx 984992 | Tc |
| gNB Rx-Tx_61564 | 984992< gNB Rx-Tx 985024 | Tc |
| gNB Rx-Tx_61565 | 985024 < gNB Rx-Tx | Tc |