3GPP TSG-RAN WG4 Meeting #95-e   					                  R4-2006306

Electronic Meeting, 25 May – 5 June,, 2020

Agenda Item: 6.8.2.2

Source: MediaTek

Title: Discussion on impact on existing RRM requirements

Document for:	Discussion

# Introduction

In this contribution, we provide our views on impact on existing RRM requirements.

# Measurement Gap

In WF on UE measurement in RAN4#94e-bis [2], we have the following issue:

- Further discuss when measurement gap is needed for PRS measurement in FR1 and FR2

- Further discuss applicability of R15 measurement gap patterns to NR positioning

  - Option 1: at least Rel-15 NR measurement gaps apply for positioning measurements

  - Other options are not excluded

In WF on RRM impact in RAN4#94e-bis [3], we have the following issue:

- FFS: Introduction of new measurement gap pattern with MGL > 6 ms.

  - Candidate MGL and MGRP if new MG patterns are specified:

  - MGL = {10, 20, 40 and 50} ms

  - MGRP = {80, 160, 320 and 640} ms

  - Combination of MGL and MGRP is FFS

- Independent MG patterns for positioning and RRM measurements is deferred to future release.

Proposal 1: Introduce the following MGL for POS

- MGL: 10ms, 18ms, 34ms, 66 ms

We see the need to introduce new measurement gap for positioning in order to enable more commercial/industrial use cases. We have the following proposal:

In table 1 we explain the design of MGL considering numerology, PRS configuration, RF tuning time, and expected-RSTD. Note that we consider comb-12 PRS, which is capable of multiplexing 12 TRPs in one slot in frequency domain. Thus, the PRS duration is given by (slot length)⋅A⋅B⋅C12.

| SCS | (A) Number of Tx beam | (B) repetition | (C) number of TRPs | PRS duration | + RF tuning time + expected RSTD |
| --- | --- | --- | --- | --- | --- |
| 15 kHz | 4 | 1,2,4 | 19 | 8ms, 16ms, 32ms | 10ms, 18ms, 34ms |
| 15 kHz | 8 | 1,2 | 19 | 16ms, 32ms | 18ms, 34ms |
| 30 kHz | 8 | 1,2,4 | 19 | 8ms, 16ms, 32ms | 10ms, 18ms, 34ms |
| 60 kHz | 16 | 1,2,4 | 19 | 8ms, 16ms, 32ms | 10ms, 18ms, 34ms |
| 120 kHz | 64 | 1,2,4 | 19 | 16ms, 32ms, 64ms | 18ms, 34ms, 66ms |

The RF tuning time is 0.5ms at beginning and end of the gap. We also consider ±0.5ms expected-RSTD. Thus the MGL = PRS duration + 2ms. See the following figure for an illustration of SCS 15kHz, 8 Tx beams, 2 repetitions, and MGL 18ms:

Proposal 2: Introduce the following MGRP for MGL in proposal 1:

- MGRP: 40ms, 80ms, 160ms, 320ms, 640ms, where

- MGRP 40ms is applicable for MGL 10ms

- MGRP 80ms is applicable for MGL 10ms, 18ms

- MGRP 160ms is applicable for MGL 10ms, 18ms, 34m

- MGRP 320ms is applicable for MGL 18ms, 34ms

- MGRP 640ms is applicable for MGL 34ms, 66ms

For MGRP, we have the following proposal:

The consideration is MGL/MGRP < 30%.

Proposal 4: Rel-15 NR measurement gaps apply for NR positioning measurements

Proposal 3: It is an UE capability whether UE supports new MGs

It should be an UE capability whether UE supports new MGs. Thus at least Rel-15 NR measurement gaps should apply for positioning measurements. Otherwise, NR positioning cannot be used for Rel-16 UEs not supporting new MGs.

# Scheduling Restrictions

In [1], we have the following agreements:

- Condition under which no gaps are needed for PRS measurements has been defined:

- If SCSs of reference and neighbor cells’ PRS are the same and the PRS are within the active BWP, and this SCS is same as active BWP SCS, then the UE can measure without gaps and without any scheduling restriction;

- otherwise whether the UE needs gaps/scheduling restriction or not to measure PRS is FFS

- Scheduling restriction under PRS based measurement:

- FR1: UE behavior on scheduling restriction in FR1 shall be the same as agreed for FR2.

- FR2:

  - UE behavior on scheduling restriction in FR2 is according to the following RAN1 agreements:

    - “In case DL PRS Resources are processed in the active BWP and there is no measurement gap configured to the UE, the UE is not expected to process DL PRS in the same OFDM symbol where other DL signals and channels are transmitted to the UE.”

  - Impact of the above rule on the scheduling restriction on PRS based measurement requirements and the priority between data and measurement in RAN4 spec are FFS.

    - If any problem is identified by RAN4 then how to resolve it is FFS.

Proposal 5: UE behavior on scheduling restriction in FR1 shall be the same that for FR2, i.e., UE drops PRS if there exist other DL channels

We have the following proposal:

# Impact of concurrent RRM/PRS processing/measurement

In WF on RRM impact in RAN4#94e-bis [3], we have the following issue:

- FFS: whether concurrent RRM/PRS processing/measurement impacts RRM and/or PRS measurements i.e.

  - Whether concurrent RRM processing and PRS measurement, impacts RRM and/or PRS measurements.

  - Whether concurrent PRS processing and RRM measurement, impacts RRM and/or PRS measurements.

- Candidate options:

  - Option 1: No impact on both intra- and inter-frequency measurements.

  - Option 2: No impact on intra-frequency but impact on inter-frequency measurements.

  - Option 3: impact on intra-frequency and inter-frequency measurements RRM and/or PRS measurement requirements need to be relaxed for both intra- and inter-frequency measurements.

  - Other options are not precluded.

Proposal 6: Concurrent RRM/PRS measurement impacts RRM/PRS measurement only if it involves changing the RF center frequency. The RRM measurement period should be extended in such cases.

Currently, the definition of intra- inter-frequency PRS measurement is not defined yet. In our view, concurrent RRM/PRS measurement impact RRM/PRS measurement only if it involves changing the RF center frequency. In such case, the PRS measurement should be prioritized, and hence some SSB for RRM measurement may be dropped by the UE.  Thus, we have the following alternative proposal:

# Priority between active BWP switching in gap and PRS measurement in gap

In our view this issue can be avoided since triggering of active BWP switching in gap can always be avoided by gNB. In other words, if gNB would like to change active BWP of an UE, the gNB can always trigger it such that the switching does not occur in the measurement gap.

Proposal 7: If UE is configured to do active BWP switching in a gap occasion, then UE performs active BWP switch after the current gap occasion, i.e., UE prioritizes PRS measurement in gap

However, as an error handling, RAN4 may still specify the UE behaviour under such scenario. Thus, we have the following proposal:

# Conclusions

In this contribution, we have the following observations and proposals:

Proposal 1: Introduce the following MGL for POS

- MGL: 10ms, 18ms, 34ms, 66 ms

Proposal 2: Introduce the following MGRP for MGL in proposal 1:

- MGRP: 40ms, 80ms, 160ms, 320ms, 640ms, where

- MGRP 40ms is applicable for MGL 10ms

- MGRP 80ms is applicable for MGL 10ms, 18ms

- MGRP 160ms is applicable for MGL 10ms, 18ms, 34m

- MGRP 320ms is applicable for MGL 18ms, 34ms

- MGRP 640ms is applicable for MGL 34ms, 66ms

Proposal 3: It is an UE capability whether UE supports new MGs

Proposal 4: Rel-15 NR measurement gaps apply for NR positioning measurements

Proposal 5: UE behavior on scheduling restriction in FR1 shall be the same that for FR2, i.e., UE drops PRS if there exist other DL channels

Proposal 6: Concurrent RRM/PRS measurement impacts RRM/PRS measurement only if it involves changing the RF center frequency. The RRM measurement period should be extended in such cases.

Proposal 7: If UE is configured to do active BWP switching in a gap occasion, then UE performs active BWP switch after the current gap occasion, i.e., UE prioritizes PRS measurement in gap

# Reference

[1] R4-1915854 WF on NR positioning RRM

[2] R4-2005378 - WF on NR Positioning UE measurements and reporting

[3] R4-2005379 WF on RRM impact