3GPP TSG-RAN WG4 Meeting #95-e              	R4-2006323

Electronic Meeting, 25th May– 5th June, 2020

Agenda item:	6.17.2.2.1

Source:	Samsung

Title:	Discussion and simulation results for NR HST PUSCH

Document for:	Discussion

# 1	Introduction

During the last RAN4#94b-e meeting, the remain issues about NR HST BS PUSCH demodulation requirement were discussed and test parameters were captured in the WF [1]

In this contribution, the review on remain issues for HST PUSCH requirements are provided, and the updated simulation results are provided for performance requirement derived.

# 2	Discussion and simulation results

2. 1 Discussion

In this section, the view on remain issue of NR HST PUSCH requirements is presented.

1T1R requirements for the tunnel scenario

In last meeting, it is agreed that RAN4 will introduce 1T1R for the tunnel scenario requirements only for conduced requirement.

Regarding the requirement configuration, in existing 1T2R requirement, both MCS 2 and MCS 16 are introduced. Generally, it would be relatively easy for uplink to use more than 2Rx for better tracking and channel estimation performance. Without Rx diversity, the targeting SNR is high with considering additional margin, it may has some test limitation. In that sense, only MCS2 requirement is preferred to introduce for 1T1R in tunnel scenario. Other test parameters can be reused from 1x2 requirement.

Proposal 1: only MCS 2 requirement is preferred to introduce for 1T1R requirement.

Test applicability rule for 1T1R requirement

Regarding the test applicability rule for 1T1R requirement, the following options are considered

- Option 3 : Define test applicability rule in the section 8.1.2.0 of TS 38.141-1 as: Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the tests with low MIMO correlation level shall apply only for the lowest and highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration. - Option 4: Define test applicability rule in the section 8.1.2.0 of TS 38.141-1 as: “In high speed train requirements, unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the tests with low MIMO correlation level shall apply only for the lowest number or two supported connectors, in addition to the highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.”

Since 1T1R is only available for conducted test, the requirements should be available for the lowest number. Considering the 1T1R is only introduced in HST scenario, it is fine to add the condition for defining the test applicability rule. In that sense, we are fine with the option 4

Proposal 2: Define test applicability rule in the section 8.1.2.0 of TS 38.141-1 as:

“In high speed train requirements, unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the tests with low MIMO correlation level shall apply only for the lowest number or two supported connectors, in addition to the highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.”

Other requirements

As agreed in WF [2], the following requirement are considering after March

- Waveform - Focus on CP-OFDM waveform - Start on study of PUSCH requirements with DFT-s-OFDM waveform after March, 2020 - CBW for CP-OFDM - Focus on 10MHz CBW/15KHz SCS, 40MHz CBW/30KHz SCS - Start to work on 5MHz CBW/15KHz SCS, 10Mhz CBW/30KHz SCS after March, 2020 - Similar applicability rule of channel bandwidths as existing PUSCH performance requirements will be used for HST - Fading channel - Focus on the PUSCH requirements with HST scenario firstly - Start after March on study of PUSCH requirements with multi-path fading channel under high Doppler value

In the Rel-15 NR BS demodulation requirement, the minimum channel bandwidth per SCS is defined for DFT-OFDM waveform. The performance difference is minor, compared with CP-OFDM waveform. Since it is agreed to introduce the additional requirements for 5MHz CBW/15KHz, and 10MHz CBW/30KHz SCS for CP-OFDM waveform. The test coverage of HST PUSCH requirement should be enough. Thus, we prefer not to define the HST PUSCH requirement with DFT-s-OFDM waveform.

Proposal 3: No HST PUSCH requirement with DFT-s-OFDM waveform.

In [3], the general applicably rule for HST PUSCH requirements with 350kph and 500kph is under discussion. As agreed, if a BS declare to support 500kph, BS passes the test of 500kph, can also consider the tests of 350kph as passed. Therefore, as for waveform with DFT-s-OFDM, if agreed to introduce, we would like to compact the test case with focusing on the requirement with 500kph only. There is no need to replicate the test cases introduced the requirements of CP-OFDM waveform.

Regarding the MCS, generally, DFT-s-OFDM, it is typical to schedule small value of MCS for guaranteeing the link budget performance. We prefer to re-use the Rel-15 test parameters for HST with DFT-s-OFDM waveform.

Proposal 4: If agreed to introduce HST PUSCH requirement with DFT-s-OFDM waveform, only 500kph requirement is preferred to introduce.

Regarding the fading channel environment, as for the high Doppler value with multipath fading channel, the requirement with high Doppler value about 600Hz, was specified in LTE, which is related with UE 300km/h velocity at Band 1(2.1GHz) carrier frequency. The requirements are only applicable for BS supporting ETU600. Different HST scenario, BS should overcome the impact of multi-path fading, excepting for high Doppler Shift. The performance loss suffered more seriously in multi-path fading channel.

For UE side, it is agreed to define the 600Hz and 1200 Hz for requirement with fading environment under 15KHz and 30KHz SCS, separately. Therefore, 600Hz and 1200Hz can be reused as start point for the feasibility study with HST requirement under high Doppler

Regarding with number of DMRS symbol, in existing Rel-15 NR BS demodulation requirement, 2 DMRS is considered with TDL300-400 fading channel is considered. Considering with large Doppler value, it is reasonable for 3 DMRS symbols should be considered.

Proposal 5:  If agreed to introduce the related requirement, the high Doppler with 600Hz and 1200Hz for 15 KHz and 30 KHz SCS can be regarded as the starting point for the feasibility study with HST requirement with high Doppler

2. 2 Simulation results for HST

In this subsection, the updated ideal and impairment results are provided to requirement finalization

Table 1: Ideal and impairment results for NR HST PUSCH with CP-OFDM for open scenario with 350km/h

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | SNR(@70%) (ideal ) | SNR(@70%) (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 10MHz | 2 | 1T2R | 1+1+1(2,7,11) | AWGN 1340Hz | -5.89 | -3.89 |
| 15KHz, 10MHz | 16 | 1T2R | 1+1+1(2,7,11) | AWGN 1340Hz | 6.87 | 8.87 |
| 30KHz, 40MHz | 2 | 1T2R | 1+1+1(2,7,11) | AWGN 2334Hz | -5.87 | -3.87 |
| 30KHz, 40MHz | 16 | 1T2R | 1+1+1(2,7,11) | AWGN 2334Hz | 6.90 | 8.9 |
| 15KHz, 10MHz | 2 | 1T8R | 1+1+1(2,7,11) | AWGN 1340Hz | -11.29 | -9.29 |
| 15KHz, 10MHz | 16 | 1T8R | 1+1+1(2,7,11) | AWGN 1340Hz | 1.07 | 3.07 |
| 30KHz, 40MHz | 2 | 1T8R | 1+1+1(2,7,11) | AWGN 2334Hz | -11.29 | -9.29 |
| 30KHz, 40MHz | 16 | 1T8R | 1+1+1(2,7,11) | AWGN 2334Hz | 1.10 | 3.1 |

Table 2: Ideal and impairment results for NR HST PUSCH with CP-OFDM for tunnel scenario with 350km/h

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | SNR(@70%) (ideal) | SNR(@70%) (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 10MHz | 2 | 1T2R | 1+1+1(2,7,11) | AWGN 1340Hz | -5.84 | -3.84 |
| 15KHz, 10MHz | 16 | 1T2R | 1+1+1(2,7,11) | AWGN 1340Hz | 7.32 | 9.32 |
| 30KHz, 40MHz | 2 | 1T2R | 1+1+1(2,7,11) | AWGN 2334Hz | -5.83 | -3.83 |
| 30KHz, 40MHz | 16 | 1T2R | 1+1+1(2,7,11) | AWGN 2334Hz | 7.31 | 9.31 |
| 15KHz, 10MHz | 2 | 1T1R | 1+1+1(2,7,11) | AWGN 1340Hz | -2.83 | -0.83 |
| 15KHz, 10MHz | 16 | 1T1R | 1+1+1(2,7,11) | AWGN 1340Hz | 10.58 | 12.58 |
| 30KHz, 40MHz | 2 | 1T1R | 1+1+1(2,7,11) | AWGN 2334Hz | -2.82 | -0.82 |
| 30KHz, 40MHz | 16 | 1T1R | 1+1+1(2,7,11) | AWGN 2334Hz | 10.56 | 12.56 |

Table 3: Ideal results and impairment results for NR HST PUSCH with CP-OFDM for open scenario with 500km/h

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | SNR(@70%) (ideal) | SNR(@70%) (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 10MHz | 2 | 1T2R | 1+1+1(2,7,11) | AWGN 1740Hz | -5.85 | -3.85 |
| 15KHz, 10MHz | 16 | 1T2R | 1+1+1(2,7,11) | AWGN 1740Hz | 7.22 | 9.22 |
| 30KHz, 40MHz | 2 | 1T2R | 1+1+1(2,7,11) | AWGN 3334Hz | -5.81 | -3.81 |
| 30KHz, 40MHz | 16 | 1T2R | 1+1+1(2,7,11) | AWGN 3334Hz | 7.30 | 9.3 |
| 15KHz, 10MHz | 2 | 1T8R | 1+1+1(2,7,11) | AWGN 1740Hz | -11.26 | -9.26 |
| 15KHz, 10MHz | 16 | 1T8R | 1+1+1(2,7,11) | AWGN 1740Hz | 1.36 | 3.36 |
| 30KHz, 40MHz | 2 | 1T8R | 1+1+1(2,7,11) | AWGN 3334Hz | -11.25 | -9.25 |
| 30KHz, 40MHz | 16 | 1T8R | 1+1+1(2,7,11) | AWGN 3334Hz | 1.43 | 3.43 |

Table 4: Ideal and impairment results for NR HST PUSCH with CP-OFDM for tunnel scenario with 500km/h

| SCS& BW | MCS | Tx/Rx | DMRS configuration | Channel | SNR(@70%) (ideal) | SNR(@70%) (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 10MHz | 2 | 1T2R | 1+1+1(2,7,11) | AWGN 1740Hz | -5.69 | -3.69 |
| 15KHz,10MHz | 16 | 1T2R | 1+1+1(2,7,11) | AWGN 1740Hz | 7.62 | 9.62 |
| 30KHz, 40MHz | 2 | 1T2R | 1+1+1(2,7,11) | AWGN 3334Hz | -5.67 | -3.67 |
| 30KHz,40MHz | 16 | 1T2R | 1+1+1(2,7,11) | AWGN 3334Hz | 7.48 | 9.48 |
| 15KHz, 10MHz | 2 | 1T1R | 1+1+1(2,7,11) | AWGN 1740Hz | -2.69 | -0.69 |
| 15KHz, 10MHz | 16 | 1T1R | 1+1+1(2,7,11) | AWGN 1740Hz | 10.63 | 12.63 |
| 30KHz, 40MHz | 2 | 1T1R | 1+1+1(2,7,11) | AWGN 3334Hz | -2.67 | -0.67 |
| 30KHz, 40MHz | 16 | 1T1R | 1+1+1(2,7,11) | AWGN 3334Hz | 10.70 | 12.7 |

2. 3 Initial results for fading channel

In this subsection, the initial investigation for the performance impact on multi-path fading channel with high Doppler scenario. The simulation assumption is provided as follows:

| Parameter | value |
| --- | --- |
|  | FR1 |
| Transform precoding | CP-OFDM |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) |
| DMRS type | type 1 |
| Number of DMRS symbols | 1+1+1 (2, 7,11) |
| symbols length | 14 |
| Start symbol index | 0 |
| Time domain resource allocation type | type A |
| Frequency domain resource | Full RB allocation of the applicable BW |
| MCS index | MCS =2,16 |
| Carrier frequency (GHz) | 4GHz |
| Propagation channel | MCS 12 TDLA 30 ns, [600,700,800]Hz for 15KHz SCS MCS 16 TDLA 30 ns, [600, 700,800]Hz for 15KHz SCS |
| SCS and BW | 15KHz, 10MHz |
| Timing offset | 0 |
| Frequency offset | 0 |
| PTRS | Without PT-RS configuration |
| Code block group | Disabled |
| Frequency hopping | Disabled |
| Limited buffer rate matching | Disabled |
| Number of HARQ transmission | 4 |

Figure 1: Initial BLER of multi-path fading with high Doppler value for PUSCH CP-OFDM waveform

Figure 2:  Initial TP of multi-path fading with high Doppler value for PUSCH CP-OFDM waveform

Figure 3:  Initial BLER of multi-path fading with high Doppler value for PUSCH DFT-s-OFDM waveform

Figure 4:  Initial TP of multi-path fading with high Doppler value for PUSCH DFT-s-OFDM waveform

Observation 1:  The fading channel with high Doppler 600Hz is feasible for MCS2 with configured 3 DMRS symbols.

Observation 2: The performance of MCS 16 under fading channel with large Doppler value suffers large degradation as Doppler increasing.

Observation 3: PUSCH with CP-OFDM waveform and DFT-s-OFDM waveform under fading channel high Doppler value have the similar results.

Proposal 6: If agreed to introduce PUSCH requirement with multi-path fading under high Doppler value, focus on the requirements with CP-OFDM waveform.

# 3	Conclusion

In this contribution, the view of simulation assumption for NR HST requirement was provided. Also, the initial ideal simulation results are provided to check the feasibility of high Doppler.

Proposal 1: only MCS 2 requirement is preferred to introduce for 1T1R requirement.

Proposal 2: Define test applicability rule in the section 8.1.2.0 of TS 38.141-1 as:

“In high speed train requirements, unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the tests with low MIMO correlation level shall apply only for the lowest number or two supported connectors, in addition to the highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.”

Proposal 3: No HST PUSCH requirement with DFT-s-OFDM waveform.

Proposal 4: If agreed to introduce HST PUSCH requirement with DFT-s-OFDM waveform, only 500kph requirement is preferred to introduce.

Proposal 5:  If agreed to introduce the related requirement, the high Doppler with 600Hz and 1200Hz for 15 KHz and 30 KHz SCS can be regarded as the starting point for the feasibility study with HST requirement with high Doppler

Observation 1:  The fading channel with high Doppler 600Hz is feasible for MCS2 with configured 3 DMRS symbols.

Observation 2: The performance of MCS 16 under fading channel with large Doppler value suffers large degradation as Doppler increasing.

Observation 3: PUSCH with CP-OFDM waveform and DFT-s-OFDM waveform under fading channel high Doppler value have the similar results.

Proposal 6: If agreed to introduce PUSCH requirement with multi-path fading under high Doppler value, focus on the requirements with CP-OFDM waveform.

# References

R4-2005534 WF on Rel-16 NR HST BS demodulation requirements, Nokia