3GPP TSG-RAN WG4 Meeting #95-e                                    R4-2006375

Electronic Meeting, 25th May – 5th June, 2020

Agenda item:			6.11.2.3

Source:			Samsung

Title:	Discussion on MAC-CE based spatial relation update for aperiodic SRS

Document for:		Approval

# 1	Introduction

The Rel-16 MIMO enhancement work item was approved in RAN#80 with a list of enhancement areas including specifying enhanced mechanism on UL and/or DL transmit beam selection to reduce latency and overhead, i.e.,

<Captured in latest Rel-16 eMIMO WID> - Perform study and, if needed, specify enhancement(s) on UL and/or DL transmit beam selection specified in Rel-15 to reduce latency and overhead

During RAN4#93 and RAN4#94-e meeting, the following agreement is achieved on MAC-CE based spatial relation update for aperiodic SRS [5]:

<Way Forward [5]> - 3.3.1 MAC-CE based spatial relation update for aperiodic SRS: - MAC-CE based spatial relation update for aperiodic SRS: - FFS RAN4 RRM requirement impact is identified from the mechanism of spatial relation updates for AP-SRS via MAC-CE <Way Forward [6]> - Issue 3-2: RAN4 RRM requirement impact due to MAC-CE based spatial relation update for aperiodic SRS - Conclusion: The decision on whether to define MAC-CE based spatial relationship update for AP-SRS should be made in ‘NR RRM core requirement enhancement WI’ scope. Postpone discussion in the scope of NR eMIMO WI scope and wait for respective conclusions.

In this discussion paper, we would like to provide the proposal to complete the discussion on MAC-CE based spatial relation update for aperiodic SRS.

# 4	Discussion

As mentioned above, RAN4 has the conclusion that whether to define MAC-CE based spatial relationship update for AP-SRS should be made in ‘NR RRM core requirement enhancement WI’ scope. In RAN4#94-bis-e meeting, RAN4 has concluded that how to introduce RRM requirement for UL spatial relationship information switching, in which MAC-CE based spatial relationship update for aperiodic SRS is discussed as one of the scenarios, however limited interests on this scenario could be observed based on the discussion in RRM enhancement WI:

| # | Scenario | PHY channel | Triggering method | associated source | Whether to introduce requirement |
| --- | --- | --- | --- | --- | --- |
| 1 | PUCCH-RRC-DL | PUCCH | RRC | DL RS | No |
| 2 | PUCCH-RRC-UL | PUCCH | RRC | UL SRS | No |
| 3 | PUCCH-MAC-DL | PUCCH | MAC | DL RS | Yes |
| 4 | PUCCH-MAC-UL | PUCCH | MAC | UL SRS | No |
| 5 | PUSCH-DCI | PUSCH | DCI | - | No |
| 6 | pSRS-RRC-DL | P-SRS | RRC | DL RS | Yes |
| 7 | pSRS-RRC-UL | P-SRS | RRC | UL SRS | No |
| 8 | spSRS-MAC-DL | SP-SRS | MAC | DL RS | Yes |
| 9 | spSRS-MAC-UL | SP-SRS | MAC | UL SRS | No |
| 10 | aSRS-DCI-DL | A-SRS | DCI | DL RS | Yes |
| 11 | aSRS-DCI-UL | A-SRS | DCI | UL SRS | No |
| 12 | aSRS-MAC-DL | A-SRS | MAC | DL RS | FFS |

Considering DCI triggered Aperiodic SRS’s spatial relationship update will be covered while the scenario of MAC CE triggered SP-SRS will also be covered, we see there is no necessity to introduce additional RAN4 requirement for MAC-CE based spatial relation update for aperiodic SRS, especially considering the defined requirement has limited restriction on UE behavior in terms of conformance testing.

Proposal 1: No new RRM requirement is introduced for the feature of MAC-CE based spatial relation update for aperiodic SRS.

# 5 Conclusion

In this discussion paper, we provided our proposal to complete the discussion on MAC-CE based spatial relation update for aperiodic SRS:

Proposal 1: No new RRM requirement is introduced for the feature of MAC-CE based spatial relation update for aperiodic SRS.

# 6 Reference

[1] RP-192271,	“Revised WID: Enhancements on MIMO for NR”, Samsung.

[2] RP-192270,	“Status Report for WI: Enhancements on MIMO for NR”, Samsung

[3] R4-1912742, “WF on RRM requirement scope for Rel-16 NR eMIMO WI”, Samsung, Huawei.

[4] R4-1911392, “Discussion on Enhancement on UL/DL Transmit Beam Selection with Reduced Latency and Overhead”, Samsung.

[5] R4-1915850, “Way forward on NR eMIMO RRM”, Samsung.

[6] R4-2002242, “WF on NR eMIMO RRM requirement”, Samsung.

[7] R4-2005340, “WF on R16 NR RRM enhancements - UL Spatial Relation Info Switching”, MediaTek.