3GPP TSG-RAN WG4 Meeting #95-e                                                         R4-2006410

Online, 25th May-5th June, 2020

Source:	Samsung, KDDI, LGE

Title:	TP for TR 37.716-21-21: update MSD values for DC_18_n3-n77

Agenda item:			8.7.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-21 to update the MSD values due to IMD for DC_18_n3-n77 based on last meeting approved TP [1].

# 2. Reference

- R4-2003142, TP for TR 37.716-21-21: DC_18_n3-n77, Samsung, KDDI.

# 3. Text Proposal

<Start of Text Proposal>

## 6.116	DC_18_n3-n77

<Unchanged sections omitted>

### 6.116.5	MSD

Table 6.116.5-1 shows the required MSD in the following table, comes from evaluation results.

Table 6.116.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n3A-n77A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n3A-n77A | 18 | IMD3 | ¦2*fB18 +fn3¦ | 820 | 5 | 25 | 865 | N/A |
|  | n3 |  |  | 1770 | 5 | 25 | 1865 |  |
|  | n77 |  |  | 3410 | 10 | 50 | 3410 | 16.3 |
|  | 18 | IMD3 | ¦2*fB18 -fn77¦ | 820 | 5 | 25 | 865 | N/A |
|  | n77 |  |  | 3505 | 10 | 50 | 3505 |  |
|  | n3 |  |  | 1770 | 5 | 25 | 1865 | 15.7 |
|  | 18 | IMD5 | ¦2*fB18 -3*fn3¦ | 820 | 5 | 25 | 865 | N/A |
|  | n3 |  |  | 1725 | 5 | 25 | 1820 |  |
|  | n77 |  |  | 3535 | 10 | 50 | 3535 | 3.7 |

<End of Text Proposal>