3GPP TSG-RAN WG4 Meeting #94bis-e                                                          R4-2006425

Online, 25th May-5th June, 2020

Source:	Samsung, KDDI

Title:	TP for TR 38.716-01-01: CA_n77(2A)_UL_n77(2A)

Agenda item:			8.1.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.716-01-01 to include CA_n77(2A)_UL_n77(2A ) according to the request in [1].

# 2. Reference

- RP-200101, Revised WID on Rel-16 NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y).

# 3. Text Proposal

<Start of Text Proposal>

## 6.x	CA_2DL_n77(2A)_ 2UL_n77(2A)

### 6.x.1 Operating band for CA

Table 6.x.1-1: intra-band non-contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n77 | n77 |

### 6.x.2 Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported bandwidth combinations for CA_2DL_n77(2A)_2UL _n77(2A)

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n77(2A) | CA_n77(2A) | 20, 40, 80, 100 | 20, 40, 80, 100 | 200 | 0 |

### 6.x.3 UE maximum output power for CA

Table 6.x.3-1 UE Power Class for intra-band non-contiguous CA

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n77(2A) |  |  |  |  | 23 | +2/-3 |  |  |

### 6.x.4 Spurious emission for Co-existence band

### There are no co-existence issues for these combinations

### 6.x.5 REFSENS

### There are no REFSENS exceptions for this combination. UL configuration for REFSENS is listed below.

Table 6.x.5-1: Intra-band non-contiguous CA with two uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | UL SCC allocation | ΔR2UL_PCC (dB) | ΔR2UL_SCC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n77(2A) | NOTE 1 | NOTE 2 | NOTE 3 | NOTE 4 | 0.0 | 0.0 | TDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.6A.1-3. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The SCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. |  |  |  |  |  |  |  |

<End of Text Proposal>