3GPP TSG-RAN4#95	R4-2006492

25th May – 05th June 2020

Source: 	Nokia

Title: 						CA_n48B A-MPR

Agenda Item: 			8.1.2

Document for:	Approval

# 1	Introduction

This contribution builds on results that were presented in [1]. Some of the content of [1] is re-produced here for convenience.

# 2	Discussion

## 2.1	A-MPR concept

For CA_n48B lower/higher A-MPR regions are proposed similarly as for CA_48C.

Figure 1: High and low A-MPR scenarios illustrated.

## 2.2	Simulation Results

### 2.3.1	Comparison of modulation type

Figure 3: Comparison of modulation type CP-OFDM (non-contiguous RA)

Figure 4: Comparison of modulation type DFT-s-OFDM (non-contiguous RA)

Observation 1: As can be seen from Figures 3 and 4 modulation type does not in-practise effect to A-MPR when aggregated BW is the same.

### 2.3.2	Comparison of SCS

Figure 5: Comparison of SCS (15, 30, 60 kHz) CP-OFDM (non-contiguous RA)

Observation 2: As can be seen from Figure 5 scs does not in-practise effect to A-MPR when aggregated BW is the same.

## 2.4	Comparison of results

Maximum A-MPR per CC combination is collected in Table 4 and Figure 5. Color coding is used to highlight what is the main reason for A-MPR. Further, yellow and green colors separate A-MPR ranges from 2 to 3.5 dB and from 4 to 6.5 dB when limiting factor is fifth order regrowth.

Table 4: max A-MPR

|  | IMD3 | -40 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IMD5 | -40 |  |  |  |  |  |  |  |  |
|  | regrowth3 | -40 | max A-MPR |  |  |  |  |  |  |  |
|  | regrowth5 | -40 | Edge allocation |  |  |  | Mid-band allocation |  |  |  |
|  |  |  | DFT-s-OFDM |  | CP-OFDM |  | DFT-s-OFDM |  | CP-OFDM |  |
| CC BW | CC BW | aggregated BW | non-cont RA | Cont RA | non-cont RA | Cont RA | non-cont RA | Cont RA | non-cont RA | Cont RA |
| 5 | 10 | 15 | 10 | 4 | 10,5 | 4,5 | 10 | 2 | 10,5 | 2,6 |
| 10 | 5 | 15 | 10 | 4,5 | 10,5 | 4,5 | 10 | 2,5 | 10,5 | 2,8 |
| 5 | 15 | 20 | 10 | 6 | 10,5 | 6,5 | 10 | 2,5 | 10,5 | 2,8 |
| 15 | 5 | 20 | 10 | 6 | 10,5 | 6,5 | 10 | 2,5 | 10,5 | 2,8 |
| 10 | 10 | 20 | 10,5 | 6,5 | 11 | 6,5 | 10,5 | 3 | 11 | 3 |
| 5 | 20 | 25 | 16,5 | 8,5 | 17 | 9 | 10 | 3 | 10,5 | 3 |
| 20 | 5 | 25 | 16,5 | 8,5 | 17 | 8,5 | 10 | 2,5 | 10,5 | 3 |
| 10 | 15 | 25 | 16,5 | 8,5 | 17 | 9 | 10 | 3 | 10,5 | 3,5 |
| 15 | 10 | 25 | 16,5 | 8,5 | 17 | 8,5 | 10 | 3,5 | 10,5 | 3,5 |
| 10 | 20 | 30 | 16,5 | 8,5 | 17 | 9 | 10,5 | 3 | 10,5 | 3,5 |
| 20 | 10 | 30 | 16,5 | 8,5 | 17 | 9 | 10 | 3,5 | 10,5 | 3,5 |
| 15 | 15 | 30 | 16,5 | 8,5 | 17 | 9 | 10 | 3,5 | 10,5 | 3,5 |
| 15 | 20 | 35 | 16,5 | 8,5 | 17 | 9 | 10 | 3,5 | 10,5 | 3,5 |
| 20 | 15 | 35 | 16,5 | 8,5 | 17 | 9 | 10 | 3,5 | 11 | 3,5 |
| 20 | 20 | 40 | 16,5 | 9 | 17 | 9 | 10,5 | 3,5 | 10,5 | 3,5 |

Figure 5: QPSK Edge and Lower A-MPR plots for all aggregated bandwidths

## 2.5	How to write A-MPR

Proposal is to define A-MPR as four different types similarly as LTE CA_48C.

- Low A-MPR range contiguous allocation (center of the band scenario)

- High A-MPR range non-contiguous allocation (edge of the band scenario)

- Low A-MPR range non-contiguous allocation (center of the band scenario)

- High A-MPR range contiguous allocation (edge of the band scenario)

In order to define the AMPR, the following parameters are applied to consider the CC-wise allocations and SCSs

- LCRB = LCRB1*2mu1 + LCRB2*2mu2,

- RBStart =	{RBStart1*2mu1, LCRB1 > 0;

NRB1*2mu1 + RBStart2*2mu2, LCRB1 = 0},

- NRB_agg = NRB1*2mu1 + NRB2*2mu2,

where mu1 = log2(SCS1 / 15 kHz) and mu2 = log2(SCS2 / 15 kHz).

### 2.5.1	Low A-MPR range contiguous allocation (center of the band scenario)

Inner 1, 0 dB

Inner 2, 2 dB

3.5 dB

Figure 6: Low A-MPR range contiguous allocation

Lower A-MPR range FOR ALL modulations and scs where

Lower A-MPR range is when Fedge, high + BWChannel_CA ≤ 3710 MHz or Fedge, low - BWChannel_CA ≤ 3540 MHz

if allocation is inner1 then A-MPR = 0 dB

where Inner1 is defined as

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB_agg – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_agg /2)

else

if allocation is inner2 then A-MPR = 2 dB

where Inner2 is defined as

RBStart,Low = max(1, floor(LCRB/6))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB_agg – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(3NRB_agg /4)

else

A-MPR=3.5 dB

### 2.5.2	High A-MPR range contiguous allocation (Edge of the band scenario)

C

R

B

A

Figure 7: High A-MPR range contiguous allocation

Higher A-MPR range FOR ALL modulations and scs

A-MPR = 0 dB in region R.

Boundaries of region R:

A: RBStart = NRB_agg /4

B: LCRB = NRB_agg/4

C: RBStart = NRB_agg 3/4  LCRB

Region condition:

R: NRB_agg /4 < RBStart < NRB_agg 3/4  LCRB  AND LCRB < NRB_agg/4

Outside region R,

when BWagg ≤ 20 MHz, A-MPR = 7 dB or

when BWagg > 20 MHz, A-MPR = 9 dB.

### 2.5.3	Low A-MPR range non-contiguous allocation (center of the band scenario)

Figure 8: Low A-MPR range non-contiguous allocation all CH BWs

A = NRB_alloc / NRB_agg

A-MPR= 	11.00;         			0.00 <= A <= 0.10

11.75 - 7.50 A; 		0.10 < A <= 0.90

5.00;           			0.90 < A <= 1.00

### 2.5.4	High A-MPR range non-contiguous allocation (edge of the band scenario)

Figure 9: High A-MPR range non-contiguous allocation up to 20 MHz aggregated BW

A = NRB_alloc / NRB_agg

A-MPR= 		11.00;           			0.00 <= A <= 0.10

11.60  - 6.00 A; 		0.10 < A <= 0.60

8.00;           			0.60 < A <= 1.00

Figure 10: High A-MPR range non-contiguous allocation  20 MHz < aggregated BW  ≤ 40 MHz

A = NRB_alloc / NRB_agg

A-MPR= 		18.00 - 13.33 A; 		0.00 <= A <= 0.30

15.71  - 5.71 A; 		0.30 < A <= 1.00

# 3	Conclusion

We propose to define CA_n48B A-MPR as presented in 2.5.1…2.5.4.

# 4	Reference

[1] R4-2000520, CA_n48B A-MPR, Nokia, RAN4#94-e