3GPP TSG-RAN WG4 Meeting # 95-e    	                                        R4-2006555

Electronic Meeting, 25 May – 5 June, 2020

Agenda Item:	6.8.2.2.

Source:	Intel Corporation

Title:	Further discussion on UE RRM impacts due to NR Pos measurement

Document for:		Discussion

# Introduction

In the last RAN4 meetings some agreements of the impacts due to NR positioning measurement on the existing RRM requirements were achieved in [1]. However there are still multiple remaining issues for the aspects below.

- New measurement gap patterns for positioning

- Concurrent PRS processing and RRM measurements

- Active BWP change during positioning measurements

Therefore, in this contribution we provided more considerations on these open issues above.

# New measurement gap patterns for positioning

In [1] the following agreements of the new gap measurement patterns for PRS measurement were achieved.

- FFS: Introduction of new measurement gap pattern with MGL > 6 ms. - Candidate MGL and MGRP if new MG patterns are specified: - MGL = {10, 20, 40 and 50} ms - MGRP = {80, 160, 320 and 640} ms - Combination of MGL and MGRP is FFS - Independent MG patterns for positioning and RRM measurements is deferred to future release.

It was noted that beside the new gap pattern definition itself there are still more efforts needed to address other relevant issues (e.g. gap sharing factors and UE behaviours). More importantly because of these new independent gap patterns the potential impacts below due to the MGL>6ms shall be investigated also.

- Gap sharing between RRM and positioning measurements is based on CSSF defined in Rel-15 of TS 38.133. - Details of CSSF for gap sharing between RRM and positioning measurements are FFS. - Details on extending measurement periods of RRM and positioning measurements due to gap sharing are FFS.

On the other hand, regarding to the limited time for this WI in Rel16, it is quite challenging to resolve these open issues for the new gap pattern of PRS measurement. It is better to defer them beyond Rel16.

Proposal 1: The new measurement gap for PRS measurement can be FFS beyond Rel16.

# BWP switching in PRS measurements

In the previous RAN4 meetings, the problems when UE active BWP change happened during PRS measurement were initiated. The key concern is that UE can’t perform any PRS measurement when UE needs to change its RF operation to other active BWP. Several open issues below were raised in the last meeting [1].

- FFS: Impact on positioning measurement being performed within the active BWP if the active BWP switching interrupts any PRS and/or SRS configured for that positioning measurement. - FFS: priority between active BWP switching in gap and PRS measurement in gap

However, as illustrated in Figure1 such problem will be valid for some intra-frequency layer scenario but not for others. For an example, when the BWP switching without the central frequency change, PRS measurement can be performed without gap. Thus, there is little impacts on intra-frequency PRS measurements due to the concurrent BWP switching.

(a)

(b)

Figure 1. PRS intra-frequency measurement w/o gap when BWP switching

On the other hand, at least if the measurement gap is needed for inter-frequency PRS measurement as illustrated in Figure 2 below, the potential collision between the ongoing BWP switching and UE PRS measurement within a gap will be possible because of RF retuning needed for PRS measurements.

(a)

(b)

Figure 2. PRS inter-frequency measurement with gap when BWP switching

In summary, whether the PRS measurement during the BWP switching is feasible or not can be summarized for several typical scenarios in Table 1.

Table 1.

|  | Centre frequency of active BWP is changed? | BWP size of active BWP is changed? | Non-gap or gap assisted PRS measurement | Concurrent PRS measurement is feasible or not |
| --- | --- | --- | --- | --- |
| Scenario 1a | No | Yes | Non-gap | Yes |
| Scenario 1b | Yes | Yes | Non-gap | No |
| Scenario 2a | No | Yes | Gap-assisted | No |
| Scenario 2b | Yes | Yes | Gap-assisted | No |

Observation 1: Whether there is impact on positioning measurement being performed within the active BWP depends on BWP switching scenario and PRS measurement types (non-gap or gap-assisted). For an example, there is no any impacts on PRS measurement from the BWP switching when

- BWP switching without central frequency change and

- Non-gap assisted PRS measurement

Moreover from the realistic NR positioning deployments, the assistance message which include PRS configuration can be known by NW and UE itself before PRS measurement by the assistance data delivery procedure [3].For an example, LMF can obtain the measured cell’s PRS configuration by “LMF-initiated assistance data delivery procedure” firstly. Then LMF can forward PRS configuration for all measured cells to UE by “LMF-initiated Location Information Transfer Procedure” accordingly.

Figure 8.12.3.2.2.1-1: LMF-initiated Assistance Data Delivery Procedure [TS38.305]

Figure 8.12.3.3.1-1: LMF-initiated Location Information Transfer Procedure[TS38.305]

From UE implementation itself, when UE was indicated the active BWP switching by the serving gNB, it can deprioritize PRS measurement and skip the next gap-assisted PRS measurement autonomously. And as the typical RF transition time for BWP switching was 200us and 900us [4] for intra-band and inter-band respectively, there is at most only one PRS measurement period need to be extended to avoid the collision between them.

Observation 2: If the active BWP switching happened when PRS measuring with gap, UE need NOT to perform the gap-assisted measurement during the current PRS period.

Figure 3. Additional PRS measurement delay due to BWP switching

For RAN4 RRM requirements, the additional time for BWP switching (e.g. ≤1 Tprs) shall be counted into the PRS measurement delay requirements.

Observation 3: For RAN4 requirements on PRS measurement delay, the additional time for BWP switching (e.g. 1 Tprs) shall be extended when BWP switching happened during UE performing PRS measurements on the neighbor cells.

# Concurrent PRS processing and RRM measurements

This problem is similar as the issue we discussed in Section 3. However, fortunately it has been addressed in RAN1 and agreed in TS 38.214[5].

#### 5.1.6.5 PRS reception procedure …. The UE assumes that for the serving cell the DL PRS is not mapped to any symbol that contains SS/PBCH. If the time frequency location of the SS/PBCH block transmissions from non-serving cells are provided to the UE then the UE also assumes that the DL PRS is not mapped to any symbol that contains the SS/PBCH block of the non-serving cell.

It is clear that with the SSB assistance data for neighbor TRPs provided, UE can assume that the PRS symbols are not mapped to SS/PBCH symbols. However, UE may not complete PRS measurement operations within the duration of PRS symbols. That is even if there is no any overlapping between PRS symbols and other signals for RRM measurements (e.g. SSB) it is possible to introduce some impacts on other NR RRM measurements (e.g. SSB) due to PRS measurement. Especially, if there are multiple resource and positioning layers to be measurement, it is more challenging to require UE to finish PRS measurement immediately after the end of PRS symbols as illustrated in Figure 4 below.

Figure 4. PRS measurement impacts on other RRM measurement

Observation 4: When PRS resource boundary is close to other RRM reference signal (e.g. SSB and CSI-RS) , up to UE processing capability the impact of PRS processing (e.g. PRS measurement gap request) on other RRM measurements is possible.

Thus, the restriction agreed in RAN1 above is not enough to avoid the collision between PRS measurements and other SSB measurement completely. Similarly the impact due to RRM processing on PRS measurement is also valid. But both of them can be immigrated with the coordination between LMF and gNB indeed.

However, regarding to the timeline for Rel16 WI, we can suggest the further solution can be studied in the future.

Proposal 2: The problem and potential solutions of concurrent PRS processing and RRM measurements can be FFS beyond Rel16.

From RAN4 perspective, we can propose that:

Proposal 3: The existing SSB and CSI measurement requirements can be applied under the following condition:

- “DL PRS to be measured is not mapped to any symbol that contains SS/PBCH and the adjacent X symbols”

# Conclusion

In this contribution, the consideration on the remaining issues for RRM impacts due to PRS measurements is provided and the following observations and proposals can be drawn:

Proposal 1: The new measurement gap for PRS measurement can be FFS beyond Rel16.

Observation 1: Whether there is impact on positioning measurement being performed within the active BWP depends on BWP switching scenario and PRS measurement types (non-gap or gap-assisted). For an example, there is no any impacts on PRS measurement from the BWP switching when

- BWP switching without central frequency change and

- Non-gap assisted PRS measurement

Observation 2: If the active BWP switching happened when PRS measuring with gap, UE need NOT to perform the gap-assisted measurement during the current PRS period.

Observation 3: For RAN4 requirements on PRS measurement delay, the additional time for BWP switching (e.g. 1 Tprs) shall be extended when BWP switching happened during UE performing PRS measurements on the neighbor cells.

Observation 4: When PRS resource boundary is close to other RRM reference signal (e.g. SSB and CSI-RS) , up to UE processing capability the impact of PRS processing (e.g. PRS measurement gap request) on other RRM measurements is possible.

Proposal 2: The problem and potential solutions of concurrent PRS processing and RRM measurements can be FFS beyond Rel16.

Proposal 3: The existing SSB and CSI measurement requirements can be applied under the following condition:

- “DL PRS to be measured is not mapped to any symbol that contains SS/PBCH and the adjacent X symbols”

# References

- R4-2005379, “Way forward on NR Positioning RRM impacts”

- R4-1913460, “Discussion on NR PRS RSTD Requirements for UE”

- 3GPP TS38.305 v16.0.0, “NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN”

- R4-1702029, “Reply LS on UE RF Bandwidth Adaptation in NR：

- 3GPP TS 38.214 v16.0.0

- 3GPP TS 36.133 v15.8.0: "Evolved Universal Terrestrial Radio Access (E-UTRA): Requirements for support of radio resource management "