3GPP TSG-RAN WG4 Meeting #95-e                 	 R4-2006572

Electronic Meeting, 25 May – 5 June, 2020

Agenda Item:	6.13.2.1

Source:	MediaTek Inc.

Title:	Interruption for Tx switching between two uplink carriers

Document for:		Discussion

# Introduction

In last meeting, the interruption requirements due to Tx switching between 2 UL carriers was discussed. A WF [1] was agreed. The remaining open issues are listed below.

- DL interruption length

- DL interruption location/starting point

In this paper, we provide our view on above issues.

# Discussion

DL interruption length

The number of OFDM symbols to be interrupted can be calculated based on UE’s capability reporting on the switch period length plus some margin due to timing advance. For an example, if switch period occurs at the beginning of UL slot #n, as shown in Figure 1, the # of interrupted DL symbol can be calculated by switch period+1 symbolsymbol duration, where 1 is added because the timing difference between DL and UL slots may not perfectly be a multiple of OFDM symbol duration. The exact numbers of Interruption length in us and in symbol for different switch periods and SCS are provided in Table 1.

Figure 1. Calculation of # of interrupted DL symbols

Table 1 Interruption length in us and in symbol for different switch periods and SCS

| Switch period | SCS (KHz) | 1 symbol duration (us) | interruption length (us) | interruption length (symbol) |
| --- | --- | --- | --- | --- |
| 35 | 15 | 72 | 107 | 2 |
|  | 30 | 36 | 71 | 2 |
|  | 60 | 18 | 53 | 3 |
| 140 | 15 | 72 | 212 | 3 |
|  | 30 | 36 | 176 | 5 |
|  | 60 | 18 | 158 | 9 |
| 210 | 15 | 72 | 282 | 4 |
|  | 30 | 36 | 246 | 7 |
|  | 60 | 18 | 228 | 13 |

Proposal 1: The length of the interruption is switch period+1 symbolsymbol duration symbols.

DL interruption location/starting point

Regarding the location of the interruption, it should the OFDM symbols fully or partial overlapped with the UL switching period, as shown in Figure 1.

Proposal 2: The location of the interruption equals the OFDM symbols fully or partial overlapped with the UL switching period

# Summary

In this paper, we provide our view on the interruption requirements for Tx switching between two uplink carriers. We have the following proposals.

Proposal 1: The length of the interruption is switch period+1 symbolsymbol duration symbols.

Proposal 2: The location of the interruption equals the OFDM symbols fully or partial overlapped with the UL switching period

# Reference

[1] R4-2005416, “WF on RRM requirements for Tx switching between two uplink carriers”, Huawei, HiSilicon