3GPP TSG RAN WG4 Meeting #95-e	R4-2006689

Online, 25th May – 5th June 2020

Agenda item:	8.4.2

Source:	SoftBank Corp., LG Electronics Inc.

Title:	TP update for TR 37.716-21-11: DC_1-11_n3

Document for:	Approval

1	Introduction

3 Band EN-DC of DC_1-11_n3 was approved in RAN4#94Bis [1] but REFSENS exceptions have TBD parts. This TP is to update the REFSENS exception results for 3rd IMD and reflect the REFSENS exception considering the spectrum allocation in Japan since B11 is the specific band used in only Japan.

2	Text Proposal

[Unchanged Parts Skipped]

5.1.x	DC_1-11_n3

### 5.1.x.1	Operating bands for DC_1-11_n3

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-11_n3 | CA_1-11 | n3 | DC_1_n3 |

### 5.1.x.2	Configurations for DC_1-11_n3

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-11A_n3A | DC_1A_n3A DC_11A_n3A | CA_1A-11A | n3A |

### 5.1.x.3	Co-existence studies

### When Uplink EN-DC configuration is DC_1A_n3A, (1) IMD3 of (B1 - n3) will fall into Rx band of Band 11. When Uplink EN-DC configuration is DC_11A_n3A, (2) IMD3 of (B11 - n3) will fall into Rx band of Band 1.

### 5.1.x.4	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.1.x.4-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n3 | 1 | 0.3 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |

Table 5.1.x.4-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-11_n3 | 1 | 0 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |

### 5.1.x.5	REFSENS requirements

Band 11 is the specific band used in only Japan. Considering the spectrum allocation in Japan, only IMD3 of B1 and n3 to Band11 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-11A_n3A | 1 | 1960 | 5 | 25 | 2150 | N/A | FDD | N/A | Yes |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | TDD | N/A | Yes |
|  | 11 | 1432 | 5 | 25 | 1480 | 15.2 | FDD | IMD3 |  |

3.  Reference

[1]  R4-2003451  TP for TR 37.716-21-11: EN-DC_1-11_n3