3GPP TSG-RAN WG4 Meeting # 95-e 	R4-2006697

# Electronic Meeting, 25th May – 5th June, 2020

Source:	Verizon, Samsung

Title:	TP for TR 38.716-02-00: CA_n77-n261

Agenda item:	8.2.3

Document for:	Approval

# Introduction

In RAN 87-e meeting, the revised WID “Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2)” [1] was approved. This contribution is a text proposal for TR 38.716-02-00 to include CA_n77A-n261 CA band combination.

# Reference

[1] 	RP-200168 [RAN 87-e] Revised WID for NR CA_DC 2 band DL with up to 2 band UL, ZTE

# Text Proposal

……..

8	FR1+FR2 Carrier Aggregation: Specific Band Combination Part

<Start of Text Proposal>

## 8.17			CA_n77-n261

### 8.17.1		Common for 1 band UL and 2 bands UL CA

#### 8.17.1.1		Operating bands for CA

Table 8.17.1.1-1: CA band combination of band n77+n261

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |

#### 8.17.1.2		Channel bandwidths per operating band for CA

Table 8.17.1.2-1: Supported bandwidths per CA band combination of band n77+n261

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  | 25 MHz | 30 MHz |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz |  | 400 MHz | BCS |
| CA_n77A-n261A | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n77A-n261D | CA_n77A-n261A CA_n77A-n261D | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |  |  |  |
|  |  | n261 | See CA_n261D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261G | CA_n77A-n261A CA_n77A-n261G | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |  |  |  |
|  |  | n261 | See CA_n261G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261H | CA_n77A-n261A CA_n77A-n261G CA_n77A-n261H | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |  |  |  |
|  |  | n261 | See CA_n261H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261I | CA_n77A-n261A CA_n77A-n261G CA_n77A-n261H CA_n77A-n261I | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |  |  |  |
|  |  | n261 | See CA_n261I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261J | CA_n77A-n261A CA_n77A-n261G CA_n77A-n261H CA_n77A-n261I CA_n77A-n261J | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 |  | See CA_n261J in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261K | CA_n77A-n261A CA_n77A-n261G CA_n77A-n261H CA_n77A-n261I CA_n77A-n261J CA_n77A-n261K | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 |  | See CA_n261K in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261L | CA_n77A-n261A CA_n77A-n261G CA_n77A-n261H CA_n77A-n261I CA_n77A-n261J CA_n77A-n261K CA_n77A-n261L | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261L in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261M | CA_n77A-n261A CA_n77A-n261G CA_n77A-n261H CA_n77A-n261I CA_n77A-n261J CA_n77A-n261K CA_n77A-n261L CA_n77A-n261M | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261M in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(2A) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(2A) in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(2G) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(2G) in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(2H) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(2H) in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(2I) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(2I) in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(3A) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(3A) in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(4A) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(4A) in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(A-G) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(A-G) in Table 5.5A.2-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(A-H) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(A-H) in Table 5.5A.2-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(A-I) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(A-I) in Table 5.5A.2-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(G-H) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  | Yes |  |  |
|  |  | n261 |  | See CA_n261(G-H) in Table 5.5A.2-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(G-I) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  | Yes |  |  |
|  |  | n261 |  | See CA_n261(G-I) in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(H-I) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  | Yes |  |  |
|  |  | n261 |  | See CA_n261(H-I) in Table 5.5A.2-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. (From Table 5.3.5-1 of 38.101-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 8.17.1.3		Co-existence studies

Table 8.17.1.3-1 gives the UL 2nd, 3rd, 4th, 5th, 6th, 7th harmonic for CA_n77A-n261A.

Table 8.x.1.3-1: Band n77 and Band n261 UL harmonics products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3300 | 4200 | 27500 | 28350 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6600 | 8400 | 55000 | 56700 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 9900 | 12600 | 82500 | 85050 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 13200 | 16800 | 110000 | 113400 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 16500 | 21000 | 137500 | 141750 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6*fy_low | 6*fy_high |
| 6th harmonics frequency limits(MHz) | 19800 | 25200 | 165000 | 170100 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7*fy_low | 7*fy_high |
| 7th harmonics frequency limits(MHz) | 23100 | 29400 | 192500 | 198450 |

In analysis, it could be seen,

- The 7th harmonic produced by UL band n77 may fall into band n261DL.

#### 8.17.1.4		∆TIB and ∆RIB values

For CA_n77A-n261A, the TIB,c and RIB values are given in the tables below.

Table 8.17.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n77-n261 | n77 | 0 |
|  | n261 | 0 |

Table 8.17.4-2: ΔRIB

| Inter-band CA Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n77-n261 | n77 | 0 |
|  | n261 | 0 |

#### 8.17.1.5		REFSENS requirements

There are no specific REFSENS requirements for this configuration.

### 8.17.2		Specific for 2 bands UL CA

#### 8.17.2.1		UE co-existence studies

Table 8.17.2.1-1 lists Band n77 +Band n261 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 8.17.2.1-1: Band n77 and Band n261 2UL bands IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3300 | 4200 | 27500 | 28350 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 23300 | 25050 | 30800 | 32550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 21750 | 19100 | 50800 | 53400 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 34100 | 36750 | 58300 | 60900 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 18450 | 14900 | 78300 | 81750 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 37400 | 40950 | 85800 | 89250 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 50100 | 46600 | 61600 | 65100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 110100 | 105800 | 10700 | 15150 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 113300 | 117600 | 40700 | 45150 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 78450 | 74100 | 42400 | 46800 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 89100 | 93450 | 64900 | 69300 |

In analysis, it could be seen, there are no harmonic or IMD issues affecting own Rx frequencies of either band n77 or n261.

Table 8.17.2.1-2 lists the protected bands required for the 2UL bands CA configuration.

Table 8.17.2.1-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n77A-n261A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | PHS |

#### 8.17.2.2		REFSENS requirements

There are no need for additional REFSENS requirements for this configuration.

<Start of Text Proposal>

……..

## 9	2 bands Dual Connectivity: Specific Band Combination Part

<Start of Text Proposal>

## 9.x	DC_n77-n261

### 9.x.1	Operating bands for DC_n77-n261

Table 9.x.1-1: Inter-band NR DC operating bands

| NR DC Band | NR Band |
| --- | --- |
| DC_n77-n261 | n77, n261 |

### 9.x.2   Configurations for DC_n77-n261

Table 9.x.2-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n77A-n261A | DC_n77A-n261A |
| DC_n77A-n261D | DC_n77A-n261A DC_n77A-n261D |
| DC_n77A-n261G | DC_n77A-n261A DC_n77A-n261G |
| DC_n77A-n261H | DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H |
| DC_n77A-n261I | DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n77A-n261J | DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I DC_n77A-n261J |
| DC_n77A-n261K | DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I DC_n77A-n261J DC_n77A-n261K |
| DC_n77A-n261L | DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I DC_n77A-n261J DC_n77A-n261K DC_n77A-n261L |
| DC_n77A-n261M | DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I DC_n77A-n261J DC_n77A-n261K DC_n77A-n261L DC_n77A-n261M |
| DC_n77A-n261(2A) | DC_n77A-n261A |
| DC_n77A-n261(2G) | DC_n77A-n261A |
| DC_n77A-n261(2H) | DC_n77A-n261A |
| DC_n77A-n261(2I) | DC_n77A-n261A |
| DC_n77A-n261(3A) | DC_n77A-n261A |
| DC_n77A-n261(4A) | DC_n77A-n261A |
| DC_n77A-n261(A-G) | DC_n77A-n261A |
| DC_n77A-n261(A-H) | DC_n77A-n261A |
| DC_n77A-n261(A-I) | DC_n77A-n261A |
| DC_n77A-n261(G-H) | DC_n77A-n261A |
| DC_n77A-n261(G-I) | DC_n77A-n261A |
| DC_n77A-n261(H-I) | DC_n77A-n261A |

<Start of Text Proposal>