3GPP TSG-RAN WG4 Meeting #95-e	R4-2006790

Online Meeting, 25 May – 5 Jun. 2020

Title:	On obsolescence of NS_201

Source:	Qualcomm Incorporated

Agenda item:	4.4.3.1

Release:	Rel-15

Work Item:	NR_newRAT-Core

Responsible WG:	RAN4

Document for:	Approval

# 1.	Introduction

NS_201 was instituted to comply with the EU’s original emissions limit of -8 dBm/200 MHz in the lower EESS band (23.6-24.0 GHz). Since then the EU has introduced new requirements (in ERC 74-01) and has revisited the lower EESS (relaxation to +1 dBm/ 200 MHz), rendering NS_201 obsolete.

# 2. 	Discussion

NS_201 was instituted to signal to the UE, an emissions limit of -8 dBm/200 MHz in the lower EESS band (23.6-24.0 GHz), as was originally in force in the EU. Due to subsequent changes (*) in EU regulations, the emissions requirements indicated by NS_201 is not applicable in the EU anymore, and is not known to be applicable in any other geographical region.

The updated EU requirements are addressed by NS_202 (and CA_NS_202), updated in a companion contribution [1].

There are significant advantages to the ecosystem, primarily in testing and validation of UEs, of not persisting with a disused NS. Since NS_201 and CA_NS_201 are no longer relevant for future deployments, we propose

Proposal: NS_201 and CA_NS_201 shall be declared obsolete in release independent manner.

(*) ERC74-01 instituted a -10 dBm/100 MHz requirement outside the SEM, A further change to the protection level of the lower EESS band from -8 dBm/200 MHz to +1 dBm/200 MHz  was initiated by the European Commission, Radio Spectrum Committee (RSCOM20-05rev3).

# 3.	References

[1] R4-2006785, ‘CR to 38.101-2: NS_202 update after changes to EU regulations’, Qualcomm, RAN4#95-e, May-Jun 2020