3GPP TSG-RAN WG4 Meeting # 95-e                                        R4-2006859

# Electronic Meeting, 25 May – 5 June, 2020

Agenda Item:	6.1.5.11

Source:	MediaTek Inc.

Title:	Discussion on SFTD measurements towards NR-U with LBT

Document for:		Discussion

- Introduction

In last meeting, the requirements for inter-RAT SFTD measurements towards NR-U with LBT have been discussed as WF [1]. In this paper, our view on the maximum scaling, k, for inter-RAT SFTD measurements for NR-U is provided.

- Discussion

Issue 1-1: Maximum scaling of inter-RAT SFTD measurements

- Final candidate options:

- Option 1: k=4.

- Option 2: k =3.

For the inter-RAT SFTD requirement, the corresponding WF [1] is attached below for reference.

Besides, maximum period for EN-DC SFTD towards NR-U with LBT would be a good reference, and it was agreed in RAN4#93, as the following:

- NLBT-fail,max

| Condition | NLBT-fail,max |
| --- | --- |
| max(TDRX,TSMTC)≤40 | [7] |
| 40<max(TDRX,TSMTC)≤320 | [5] |
| TDRX > 320 | [3] |

It can be observed that the corresponding maximum scaling, k, for EN-DC SFTD as Table 1.

Table 1. Maximum scaling for EN-DC SFTD

| Procedure | # of samples in Rel-15 | NLBT-fail,max | Condition | Max. Scaling (K) |
| --- | --- | --- | --- | --- |
| ENDC SFTD | 5 | 7 | max(TDRX,TSMTC)≤40 | 2.4x |
|  | 8 | 5 | 40<max(TDRX,TSMTC)≤320 | 1.63x |
|  | 5 | 3 | TDRX > 320 | 1.6x |

Observation 1: For EN-DC SFTD measurement in NR-U, the maximum scaling is ranged from 1.6 to 2.4.

For inter-RAT SFTD, although network may not have rudimentary timing information, the non-scaled R15 measurement period of inter-RAT SFTD has been specified as longer than other measurements. Thus, it cannot justify the larger scaling factor of inter-RAT SFTD should be required. As compromised, since the scaling fator of EN-DC SFTD is k = 1.6 - 2.4, we can agree with k=3, which is already bit larger than the k in EN-DC SFTD, while the non-scaled measurement time of inter-RAT SFTD has been longer than the non-scaled measurement time of ENDC SFTD.

Proposal 1: k=3, as the maximum scaling of inter-RAT SFTD measurements towards NR-U (Option 2).

- Summary

In this paper, our view of the maximum scaling for inter-RAT SFTD measurements towards NR-U with LBT is provided. We have the following observations and proposals:

Observation 1: For EN-DC SFTD measurement in NR-U, the maximum scaling is ranged from 1.6 to 2.4.

Proposal 1: k=3, as the maximum scaling of inter-RAT SFTD measurements towards NR-U (Option 2).

References

[1] R4-2005375 WF on NR-U RRM Requirements (Part 3), Nokia.