3GPP TSG-RAN WG4 Meeting #95-e                 	R4-2006885

Electronic Meeting, 25th May – 5th June, 2020

Agenda item:		6.6.3.2.1

Source:	MediaTek Inc.

Title:	Discussion on LTE CRS based and NR SSB based measurement in NR IDLE/INACTIVE mode

Document for:		Discussion

# 1	Introduction

In RAN4 #94Bis meeting, it is agreed that companies are encouraged to provide input and views on following early measurement reporting (EMR) issues [1]:

Agreements - Actively measured condition: - At least following conditions apply for an actively measured EMR carrier - T331 timer is running. - Serving cell is in the validity area. - Serving cell support EMR. - Definition of overlapping and non-overlapping - An overlapping carrier is a carrier which the UE is actively measuring for EMR and mobility - A non-overlapping carrier is a carrier which the UE is actively measuring for EMR Note: overlapping and overlapping carrier relates to a carrier configured for EMR. - Define measurement requirements for NR inter-frequency and LTE inter-RAT EMR carriers - All companies support the following principle with some additional modifications: - NR inter-frequency EMR measurement requirements on an overlapping NR EMR carrier follow existing NR inter-frequency measurements. - LTE inter-RAT EMR measurement requirements on an overlapping LTE EMR carrier follow existing LTE inter-RAT measurements. - Defining the number of EMR carriers: - if N>14: Option 1: no RAN4 requirements defined (UE implementation) - Total number of carriers that the UE needs to measure for cell reselection and EMR (overlapping or not overlapping), including NR and LTE carriers, cannot exceed Rel-15 UE capability (as defined in 38.133, section 4.2.2.1) - Conditions when UE is required to perform measurements for EMR - RAN4 at least defines UE EMR requirements for EMR carriers while T331 timer is active - RAN4 not to define different or additional UE behaviour than what RAN2 has defined related to EMR measurement at cell change - Define cell detected status based on LTE baseline and on: - The detected cell is on the overlapping or non-overlapping carrier (as indicated by dedicated RRC signalling or SIB), and - A T331 is provided during connection release, and - The carrier frequency of the detected cell and the serving cell are among the supported band combination of the UE Way Forward - Companies are invited to provide input on measurement requirements for NR inter-frequency and LTE inter-RAT EMR carrier - Definition on UE measurement capability to support beam-level measurements for EMR. - Detailed beam level measurement capability requirements: - option 1: same as the number of beams in the existing inter-frequency requirements for RRC_CONNECTED state - option 2: other? - NR inter-frequency EMR measurement requirements on a non-overlapping carrier - Consider FR1 and FR2 aspects - LTE inter-RAT EMR measurement requirements on a non-overlapping carrier - Measurement requirements for non-overlapping carriers. - Companies are invited to provide input on the number of EMR carriers: - Total number of overlapping EMR carriers - Total number of non-overlapping EMR carriers - Total number of NR inter-frequency carriers - Total number of LTE inter-RAT carriers - Companies are invited to provide input on when UE is required to perform measurements for EMR - Will RAN4 define UE EMR requirements for EMR carriers when T331 timer has expired? - Does RAN4 define transition requirements for EMR measurement at cell change? - Companies are invited to provide input on Cell detected status - The detected cell remains detectable in terms of signal quality - The detected SSBs of a cell remains detectable in terms of signal quality - T331 status – specifically and “timer is not expired” - Companies are invited to provide input on Cell edge conditions and EMR

In this paper, we will discuss all the related issues for NR IDLE/INACTIVE CA and DC measurement.

# 2	Discussion on scenarios of the early measurement reporting

We would like to firstly discuss the scenarios for the early measurement reporting shown in Table 1. The first one is LTE CRS based measurement in LTE IDLE/INACTIVE mode and RAN4 had already discussed the corresponding requirements in the LTE euCA section. The remaining 3 scenarios are:

1. NR SSB based measurement in LTE IDLE/INACTIVE mode for DC purpose;

2. LTE CRS based measurement in NR IDLE/INACTIVE mode for DC purpose;

3. NR SSB based measurement in NR IDLE/INACTIVE mode for both DC and CA purposes.

Table 1: Measurements of inter-frequency CA candidate cells

| scenarios | EMR requirement | Purpose | Spec | Requirement Framework | Capability |
| --- | --- | --- | --- | --- | --- |
| 1 | LTE CRS based measurement in LTE IDLE/INACTIVE | CA | 36.133 | Done | ca-IdleModeMeasurements-r15 in LTE RRC |
| 2 | NR SSB based measurement in LTE IDLE/INACTIVE | DC (EN-DC) | 36.133 | TBD | endc-IdleModeMeasurements-r16 in LTE RRC |
| 3 | LTE CRS based measurement in NR IDLE/INACTIVE | DC (NE-DC) | 38.133 | TBD | idleModeMeasurementsEUTRA in NR RRC |
| 4 | NR SSB based measurement in NR IDLE/INACTIVE | DC ,CA (NR-DC, NR-SA) | 38.133 | TBD | idleModeMeasurementsNR in NR RRC |

In this paper, we will focus on the scenario 3 and 4.

# 3	Discussion on requirement framework of NR IDLE/INACTIVE DC and CA measurement

As listed in the WF [1], so far RAN4 has listed many related issues for the NR IDLE/INACTIVE mode DC and CA measurement. We will provide our own view on each of them.

- Beam-level UE measurement capability for EMR:

In RAN2 #109-e meeting [2], it was agreed that

For early measurement configuration and reporting, ASN.1 signalling to allow the configuration of

-up to 8 E-UTRA and 8 NR carries to be measured

-up to 8 E-UTRA and 8 NR carriers to be reported

-up to 32 beams to be included in the NR results

LS to be sent to RAN4 (Ericsson), with the purpose of:

- informing RAN2 decision (proposal 1)

asking for input:

- clarification on the UE requirement aspects of early measurement performance and reporting

However, the number in ASN.1 signaling upper bound is not UE requirement. The required number of carriers or beams to be reported is up to RAN4. In Rel-15 RAN4 never had a discussion on the beam level requirement in IDLE/INACTIVE mode and it is also strange to specify beam level requirement in IDLE/INACTIVE mode only for EMR. Moreover, in Rel-15 UE is never mandated to acquire SSB index during IDLE mode measurement. Therefore, we prefer not to introduce beam level requirement for EMR in Rel-16.

Proposal 1: RAN4 to confirm there is no beam-level UE measurement capability specified for EMR purpose

- Measurement requirement of non-overlapping EMR carriers :

In euCA, the requirement of EMR overlapping carriers is the same with that of re-selection carriers. Besides, RAN4 didn’t specified measurement period for EMR non-overlapping carriers. We suggest to reuse this principle in NR IDLE/INACTIVE mode CA and DC measurement.

Proposal 2: RAN4 not to specify the measurement period of non-overlapping EMR carriers

- Number of carriers to measure for EMR:

Based on the agreement “Total number of carriers that the UE needs to measure for cell reselection and EMR (overlapping or not overlapping), including NR and LTE carriers, cannot exceed Rel-15 UE capability (as defined in 38.133, section 4.2.2.1),” our understanding is that no matter UE is supporting re-selection only, or supporting both re-selection and any EMR measurement, the measurement capability will not be changed as shown in the following Table 2:

Table 2: Measurement capability for UE in IDLE/INACTIVE mode

| case | Scenarios that UE support | Intra-frequency carriers | NR inter-frequency carriers X | FDD E-UTRA inter-RAT carriers Y | TDD E-UTRA inter-RAT carriers Z | total carriers N |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Only reselection | 1 | X≤7 | Y≤7 | Z≤7 | N≤14 |
| 2 | Reselection +scenario 3 | 1 | X≤7 | Y≤7 | Z≤7 | N≤14 |
| 3 | Reselection +scenario 4 | 1 | X≤7 | Y≤7 | Z≤7 | N≤14 |
| 4 | Reselection +scenario 3 +scenario 4 | 1 | X≤7 | Y≤7 | Z≤7 | N≤14 |

The first thing we would like to clarify is: for each case, whether RAN4 has common understanding that no RAN4 requirement is defined (up to UE implementation) when any of limitations listed in Table 2 is exceeded, i.e., X>7, or Y>7, or Z>7, or N>14.

Proposal 3: RAN4 to clarify no RAN4 requirement is defined (up to UE implementation) when any of following limitations, measurement capability for re-selection purpose, is exceeded: 
    • 7 NR inter-frequency carriers 
    • 7 FDD E-UTRA inter-RAT carriers 
    • 7 TDD E-UTRA inter-RAT carriers
    • total 14 carrier frequency layers

The next question is what the corresponding UE behaviour is when RAN4 introduces more limitations on number of EMR carriers to be measured or reported? If we take case 4 for example, the detail limitations can be divided into overlapping and non-overlapping types, as shown in below Table 3. In last meeting, companies discussed the limitation on number of overlapping EMR carriers, number of non-overlapping EMR carriers, number of NR inter-frequency EMR carriers, and number of LTE inter-RAT EMR carriers. We would like to clarify whether RAN4 has common understanding that no RAN4 requirement is defined (up to UE implementation) when any of limitations listed in below table is exceeded, i.e., kO >[TBD], or kN >[TBD], or xO + xN >[TBD], or yO + yN+ zO + zN >[TBD], or X>7, or Y>7, or Z>7, or N>14.

Proposal 4: RAN4 to clarify no RAN4 requirement is defined (up to UE implementation) when any of limitations shown in following table, measurement capability for re-selection purpose and EMR purpose, is exceeded 
                  Table 3: Measurement capability for UE in IDLE/INACTIVE mode

| Case 4 | Intra-frequency carriers 1 | NR inter-frequency carriers X≤7, (X= xR + xO + xN) | FDD E-UTRA inter-RAT carriers Y≤7, (Y= yR + yO + yN) | TDD E-UTRA inter-RAT carriers Z≤7, (Z= zR + zO + zN) | Total carriers N≤14, (K= kO+ kN) |
| --- | --- | --- | --- | --- | --- |
| Re-selection |  | xR | yR | zR |  |
| Overlapping EMR |  | xO | yO | zO | kO≤[TBD] (kO= xO + yO+ zO) |
| Non-overlapping EMR |  | xN | yN | zN | kN≤[TBD] (kN= xN + yN+ zN) |
|  |  | xO + xN ≤[TBD] | yO + yN+ zO + zN ≤[TBD] |  |  |

- Will RAN4 define UE EMR requirements for EMR carriers when T331 timer has expired

Considering that RAN2 already agreed “it is up to UE implementation whether to continue monitoring the EMR after the expiry of T331 timer,” there is no need to specify the corresponding measurement delay for this case. However, we suggest to specify the measurement accuracy in case that UE would like to report the EMR measurement after T331 timer expires.

Proposal 5: RAN4 to specify the measurement accuracy of EMR carriers for the UE would like to report EMR after T331 timer expires

- Does RAN4 define transition requirements for EMR measurement at cell change

We don’t think RAN4 should specify any requirement for the transition period. The transition period might have various scenarios, and it is impossible for RAN4 to clarify the UE behaviour in all scenarios. So we suggest

Proposal 6: RAN4 not to specify any transition period requirement for EMR measurement at cell change

- Not in cell center conditions and EMR

Considering that the EMR measurement results might not be useful once UE changes the PCell, so we prefer not to define EMR measurement requirements when UE is in the cell edge. In last meeting, companies said that it is RAN2’ agreement “The fields s-NonIntraSearchP and s-NonIntraSearchQ in SIB2 do not affect the idle/inactive UE measurement procedures.” However, our understanding is that “RAN2 agreement is to let RAN4 decide whether s-NonIntraSearchP and s-NonIntraSearchQ affect the idle/inactive UE measurement behaviour is UE implementation”

The fields s-NonIntraSearchP and s-NonIntraSearchQ in SIB2 do not affect the idle/inactive UE measurement procedures. How the UE performs idle/inactive measurements is up to UE implementation as long as the requirements in TS 38.133 [14] are met for measurement reporting

So we suggest

Proposal 7: RAN4 to reuse the “s-NonIntraSearchP and s-NonIntraSearchQ” to define the not in cell center condition in EMR

# 4	Summary

In this contribution, we have the following proposals:

Proposal 1: RAN4 to confirm there is no beam-level UE measurement capability specified for EMR purpose

Proposal 2: RAN4 not to specify the measurement period of non-overlapping EMR carriers

Proposal 3: RAN4 to clarify no RAN4 requirement is defined (up to UE implementation) when any of following limitations, measurement capability for re-selection purpose, is exceeded: 
    • 7 NR inter-frequency carriers 
    • 7 FDD E-UTRA inter-RAT carriers 
    • 7 TDD E-UTRA inter-RAT carriers
    • total 14 carrier frequency layers

Proposal 4: RAN4 to clarify no RAN4 requirement is defined (up to UE implementation) when any of limitations shown in following table, measurement capability for re-selection purpose and EMR purpose, is exceeded 
                  Table 3: Measurement capability for UE in IDLE/INACTIVE mode

| Case 4 | Intra-frequency carriers 1 | NR inter-frequency carriers X≤7, (X= xR + xO + xN) | FDD E-UTRA inter-RAT carriers Y≤7, (Y= yR + yO + yN) | TDD E-UTRA inter-RAT carriers Z≤7, (Z= zR + zO + zN) | Total carriers N≤14, (K= kO+ kN) |
| --- | --- | --- | --- | --- | --- |
| Re-selection |  | xR | yR | zR |  |
| Overlapping EMR |  | xO | yO | zO | kO≤[TBD] (kO= xO + yO+ zO) |
| Non-overlapping EMR |  | xN | yN | zN | kN≤[TBD] (kN= xN + yN+ zN) |
|  |  | xO + xN ≤[TBD] | yO + yN+ zO + zN ≤[TBD] |  |  |

Proposal 5: RAN4 to specify the measurement accuracy of EMR carriers for the UE would like to report EMR after T331 timer expires

Proposal 6: RAN4 not to specify any transition period requirement for EMR measurement at cell change

Proposal 7: RAN4 to reuse the “s-NonIntraSearchP and s-NonIntraSearchQ” to define the not in cell center condition in EMR

# Reference

- R4-2005847, WF on MR-DC RRM requirements, Nokia, Apr., 2020.

- R2-2002501, Report of 3GPP TSG RAN2#109-e meeting, Feb., 2020.