3GPP TSG-RAN4 Meeting #95-e	R4-2006945

# Online, 25th May – 5th June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0328 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38101-1 on switching between 1Tx carrier and 2Tx carrier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1 |  |  |  |  |  | Date: |  |  | 2020-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to objectives of NR_RF_FR1, UE requirements for switching between NR SUL/UL carrier 1 with 1Tx chain and NR UL carrier 2 with 2Tx chains are introduced to TS 38.101-1 for UEs configured with SUL or UL-CA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the switching time masks for the UE switching operation between NR SUL/UL carrier 1 with 1Tx chain and NR UL carrier 2 with 2Tx chains. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification/UE requirements for switching between NR SUL/UL carrier 1 with 1Tx chain and NR UL carrier 2 with 2Tx chains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.3 and new section 6.3C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Start of Change1>>

### 6.3A.3	Transmit ON/OFF time mask for CA

#### 6.3A.3.1	Void

#### 6.3A.3.2	Void

#### 6.3A.3.3	Transmit ON/OFF time mask for inter-band CA

##### 6.3A.3.3.1	General

For inter-band carrier aggregation with uplink assigned to two NR bands, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in clause 6.3.3.1 shall only be applicable for each component carrier when all the component carriers are OFF.

#### 6.3A.3.3.2	Time mask for UE switching between two uplink carriers

The switching time mask defined in this clause is applicable when UE indicating support of UplinkTxSwitchingPeriod between an uplink band pair of an inter-band UL CA configuration, and is only applicable when uplink transmission is switched between NR UL carrier 1 on which the UE is capable of using one transmit antenna connector and NR UL carrier 2 on which the UE is capable of using two transmit antenna connectors, where the two uplink carriers are in different bands with different carrier frequencies in FR1.

The switching periods described in Figure 6.3A.3.3.2-1a and Figure 6.3A.3.3.2-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling [7], and the length of the uplink switching period depends on UE capability UplinkTxSwitchingPeriod.

The requirements apply for the case of co-located and synchronized network deployment for the two uplink carriers.

The requirements apply for the case of single TAG for the two uplink carriers, i.e., the same uplink timing for the two carriers as described in sub-clause 4.2 of TS 38.213 [8].

For UE supporting uplink Tx switching, it is scheduled both 2-layer and single-layer PUSCH transmissions on carrier 2.

Figure 6.3A.3.3.2-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching periods are located in carrier 1

Figure 6.3A.3.3.2-1b: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching periods are located in carrier 2

## <<End of Change1>>

## <<Start of Change2>>

## 6.3C	Output power dynamics for SUL

For SUL band combinations defined in sub-clause 5.2C and 5.5C, the output power dynamics requirements in clause 6.3C apply.

### 6.3C.1	Void

### 6.3C.2	Void

### 6.3C.3	Transmit ON/OFF time mask for SUL

#### 6.3C.3.1	Time mask for UE switching between two uplink carriers

The switching time mask defined in this clause is only applicable when uplink transmission is switched between NR SUL carrier 1 on which the UE is capable of using one transmit antenna connector and NR UL carrier 2 on which the UE is capable of using two transmit antenna connectors, where the two uplink carriers are in different bands with different carrier frequencies in FR1.

The switching periods described in Figure 6.3C.3.1-1a and Figure 6.3C.3.1-1b are located in either NR SUL carrier 1 or UL carrier 2 as indicated in RRC signalling [7], and the length of the uplink switching period depends on UE capability UplinkTxSwitchingPeriod.

For UE supporting uplink Tx switching, it is scheduled both 2-layer and single-layer PUSCH transmissions on carrier 2.

Figure 6.3C.3.1-1a: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching periods are located in carrier 1

Figure 6.3C.3.1-1b: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching periods are located in carrier 2

## <<End of Change2>>