3GPP TSG-RAN4 Meeting #95-e	R4-2006972

# Online, , 25th May 2020 - 5th Jun 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6845 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR CGI measurements with autonomous gaps for 36.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-Core |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Requirements are added for the delay and also the interruption caused to LTE when a UE reads the CGI of an NR target cell with autonomous gaps |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section for requirements is added. Delay and interruption needs to be updated based on agreements for NR CGI reading in 38.133 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR CGI reading is not fully specified for interRAT and EN-DC operations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.4.27, 8.1.2.4.28, 8.17.16, 8.17.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.1.2.4.27	E-UTRA FDD - NR CGI measurements with autonomous gaps

#### 8.1.2.4.27.1	Introduction

The requirements in this clause are specified for CGI identification of an NR target cell and are applicable for a UE:

-	in RRC_CONNECTED state, and

-	configured with LTE-FDD standalone,  or EN-DC operation mode using LTE-FDD

The overall CGI reporting delay includes RRC procedure delay defined in clause 12 in TS 38.331 [2] and CGI reporting delay in clause 8.1.2.4.27.2.

#### 8.1.2.4.27.2	CGI identification of an NR cell with autonomous gaps

The UE shall identify and report the CGI of an NR target cell when requested by the network for the purpose of ‘reportCGI’. Only one cell is provided to the UE with cellForWhichToReportCGI for identifying the CGI.The UE may make autonomous gaps in both downlink reception and uplink transmission of both E-UTRA and NR serving cell(s) for receiving MIB and SIB1 message according to clause 5.5.3 of TS 38.331 [2]. Note that EUTRAN does not configure si-RequestForHO if reportConfig is linked to a measObject set to measObjectNR. The UE shall be able to identify a new CGI of a known NR cell within:

Tidentify_CGI_NR = ( TMIB_NR + T SIB1_NR) ms

Where:

TMIB_NR is the maximum time to acquire MIB. TMIB_NR = TBD ms.

TSIB1_inter is the maximuim time period o acquire SIB1 message. TSIB1_NR = TBD  ms.

An NR cell shall be considered detectable when following conditions for relevant SSB are fulfilled:

Interfrequency SS-RSRP related side conditions given in clauses 10.1.4 and 10.1.5 in [50]for FR1 and FR2, respectively, for a corresponding Band,

-	Interfrequency SSB_RP and SSB Ês/Iot according to Annex B.2.3 in [50] for a corresponding Band.

The requirement for identifying the CGI of an NR cell within Tidentify_CGI_NR is applicable when no DRX is used as well as when any of the DRX or specified in TS36,331 [2] or TS 38.331 [38] is used.

8.1.2.4.27.3	Interruptions

Within the time, Tidentify_CGI_NR ms, over which the UE identifies the new CGI of an NR cell, the UE may have up to TBD uplink and downlink interruptions of duration up to TBD duration for MIB decoding and additionally up to TBD uplink and downlink interruptions of up to TBD durationfor SIB decoding

#### 8.1.2.4.27.4	CGI reporting delay

The CGI reporting delay is defined as the time between a command that will trigger a CGI report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty of 2 x TTIDCCH resulting when inserting the measurement report to the TTI of the uplink DCCH. This measurement reporting delay excludes any delay caused by lack of UL resources for UE to send the measurement report.

The CGI reporting delay shall be less than Tidentify_CGI_NR defined in clause 8.1.2.4.27.2.

### 8.1.2.4.28	E-UTRA TDD - NR CGI measurements with autonomous gaps

#### 8.1.2.4.28.1	Introduction

The requirements in this clause are specified for CGI identification of an NR target cell and are applicable for a UE:

-	in RRC_CONNECTED state, and

-	configured with LTE-FDD standalone,  or EN-DC operation mode using LTE-FDD

The overall CGI reporting delay includes RRC procedure delay def ined in clause 12 in TS 38.331 [2] and CGI reporting delay in clause 8.1.2.4.28.2.

#### 8.1.2.4.28.2	CGI identification of an NR cell with autonomous gaps

The UE shall identify and report the CGI of an NR target cell when requested by the network for the purpose of ‘reportCGI’. Only one cell is provided to the UE with cellForWhichToReportCGI for identifying the CGI.The UE may make autonomous gaps in both downlink reception and uplink transmission of both E-UTRA and NR serving cell(s) for receiving MIB and SIB1 message according to clause 5.5.3 of TS 38.331 [2]. Note that EUTRAN does not configure si-RequestForHO if reportConfig is linked to a measObject set to measObjectNR. The UE shall be able to identify a new CGI of a known NR cell within:

Tidentify_CGI_NR = ( TMIB_NR + T SIB1_NR) ms

Where:

TMIB_NR is the maximum time to acquire MIB. TMIB_NR = TBD ms.

TSIB1_inter is the maximuim time period o acquire SIB1 message. TSIB1_NR = TBD  ms.

An NR cell shall be considered detectable when following conditions for relevant SSB are fulfilled:

Interfrequency SS-RSRP related side conditions given in clauses 10.1.4 and 10.1.5 in [50]for FR1 and FR2, respectively, for a corresponding Band,

-	Interfrequency SSB_RP and SSB Ês/Iot according to Annex B.2.3 in [50] for a corresponding Band.

The requirement for identifying the CGI of an NR cell within Tidentify_CGI_NR is applicable when no DRX is used as well as when any of the DRX or specified in TS36,331 [2] or TS 38.331 [38] is used.

8.1.2.4.28.3	Interruptions

Within the time, Tidentify_CGI_NR ms, over which the UE identifies the new CGI of an NR cell, the UE may have up to TBD uplink and downlink interruptions of duration up to TBD duration for MIB decoding and additionally up to TBD uplink and downlink interruptions of up to TBD durationfor SIB decoding

#### 8.1.2.4.28.4	CGI reporting delay

The CGI reporting delay is defined as the time between a command that will trigger a CGI report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty of 2 x TTIDCCH resulting when inserting the measurement report to the TTI of the uplink DCCH. This measurement reporting delay excludes any delay caused by lack of UL resources for UE to send the measurement report.

The CGI reporting delay shall be less than Tidentify_CGI_NR defined in clause 8.1.2.4.28.2.

Change 2

### 8.17.16	E-UTRA FDD - NR CGI measurements with autonomous gaps

The requirements in clause 8.1.4.27 also apply in EN-DC operation.

### 8.17.17	E-UTRA TDD - NR CGI measurements with autonomous gaps

The requirements in clause 8.1.4.28 also apply in EN-DC operation.