3GPP TGS-RAN WG4 Meeting #95e	R4-2007110
Electronic Meeting, May 25 – June 5, 2020

Agenda item:	10.1.1

Source:	Intel Corporation

Title:	UE RF requirements for new FR2 FWA use case

Document for:	Approval

# 1	Introduction

To ensure countries with stricter regulations have requirements for a FWA use case, a new Rel-17 work item was approved in the last plenary meeting [1]. RAN4 UE RF discussions will focus on two main objectives: defining the RF requirements of the new FWA use case for bands n257 and n258 and how to introduce these requirements into the specifications (either through a new UE capability or a new UE power class).

Discussions started in the last RAN4 meeting and resulted in the following updates to the RF requirement scope [2-3]:

Also, to facilitate the analysis of the minimum peak EIRP and EIS requirements, two tables with budget parameters to be considered were included in the way forward [3].

Lastly, two options were captured in a separate WF on how to introduce the requirements of the new FWA use case [4].

In this paper we provide derivations for the minimum peak EIRP and EIS requirements of the new FWA use case.

# 2	Discussion

Two main items are addressed in the core part of the WI objectives: defining the requirements and how to introduce the requirements into the specifications. We will focus this paper on deriving the minimum peak EIRP and EIS values.

## 2.1	RF requirements

A higher minimum peak EIRP can be achieved by increasing the number of antenna elements, thus we need more than the 4-elements used for PC3. We will use 8 elements in our derivations, which is the same number of elements used in PC2 derivations [5].

Minimum peak EIRP

Table 1 lists all parameters needed to derive the minimum peak EIRP. Compared to PC3, and as mentioned in the preceding section, the derived values yield a higher minimum peak EIRP mainly because of the larger number of antenna elements. The final result is a minimum peak EIRP of 28.3 dBm, which corresponds to a significant 6 dB increase in minimum peak EIRP compared to PC3 (22.4 dBm). The value is also close to the 29 dBm PC2 requirement [6].

Table 1. Minimum peak EIRP evaluation of new FWA use case

| Parameter | Unit | Operating bands n257/n258 | Comments |
| --- | --- | --- | --- |
| Pout per element | dBm | 12.5 |  |
| # of antennas in array |  | 8 | Increases peak EIRP value compared to the 4 antenna elements used in PC3 |
| Total conducted power per polarization | dBm | 21.5 |  |
| Avg. antenna element gain | dBi | 4.0 |  |
| Antenna roll-off loss vs freq. | dB | -2.0 |  |
| Realized antenna array gain | dBi | 11.0 | 10*log(# ant.) + avg. element gain – roll-off |
| Polarization gain | dB | 2.80 |  |
| Mismatch and transmission line loss including load pull | dB | -2.50 |  |
| Beam forming loss (phase shifter and amplitude error) | dB | -0.50 |  |
| Finite beam table | dB | -0.25 |  |
| Beam forming loss (one beam table fits all) | dB | -0.25 |  |
| Form-factor integration losses | dB | -3.50 |  |
| Total implementation loss | dB | -7.00 |  |
| Peak EIRP (Minimum) | dBm | 28.30 | Almost 6 dB increase from PC3 value |

Observation 1: The derived minimum peak EIRP value for the new FWA use case is 28.3 dBm. This represents ~ 6 dB increase from the PC3 requirement value.

Proposal 1: Define the n257 and n258 minimum peak EIRP requirement of the new FWA use case with 23dBm max TRP as 28.3 dBm.

Minimum peak EIS

Similar to our previous section, Table 2 captures the parameters needed to derive the minimum peak EIS. The derived value yields a more sensitive minimum peak EIS compared to PC3 mostly due to the larger number of antenna elements and overall higher gain. The final result is a minimum peak EIS of -92 dBm, which represents a difference of almost 4dB compared to PC3 (-88.3 dBm).

Table 2. Minimum peak EIS evaluation of new FWA use case

| Parameter | Unit | Value |
| --- | --- | --- |
| Operating bands |  | n257/n258 |
| Modulation |  | QPSK |
| Bandwidth | MHz | 50 |
| SNR | dB | -1 |
| Thermal noise - 10log[(k*T*BW)/1mW] | dBm | -97 |
| Noise figure | dB | 10 |
| Effective array gain - 10log(# ant.) + element gain – roll-off | dB | 11 |
| Total implementation loss | dB | 7 |
| Peak EIS (Minimum) | dBm | -92 |

Observation 2: The derived minimum peak EIS value for bands n257 and n258 of the new FWA use case is -92 dBm. This corresponds to a difference of almost 4 dB compared to the PC3 requirement of -88.3 dBm.

Proposal 2: Define the n257 and n258 minimum peak EIS requirement of the new FWA use case with 23dBm max TRP as -92 dBm (for 50 MHz bandwidth).

# 3	Conclusions

In this paper we derived values for the minimum peak EIRP and EIS requirements of the new FWA use case. The following observations and proposals have been made:

Observation 1: The derived minimum peak EIRP value for the new FWA use case is 28.3 dBm. This represents ~ 6 dB increase from the PC3 requirement value.

Proposal 1: Define the n257 and n258 minimum peak EIRP requirement of the new FWA use case with 23dBm max TRP as 28.3 dBm.

Observation 2: The derived minimum peak EIS value for bands n257 and n258 of the new FWA use case is -92 dBm. This corresponds to a difference of almost 4 dB compared to the PC3 requirement of -88.3 dBm.

Proposal 2: Define the n257 and n258 minimum peak EIS requirement of the new FWA use case with 23dBm max TRP as -92 dBm (for 50 MHz bandwidth).

# 4	References

- RP-200503, “New WID on Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258,” Softbank, RAN # 87e, March 2020

- R4-2005713, “Email discussion summary for [94e Bis][32]NR_FR2_FWA_Bn257_Bn258, SoftBank, RAN4 #94e-Bis, April 2020

- R4-2005177, “WF on FR2 new FWA UE RF and RRM requirements, Huawei, HiSilicon, SoftBank, RAN4 #94e-Bis, April 2020

- R4-2005176, “WF on the option for FR2 FWA UE, NTT DOCOMO,” RAN4 #94e-Bis, April 2020

- R4-1808100, “Discussion on NR FR2 UE Type #2 and related RF requirement,” LGE, SK Telecom, KT, LG Uplus, RAN4 #87, May 2018

- 3GPP TS38.101-2, version 16.0.0, April 2019