3GPP TSG-RAN WG4 Meeting#95              	R4-2007155

E-meeting, May 25th – June 5th, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Text Proposal for NR EMR requirements for 36.133

Agenda item:	6.6.3.1.2

Document for:	Discussion

Introduction

Based on the discussion and proposals in [1] we have collected a TP collecting the proposals into requirements.

Text Proposal

## 4.9	Idle Mode CA Measurement

### 4.9.1	Introduction

A UE supporting ca-IdleModeMeasurements or [TBDfeature] shall perform the idle mode measurement on the inter-frequency CA or inter-RAT DC candidate frequencies/cells indicated by higher layers and meet the requirement specified in this section.

Additionally, a UE supporting ca-IdleModeMeasurements or [TBDfeature] shall perform the idle mode measurement on serving cell and meet the requirement specified in this section.

A UE only supporting ca-IdleModeMeasurements shall fulfil the requirements defined for a UE supporting ca-IdleModeMeasurements as defined in this section.

A UE only supporting [TBDfeature] shall fulfil the requirements defined for a UE supporting [TBDfeaure] as defined in this section.

A UE supporting both ca-IdleModeMeasurements and [TBDfeature] shall fulfil the requirements for a UE supporting ca-IdleModeMeasurements and [TBDfeature] as defined in this section.

### 4.9.2	Requirement

For a UE which supports ca-IdleModeMeasurements or [TBDfeature] the UE shall support the idle mode CA measurements on the serving cell, overlapping and non-overlapping carriers. An overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting and inter-frequency mobility measurements. A non-overlapping carrier is defined as a carrier configured by higher layer for early measurement reporting while not configured for inter-frequency mobility measurements.

#### 4.9.2.1	Detected cell requirement during state transition and Idle mode

This subclause defines the requirements for the detected cell status for the idle mode CA measurement when UE transitions from RRC Connected mode to Idle mode and after UE has entered Idle mode. The requirements are applicable to an E-UTRAN carrier aggregation capable UE which has been configured with one or more downlink SCells during the Connected mode and which supports ca-IdleModeMeasurements. The requirements are applicable for E-UTRAN FDD and TDD SCell(s). The requirements are additionally applicable to an EN-DC capable UE which has been configured with an NR PSCell and one or more NR SCell(s) during the connected mode and which support [TBDfeature]. The requirements are applicable for NR FDD and TDD PSCell and SCell(s).

Upon releasing the connection and if the UE has been configured with idle mode CA measurement reporting, following requirements apply concerning the detected cells in Connected mode upon state transitioning to Idle mode and during Idle mode:

-	A cell which is detected cell in Connected mode prior to connection release, shall remain detected after UE has entered Idle mode and during Idle mode, provided that the following conditions are met:

-	The UE has been provided with a list of cells and/or carrier frequencies for early measurement reporting by dedicated RRC signaling and

-	The detected cell is among the list of cells or on a carrier frequency provided for early measurement reporting, and

-	The UE is provided with a valid timer T331 by dedicated RRC signaling, and

-	The detected cell remains detectable until UE reconnects to the network and transmits the early measurement report.

-	The carrier frequency of the detected cell and the carrier frequency of the serving cell are among the supported band combination of the UE.

An inter-frequency cell is considered detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in Annex B.1.1 and Annex B.1.2 for a corresponding Band.

An NR inter-RAT cell is considered detectable according to the conditions in Annex B.1.2 and B.1.3 for a corresponding band in 38.133.

In the absence or expiration of T331, it is up to UE implementation to apply the requirements on the detected cell status in this subclause.

#### 4.9.2.2	Measurements of inter-frequency CA candidate cells

While T331 is running, the UE shall perform measurement on the configured overlapping and non-overlapping inter-frequency carriers for idle mode measurement reporting.

A UE which supports ca-IdleModeMeasurements shall support the idle mode CA measurements of at least [1, 2 or 3] inter-frequency carriers of which zero or one inter-frequency carrier may be a non-overlapping carrier.

For overlapping carriers, the inter-frequency measurement requirements in section 4.2.2.4 apply.

For non-overlapping carriers, at least prior to transmission of the idle mode measurement report, the UE shall perform at least a single measurement on detected cells on the non-overlapping inter-frequency carrier(s) configured to be measured for early measurement reporting.

In the absence or expiration of T331, it is up to UE implementation to perform the idle mode CA measurement.

For overlapping carriers, the UE shall be capable of performing RSRP and RSRQ measurements of the overlapping carriers, and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements of the overlapping carriers to higher layers, with measurement accuracy as specified in sub-clauses 9.1.3B.2 and 9.1.6B.2, respectively. The UE shall be able to report idle mode CA measurements when idle mode CA measurement reporting is requested by the network.

#### 4.9.2.3	Measurements on serving cell

The UE shall measure the RSRP and RSRQ level of the serving cell and evaluate the cell selection criterion S defined in section 4.2.2.1 and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements of the serving cell to higher layers, with measurement accuracy as specified in sub-clauses 9.1.2B.2 and 9.1.5B.2, respectively.

#### 4.9.2.4	Measurements of inter-RAT NR DC candidate cells

While T331 is running, the UE shall perform measurement on the configured overlapping and non-overlapping NR inter-RAT carriers for idle mode measurement reporting.

A UE which supports [TBDfeature] shall support the idle mode CA measurements of at least [2] NR inter-RAT carriers of which [TBD] inter-RAT carrier may be a non-overlapping carrier.

For overlapping carriers, the NR inter-RAT measurement requirements in section 4.2.2.5.6 apply.

For non-overlapping NR inter-RAT carriers in FR1, at least prior to transmission of the idle mode measurement report, the UE shall perform at least a single measurement on detected cells on the non-overlapping NR inter-RAT carrier(s) in FR1 configured to be measured for early measurement reporting.

For non-overlapping NR inter-RAT carriers in FR2, the NR inter-RAT measurement requirements in section 4.2.2.5.6 apply.

In the absence or expiration of T331, it is up to UE implementation to perform the idle mode CA measurement.

For overlapping carriers, the UE shall be capable of performing SS-RSRP and SS-RSRQ measurements of the overlapping carriers, and the UE physical layer shall be capable of reporting SS-RSRP and SS-RSRQ measurements of the overlapping carriers to higher layers, with measurement accuracy as specified in sub-clauses [TBD] and [TBD], respectively. The UE shall be able to report idle mode CA measurements when idle mode CA measurement reporting is requested by the network.

References

- R4-2007154, NR inter-RAT EMR requirements for 36.133, Nokia, Nokia Shanghai Bell