3GPP TSG-RAN WG4 Meeting #95-e	R4-2007191

# Electronic Meeting, 25th May – 5th June, 2020

Title: 	Discussion and simulation for URLLC BS high reliability with ultra-low BLER

Source: 	Huawei, HiSilicon

Agenda item:	6.9.1.1.2

Document for:	Discussion

Background

The URLLC high reliability with ultra-low BLER for BS will be discussed in this paper. According to the WF [1], some agreements were made and some open issues were remained to be discussed further:

Agreements:

- No slot aggregation for the ultra-low BLER requirement

- CP-OFDM waveform only

- UE DM-RS configuration is 1+1. Reconfirm or revise this decision in case the decision for DM-RS configuration for the slot aggregation requirement differs. BS DM-RS configuration is 1+1. Reconfirm or revise this decision in case the decision for DM-RS configuration for the slot aggregation requirement differs. These agreements apply for FR1 only.

- Regarding number of BS requirements: It is not of importance to agree a number; rather agree the number of different scenarios and the number of requirements will fall out. Note that currently 2 SCS and 2 mapping type are agreed; this number could change depending on agreements (There can be applicability rules to reduce testing of course).

- PUSCH bandwidth and number of RB

  - Same as agreed for the slot aggregation (high BLER) requirement

- Value for X

  - X value as [1] dB for BS requirements

Open issues:

- How to capture X in the specification

  - Option 1: Do not capture in specifications; include directly into core spec requirement by assuming part of IM

  - Option 2: Capture as part of TT in the conformance specification

  - Option 3: Do not capture in specifications, X is not part of IM.

- BS TDD pattern

  - Option 1: 3D1S1U (S=10:2:2) for 15 KHz, 7D1S2U (S=6:4:4) for 30 KHz

  - Option 2: DSUU

- Number of BS tests

  - Option 1: 1 using applicability rule

  - Option 2: 4

- FR2 requirements for ultra-low BLER

  - Keep it open meanwhile prioritize discussion on introducing FR1 requirements in Q2; and interested companies are encouraged to bring more information and analysis for the deployment/usage scenarios in FR2 with ultra-low BLER and/or higher BLER for high reliability and low latency

In this paper, the open issues are discussed and our views are provided.

Simulation

Simulation assumption

The parameters for BS are discussed in this section. Regarding to the TDD patterns, we propose to use the existing configurations. The agreed and proposed parameters are summarised in Table 2-1 according to WF [1] and [2]:

Table 2-1: Simulation assumption for BS PUSCH test requirements

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1x2, ULA low |
| PUSCH configuration | Mapping type | Type A and Type B |
|  | Starting symbol (S) | 0 |
|  | Length (L) | 14 |
|  | PUSCH aggregation factor | 1 |
| PUSCH DMRS configuration | DMRS Type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 1 |
| Number of HARQ Processes |  | 1 |
| PT-RS |  | Disabled |
| Propagation condition |  | AWGN |
| MCS Table |  | Table 3, MCS 5 |
| SCS and BW |  | 15 kHz for 10 MHz 30 kHz for 40 MHz |
| Frequency domain resource |  | Full Bandwidth |
| TDD pattern |  | 15 kHz SCS: 3D1S1U, S=10:2:2 30 kHz SCS: 7D1S2U, S=6:4:4 |
| Testing metric |  | Target BLER: 1-10-5 |

Simulation results

The simulation with parameters listed in Table 2-1 are still running. The results will be updated when it is ready.

Discussion

- How to capture X in the specification

Usually the core requirement captures the averaged value of (Ideal SNR + IM) from all companies. Then the conformance requirement captures: core requirement + TT.

Based on the experience of LTE BS and NR BS specification, sometimes large span among companies cannot be solved, then RAN4 agreed to add extra margin on top of the impairment results for final performance requirements definition that is similar way as UE side. But this extra margin was never explicitly captured in the core specification and conformance requirements, so we think that we can follow the same way forward.

From our understanding, we are discussing X that is similar as the extra margin used in BS performance requirements. Although X is used to reduce the test time, extra margin can be added for different reasons.

Proposal 1: Do not capture in specifications, include directly into core spec requirement but not part of IM. 
SNR value in core spec = average (Ideal SNR + IM) from all companies + X.

- BS TDD pattern

We prefer to use the exiting TDD pattern for BS performance requirements and this proposal is also align with the configuration for BS side with target BLER of 10^-2.

Proposal 2: We propose to use TDD pattern of 3D1S1U (S=10:2:2) for 15 KHz, 7D1S2U (S=6:4:4) for 30 KHz.

- Number of tests to define

We prefer to define one test for the ultra-low BLER target. More tests will be defined with higher BLER target.

Proposal 3: We propose to define one test with applicability rule for ultra-low BLER target.

Proposals

In this paper, following are proposed:

Proposal 1: Do not capture in specifications, include directly into core spec requirement but not part of IM. 
SNR value in core spec = average (Ideal SNR + IM) from all companies + X.

Proposal 2: We propose to use TDD pattern of 3D1S1U (S=10:2:2) for 15 KHz, 7D1S2U (S=6:4:4) for 30 KHz.

Proposal 3: We propose to define one test with applicability rule for ultra-low BLER target.

Reference

[1] R4-2005526, Way forward on NR URLLC performance requirements with ultra-low BLER, #94bis, Ericsson.

[2] R4-2002422, Way forward on parameters and test methodology for URLLC ultra-low BLER test, #94, Ericsson.