3GPP TSG-RAN WG4 Meeting #95-e	R4-2007193

# Electronic Meeting, 25th May – 5th June, 2020

Title: 	Discussion and simulation on URLLC UE performance requirements for verifying PDSCH mapping Type B and processing capabiltiy 2

Source: 	Huawei, HiSilicon

Agenda item:	6.9.1.2.1

Document for:	Discussion

Background

RAN4 agreed to verify PDSCH mapping type B and PDSCH processing capability 2 together. In this paper, the simulation results are provided based on the agreed parameters. In addition, the open issues are discussed. Agreements and open issues of #94bis e-meeting are listed below based on the WF [1]:

Agreements:

- SCS/CBW:

  - FDD: 15 KHz/10 MHz

  - TDD: 30 KHz/ 40 MHz

- TDD pattern (30KHz SCS): DDDSU, S=10:2:2

- Number of HARQ process: FDD: 2

- K1 for FDD: K1=0

- Channel model: TDLA30-10

- Antenna configuration: 2x2 and 2x4, ULA Low.

- PDSCH symbol length for FDD and TDD:2

- Number of RBs: Full bandwidth only for MCS4

Open Issues:

- Number of HARQ process for TDD:

  - Option 1: 2

  - Option 2: 4

- MCS:

  - Option 1: Only MCS 4

  - Option 2: Only MCS 17

  - Option 3: MCS 4 and MCS 17

- Number of RBs: FFS for MCS17

- PDSCH symbol length for FDD and TDD: FFS whether to define additional requirements for 4os or 7os

- Test metrics: Based on 70% throughput or 30% BLER

- FR2 requirements for PDSCH mapping Type B and processing capability 2: Keep it open meanwhile prioritize discussion on introducing FR1 requirements in Q2; and interested companies are encouraged to bring more information and analysis for the deployment/usage scenarios

Simulation

Simulation assumption

The test parameters according to the agreements of #94 and #94bis e-meeting and the parameters we proposed are concluded in table below:

Table 2-1: Test parameters for UE PDSCH performance requirements

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Duplex mode |  | FDD/TDD |
| Antenna configuration |  | 2x2 and 2x4, ULA low |
| PDSCH configuration | Mapping type | Type B |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 2 |
|  | PDSCH aggregation factor | 1 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 1 |
| Maximum number of HARQ transmission |  | 1 |
| Number of HARQ process |  | 2 |
| Propagation condition |  | TDLA30-10 |
| MCS Table |  | Table 1, MCS 4 |
| SCS and BW |  | FDD:15KHz, 10MHz TDD:30KHz, 40MHz |
| TDD Pattern |  | DDDSU, S=10:2:2 |
| Frequency domain resource |  | Full BW |
| K1 |  | 0 |
| Testing metric |  | Throughput: 70% |

Simulation results:

PDSCH performance with for FDD and TDD were simulated based on the parameter assumption in Table 2-1.

- FDD

The performance for 2x2 and 2x4 for symbol lengths of 2 and 7 are simulated as shown in figure below:

Figure 2-1: Throughput vs SNR for PDSCH FDD with symbol lengths of 2 and 7 for antenna configuration 2x2 and 2x4

According to the simulation results shown in Figure 2-1, the SNR values of 70% throughput are listed in table below:

Table 2-2: SNR values of 70% throughput for FDD

| Symbol length SNR Antenna configuration | 2x2 | 2x4 |
| --- | --- | --- |
| L=2 | -2.4 | -5.7 |
| L=7 | -2.8 | -5.8 |

- TDD

The PDSCH mapping Type B for TDD was agreed that only scheduled on S slot. The simulation results of PDSCH TDD with symbol lengths of 2 and 7 are shown in below:

Figure 2-2: Throughput vs SNR for PDSCH TDD with symbol lengths of 2 and 7 for antenna configuration 2x2 and 2x4

According to the simulation results shown in Figure 2-2, the SNR values of 70% throughput are listed in table below:

Table 2-3: SNR values of 70% throughput for TDD

| Symbol length SNR Antenna configuration | 2x2 | 2x4 |
| --- | --- | --- |
| L=2 | -2.2 | -5.5 |
| L=7 | -2.3 | -5.5 |

The performance is similar with different symbol length, as we already mentioned during the #94bis e-meeting that 7os has been defined in Rel-15 and short symbol length is the typical configuration for URLLC. Thus, we prefer to only define 2os.

Proposal 1: Only define symbol length of 2 to verify mapping Type B and processing capability 2.

Discussion

For TDD, only S slot will be scheduled, the corresponding HARQ process only needs to counter S, no need to consider D slot, i.e.

Thus, 2 HARQ processes are enough for “DDDSU”.

Proposal 2: HARQ process is 2 for TDD.

Proposals

In this contribution, we discussed the open issues for the performance requirements for PDSCH mapping Type B and PDSCH processing time capability 2. Proposals are list as follows:

Proposal 1: Only define symbol length of 2 to verify mapping Type B and processing capability 2.

Proposal 2: HARQ process is 2 for TDD.

Reference

- R4-2005527, “Way forward for NR URLLC performance requirements”, Intel Corporation.