3GPP TSG-RAN WG4 Meeting #95-e	R4-2007197

# Electronic Meeting, 25th May – 5th Apr, 2020

Title: 	Discussion and simulation on URLLC BS performance requirements for low latency

Source: 	Huawei, HiSilicon

Agenda item:	6.9.1.2.2

Document for:	Discussion

Background

According to the WF [1] from the last meeting, when considering the URLLC BS demodulation requirements for low latency, the demodulation requirements PUSCH mapping Type B was agreed to be verified. During RAN4 #94 meeting, some parameters for FR1 have been agreed:

Agreements:

- MCS: MCS 5 from Table 3

- Number of PRB: Full bandwidth for MCS5

- Not introduce performance requirements for DFT-s-OFDM

- SCS/CBW: 15 kHz for 10 MHz, 30 kHz for 40 MHz

- Test applicability for different SCS and channel bandwidth: Reuse the test applicability rules defined for NR Rel-15 PUSCH performance requirements

- Whether to define requirements for FR2: Keep it open meanwhile prioritize discussion on introducing FR1 requirements in Q2; and interested companies are encouraged to bring more information and analysis for the deployment/usage scenarios in FR2 with ultra-low BLER and/or higher BLER for high reliability and low latency

Open issues:

- Symbol length

  - Option 1: 4os

  - Option 2: 7os

- SCS/CBW (15 KHz/10 MHz, 30 KHz/40 MHz have been agreed)

  - Option 1: 15 KHz for 5/10 MHz, 30 KHz for 10/40 MHz

  - Option 2: Only 15 KHz/10 MHz, 30 KHz/40 MHz

- Test metrics

  - Option 1: 70% throughput

  - Option 2: 30% BLER

Note: Discuss whether to word as throughput, BLER, success rate or something similar.

- Whether to define requirements for FR2

  - Option 1: No

  - Option 2: Yes

- Test applicability rule for FR1 and FR2 (only if FR2 is agreed)

  - Option 1: Only FR1 or FR2 will be tested based on BS declaration.

  - Option 2: Both FR1 and FR2 should be tested for BS that supports both FR1 and FR2.

  - Option 3: Which tests related to FR1 and FR2 be tested shall be based on BS declaration: [FR1], [FR2], [FR1&FR2]

- SCS/CBW for FR2 (only if FR2 is agreed)

  - 60 KHz:

    - Option 1: 50 MHz

    - Option 2: 50 MHz and 100 MHz

  - 120 KHz

    - Option 1: 100 MHz

    - Option 2: 50 MHz and 100 MHz

In this paper, the remained issues are discussed and analysed. Our views are provided.

Simulation

Simulation assumption

The test parameters according to the agreements of #94 and #94bis e-meeting and the parameters we proposed are concluded in table below:

Table 2-1 Simulation assumption for PUSCH mapping Type B

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1x2, low |
| SCS and BW |  | 15 kHz / 10 MHz 30 kHz / 40 MHz |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | 0 |
| Time domain resource assignment | PUSCH mapping type | Type B |
|  | Start symbol | 0 |
|  | Allocation length | 4 |
|  | PUSCH aggregation factor | 1 |
| SCS and BW |  | 15 kHz / 10 MHz 30 kHz / 40 MHz |
| TDD pattern |  | 15 kHz SCS: 3D1S1U, S=10:2:2 30 kHz SCS: 7D1S2U, S=6:4:4 |
| MCS Table |  | Table 3, MCS 5 |
| Propagation condition |  | TDLC300-100 Low |
| Maximum number of HARQ transmissions |  | 1 |
| Frequency domain resource assignment |  | Full bandwidth |
| Test metric |  | 70% TP |

Simulation results

The performance for PUSCH mapping Type B for FDD/TDD with symbol length of 4 are presented in figure below:

- FDD

Based on the agreed and proposed parameters listed in Table 2-1, the simulated performances for FDD are shown in Figure 2-1 for symbol length of 4.

Figure 2-1: Throughput vs SNR for PUSCH FDD with symbol length of 4

The SNR values of 70% throughput for FDD are list in table below:

Table 2-2: SNR values of 70% throughput for FDD

| Symbol length SNR Antenna configuration | 1x2 |
| --- | --- |
| L=4 | -8.5 |

- TDD

Based on the agreed and proposed parameters listed in Table 2-1, the simulated performances for TDD are shown in Figure 2-1 for symbol length of 4.

Figure 2-2: Throughput vs SNR for PUSCH TDD with symbol length of 4

The SNR values of 70% throughput for TDD are list in table below:

Table 2-3: SNR values of 70% throughput for TDD

| Symbol length SNR Antenna configuration | 1x2 |
| --- | --- |
| L=4 | -8.2 |

Based on our analysis, the performance is similar with different symbol lengths. As the short duration is the typical scheduling, we propose to define 4os.

Proposal 1: We propose to define symbol length of 4.

Proposals

In this contribution, we discussed PUSCH mapping Type B. Following has been proposed:

Proposal 1: We propose to define symbol length of 4.

Reference

[1] R4-2005528, Way forward on NR BS URLLC performance requirements, #94bis, Huawei, Hisilicon.