# 3GPP TSG-RAN WG4 Meeting # 95-e 															R4-2007308

# Electronic Meeting, 25 May – 5 June, 2020

Source: 	Huawei, HiSilicon

Title: 	TP on co-existence simulation assumption on 6.425-7.125GHz and 10.0-10.5GHz

Agenda Item:	11.1.3

Document for:	Approval

Introduction

The SID on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz was approved at RAN #87e [1], in which RAN4 is tasked to study and evaluate following parameters:

  - Transmitter and receiver characteristics for the NR UE.

  - Transmitter and receiver characteristics for the NR BS.

Note: For 10000-10500MHz, TR 38.820 (7-24GHz SI) will be an input to this study.

To fulfil such tasks, RAN4 needs to perform coexistence study to identify conditions of coexistence in adjacent channels, such as ACLR and ACS. In last meeting, RAN4 made some initial discussion and a WF [2] was approved.

Discussion

Coexistence Scenarios

Deployment scenarios for indoor hotspot, dense urban, and urban macro are discussed in last meeting. Indoor hotspot and dense urban can be down-prioritized for adjacent channel co-existence simulation on 6.425-7.125GHz and 10.0-10.5GHz.

Regarding to the carrier frequency, the carrier frequency candidates are 7GHz and 10GHz.

The simulation bandwidth can be set as 100 MHz per CC.

The duplex mode is TDD for both 7GHz and 10GHz. Aggressor and victim operators should be set as the same UL or DL configuration. RAN4 doesn’t need to consider cross link interference at this stage.

Table 1 summarizes the proposed initial simulation scenarios for adjacent channel co-existence study on 6.425-7.125GHz and 10.0-10.5GHz.

Proposal 1: It is proposed to focus on following scenario for adjacent channel co-existence study on 6.425-7.125GHz and 10.0-10.5GHz.

Table 2.1-1: Summary of initial simulation scenarios for 6.425-7.125GHz and 10.0-10.5GHz

| Scenario No. | Deployment Scenario | Simulation frequency | Aggressor | Victim | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | Urban macro | 7 GHz | NR, 100MHz, DL | NR, 100MHz, DL |  |
| 2 |  |  | NR, 100MHz, UL | NR, 100MHz, UL |  |
| 3 |  | 10 GHz | NR, 100MHz, DL | NR, 100MHz, DL |  |
| 4 |  |  | NR, 100MHz, UL | NR, 100MHz, UL |  |
| 5 | Dense urban | 7 GHz | NR, 100MHz, DL | NR, 100MHz, DL | Down-prioritized |
| 6 |  |  | NR, 100MHz, UL | NR, 100MHz, UL | Down-prioritized |
| 7 |  | 10 GHz | NR, 100MHz, DL | NR, 100MHz, DL | Down-prioritized |
| 8 |  |  | NR, 100MHz, UL | NR, 100MHz, UL | Down-prioritized |
| 9 | Indoor hotspot | 7 GHz | NR, 100MHz, DL | NR, 100MHz, DL | Down-prioritized |
| 10 |  |  | NR, 100MHz, UL | NR, 100MHz, UL | Down-prioritized |
| 11 |  | 10 GHz | NR, 100MHz, DL | NR, 100MHz, DL | Down-prioritized |
| 12 |  |  | NR, 100MHz, UL | NR, 100MHz, UL | Down-prioritized |

Simulation Assumptions

Network layout model

Network layout model are listed in table 2.2.1-1. For the ISD, we can reuse the FR1’s assumption 500m.

Table 2.2.1-1 Network layout model for 7GHz and 10GHz

| Scenarios |  | Assumption |
| --- | --- | --- |
| Urban macro | layout | TR 38.803 |
|  | Multi-operators | Coordinated and uncoordinated referring to TR 36.942 |
|  | ISD | 500m |
|  | BS antenna height | 25m |
|  | UE location | in table 5.2.1.1-1 fromTR 38.803 |
|  | UE distribution (horizontal) | Uniform |
|  | Minimum BS - UE distance (2D) | 35m |
| Dense Urban | layout | TR 38.803 |
|  | Multi-operators | Coordinated |
|  | Number of micro BSs per macro cell | 3 (3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS.) |
|  | Radius of UE dropping within a micro cell | < 72.17 m ( Value = ISD/3*sqrt(3)/2/2 ) |
|  | BS antenna height | 10 m |
|  | UE location | in table 5.2.1.2-1 fromTR 38.803 |
|  | UE distribution (horizontal) | Uniform |
|  | Minimum BS - UE distance (2D) | 3m |
| Indoor | layout | TR 38.803 |
|  | Multi-operators | Coordinated |
|  | Inter-site distance | 20m |
|  | BS antenna height | 3 m (ceiling) |
|  | UE location | in table 5.2.1.3-1 fromTR 38.803 |
|  | UE distribution (horizontal) | Uniform |
|  | Minimum BS - UE distance (2D) | 0m |

Propagation model

Propagation model are listed in table 2.2.2-1.

Table 2.2.2-1	Propagation model for 7GHz and 10GHz

| Propagation model | Path Loss | TR 38.803 with UMa LOS formula from TR 38.901 |
| --- | --- | --- |
|  | LOS probability | TR 38.803 |
|  | O-to-I penetration loss | TR 38.803 |

Transmission power control model

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 is applied with following parameters.

-	CLx-ile = 88 + 10*log10(200/X), where X is UL transmission BW (MHz)

-	γ = 1

Received power model

The Received power model in TR 38.803 can be reused.

ACLR and ACS modelling

The ACLR and ACS modelling in TR 38.803 can be reused.

Link level performance for 5G NR coexistence

The Link level performance for 5G NR coexistence in TR 38.803 can be reused.

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Throughput SNIR, bps/Hz =0                                  for SNIR                                         ∝∙SSNIR                     for SNIRMIN≤SNIR<SNIRMAX ∝∙SSNIRMAX               for SNIR ≥ SNIRMAX

Where:

S(SNIR)   Shannon bound, S(SNIR) =log2(1+SNIR)  bps/Hz
			Attenuation factor, representing implementation losses
SNIRMIN  	Minimum SNIR of the code set, dB
SNIRMAX  Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in table 2.2.6-1 represent a baseline case, which assumes:

-	1:1 antenna configurations

-	AWGN channel model

-	Link Adaptation (see table 2.2.6-1 for details of the highest and lowest rate codes)

-	No HARQ

Table 2.2.6-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

Other simulation parameters

Based on the TR 38.820, 7 dB NF is used for 10GHz as an example. 7GHz belongs to FR1, so 5dB NF is reasonable. If we assume that the micro BS and Macro BS have the same coverage capability. We can use this formula 20*log(2/3*ISD/(2*Radius)) = 20*log(333.3/144)=7.3dB. Thus, BS max TX power for micro is 36dBm = 43dBm-7.3dB.

Table 2.2.7-1: Other simulation parameters

| Parameters | 7GHz | 10GHz |
| --- | --- | --- |
| Channel bandwidth | 100MHz | 100MHz |
| Scheduled channel bandwidth per UE (DL) | 100MHz | 100MHz |
| Scheduled channel bandwidth per UE (UL) | 100MHz | 100MHz |
| The number of active UE (DL) | Same as the number of BS beam | Same as the number of BS beam |
| The number of active UE (UL) | Same as the number of BS beam | Same as the number of BS beam |
| Traffic model | Full buffer | Full buffer |
| DL power control | NO | NO |
| UL power control | YES | YES |
| BS max TX power in dBm (Urban Macro) | 43dBm | 43dBm |
| BS max TX power in dBm (Dense Urban) | 36dBm | 36dBm |
| BS max TX power in dBm (Indoor) | 24dBm | 24dBm |
| UE max TX power in dBm | 23dBm | 23dBm |
| UE min TX power in dBm | -33dBm | -33dBm |
| BS Noise figure in dB | 5dB | 7dB |
| UE Noise figure in dB | 9dB | 9dB |
| UE antenna | Isotropic (0dBi) | Isotropic (0dBi) |
| Handover margin | 3dB | 3dB |

Co-existence simulation methodology

Adopt following simulation steps.

1.	Aggressor and victim network are generated.

-	UEs are distributed randomly across the network.

2.	UE associations: UEs are associated to base station based on coupling loss.

-	Associations are made assuming a single element at both UE and BS.

3.	Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

4.	Throughput is computed in the victim systems without considering ACI as below:

-	ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

5.	Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

6.	RF parameters are determined based on the degradation cause by ACI as below:

-	LossACI=1-ThputACIThputSINGLE.

Text proposal

<TP for TR 38.9de>

- 4	Co-existence study

- 4.1	Co-existence simulation scenarios

Table 4.1-1 summarizes the proposed initial simulation scenarios for 6.425-7.125GHz and 10.0-10.5GHz

Table 4.1-1: Summary of initial simulation scenarios for 6.425-7.125GHz and 10.0-10.5GHz

| Scenario No. | Deployment Scenario | Simulation frequency | Aggressor | Victim | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | Urban macro | 7 GHz | NR, 100MHz, DL | NR, 100MHz, DL |  |
| 2 |  |  | NR, 100MHz, UL | NR, 100MHz, UL |  |
| 3 |  | 10 GHz | NR, 100MHz, DL | NR, 100MHz, DL |  |
| 4 |  |  | NR, 100MHz, UL | NR, 100MHz, UL |  |
| 5 | Dense urban | 7 GHz | NR, 100MHz, DL | NR, 100MHz, DL | Down-prioritized |
| 6 |  |  | NR, 100MHz, UL | NR, 100MHz, UL | Down-prioritized |
| 7 |  | 10 GHz | NR, 100MHz, DL | NR, 100MHz, DL | Down-prioritized |
| 8 |  |  | NR, 100MHz, UL | NR, 100MHz, UL | Down-prioritized |
| 9 | Indoor hotspot | 7 GHz | NR, 100MHz, DL | NR, 100MHz, DL | Down-prioritized |
| 10 |  |  | NR, 100MHz, UL | NR, 100MHz, UL | Down-prioritized |
| 11 |  | 10 GHz | NR, 100MHz, DL | NR, 100MHz, DL | Down-prioritized |
| 12 |  |  | NR, 100MHz, UL | NR, 100MHz, UL | Down-prioritized |

- 4.2	Co-existence simulation assumption

- 4.2.1 Network layout model

Network layout model are listed in table 4.2.1-1.

Table 4.2.1-1 Network layout model for 7GHz and 10GHz

| Scenarios |  | Assumption |
| --- | --- | --- |
| Urban macro | layout | TR 38.803 |
|  | Multi-operators | Coordinated and uncoordinated referring to TR 36.942 |
|  | ISD | 500m |
|  | BS antenna height | 25m |
|  | UE location | in table 5.2.1.1-1 fromTR 38.803 |
|  | UE distribution (horizontal) | Uniform |
|  | Minimum BS - UE distance (2D) | 35m |
| Dense Urban | layout | TR 38.803 |
|  | Multi-operators | Coordinated |
|  | Number of micro BSs per macro cell | 3 (3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS.) |
|  | Radius of UE dropping within a micro cell | < 72.17 m ( Value = ISD/3*sqrt(3)/2/2 ) |
|  | BS antenna height | 10 m |
|  | UE location | in table 5.2.1.2-1 from TR 38.803 |
|  | UE distribution (horizontal) | Uniform |
|  | Minimum BS - UE distance (2D) | 3m |
| Indoor | layout | TR 38.803 |
|  | Multi-operators | Coordinated |
|  | Inter-site distance | 20m |
|  | BS antenna height | 3 m (ceiling) |
|  | UE location | in table 5.2.1.3-1 fromTR 38.803 |
|  | UE distribution (horizontal) | Uniform |
|  | Minimum BS - UE distance (2D) | 0m |

- 4.2.2 Propagation model

Propagation model are listed in table 2.2.2-1.

Table 4.2.2-1	Propagation model for 7GHz and 10GHz

| Propagation model | Path Loss | TR 38.803 with UMa LOS formula from TR 38.901 |
| --- | --- | --- |
|  | LOS probability | TR 38.803 |
|  | O-to-I penetration loss | TR 38.803 |

- 4.2.3 Transmission power control model

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 is applied with following parameters.

-	CLx-ile = 88 + 10*log10(200/X), where X is UL transmission BW (MHz)

-	γ = 1

- 4.2.4 Received power model

The Received power model in TR 38.803 can be reused.

- 4.2.5 ACLR and ACS modelling

The ACLR and ACS modelling in TR 38.803 can be reused.

- 4.2.6 Link level performance for 5G NR coexistence

The Link level performance for 5G NR coexistence in TR 38.803 can be reused.

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Throughput SNIR, bps/Hz =0                                  for SNIR                                         ∝∙SSNIR                     for SNIRMIN≤SNIR<SNIRMAX ∝∙SSNIRMAX               for SNIR ≥ SNIRMAX

Where:

S(SNIR)   Shannon bound, S(SNIR) =log2(1+SNIR)  bps/Hz
			Attenuation factor, representing implementation losses
SNIRMIN  	Minimum SNIR of the code set, dB
SNIRMAX  Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in table 2.2.6-1 represent a baseline case, which assumes:

-	1:1 antenna configurations

-	AWGN channel model

-	Link Adaptation (see table 4.2.6-1 for details of the highest and lowest rate codes)

-	No HARQ

Table 4.2.6-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

- 4.2.7 Other simulation parameters

Table 4.2.7-1: Other simulation parameters

| Parameters | 7GHz | 10GHz |
| --- | --- | --- |
| Channel bandwidth | 100MHz | 100MHz |
| Scheduled channel bandwidth per UE (DL) | 100MHz | 100MHz |
| Scheduled channel bandwidth per UE (UL) | 100MHz | 100MHz |
| The number of active UE (DL) | Same as the number of BS beam | Same as the number of BS beam |
| The number of active UE (UL) | Same as the number of BS beam | Same as the number of BS beam |
| Traffic model | Full buffer | Full buffer |
| DL power control | NO | NO |
| UL power control | YES | YES |
| BS max TX power in dBm (Urban Macro) | 43dBm | 43dBm |
| BS max TX power in dBm (Dense Urban) | 36dBm | 36dBm |
| BS max TX power in dBm (Indoor) | 24dBm | 24dBm |
| UE max TX power in dBm | 23dBm | 23dBm |
| UE min TX power in dBm | -33dBm | -33dBm |
| BS Noise figure in dB | 5dB | 7dB |
| UE Noise figure in dB | 9dB | 9dB |
| UE antenna | Isotropic (0dBi) | Isotropic (0dBi) |
| Handover margin | 3dB | 3dB |

- 4.3	Co-existence simulation methodology

Adopt following simulation steps.

1.	Aggressor and victim network are generated.

-	UEs are distributed randomly across the network.

2.	UE associations: UEs are associated to base station based on coupling loss.

-	Associations are made assuming a single element at both UE and BS.

3.	Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

4.	Throughput is computed in the victim systems without considering ACI as below:

-	ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

5.	Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

6.	RF parameters are determined based on the degradation cause by ACI as below:

-	LossACI=1-ThputACIThputSINGLE.

- 4.4	Co-existence simulation results

<End of TP >

- 4 References

- RP-200513, New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz, Ericsson, Huawei, HiSilicon

- R4-2005174, WF on Simulation Assumptions for the SI on 6.425-7.125GHz and 10.0-10.5GHz, Nokia

- TR 36.828, Further enhancements to LTE Time Division Duplex (TDD) for Downlink-Uplink (DL-UL) interference management and traffic adaptation

- TR 38.803, Study on new radio access technology: Radio Frequency (RF) and co-existence aspects

- TR 38.817, General aspects for Base Station(BS) Radio Frequency(RF) for NR