3GPP TSG-RAN WG4 Meeting #95-e 	R4-2007310

Electronic Meeting, 25 May – 5 June, 2020

Source: 	Huawei

Title: 	Reply of IMT parameters for other frequency ranges below 5 GHz

Agenda Item:	11.1.2.

Document for:	Approval

- Introduction

The LS RP-200042 requests information on some new IMT frequency bands 6 425-7 025 MHz, 7 025-7 125 MHz and 10.0-10.5 GHz which are discussed in separate agenda item. But also consideration of parameters for existing bands, 470-694 MHz, 694 960 MHz, 1 710-1 885 MHz, 1 885-1 980 MHz, 2 010-2 025 MHz, 2 110-2 170 MHz, 2 500 2 690 MHz, 3 300-3 400 MHz, 3 600-3 800 MHz, 4 800-4 990 MHz some of which have been replied to in the past and many of which are covered by generic FR1 requirements.

LS reply to WP5D have been discussed in last RAN4#94e-Bis meeting and the WF was approved in [2]. This paper provide further discussion on the remaining issues.

- Discussion

For IMT parameters, open issues are BS average output power and UE average output power. For the BS the average output power (conducted or TRP) is the same meaning as 4.5 Maximum output power, note for EIRP in any direction this is not necessarily the case, the peak EIRP is related to the array gain whereas the average EIRP is more closely related to the element gain. For UE the average output power is related the deployment scenario. Hence single value might not be possible for reply. It can be simulated once the deployment parameters are decided. We think the two parameters are not needed for the reply to WP5D. Similar proposal is approved for frequency range 6.425-7.125GHz and 10.0-10.5GHz [3].

On AAS BS antenna characteristics, it was agreed to provide the parameters for frequency range 1710 – 4990 MHz. Two set of antenna parameters might be necessary

  - Around 2GHz

As with sub 1 GHz for the non-AAS assumptions we have to look back to TR 36.942.

The AAS simulation work was done assuming frequencies around 2GHz, and the goal was that a minimum equivalent protection and performance was offered.

Antenna definitions and assumptions are captured in TR 37.840 and TR 37.842. Initial investigations were on single column antennas of the same size as typical passive antennas with additional investigations of multiple column antennas up to 4 columns

Table 2-1 Beamforming antenna characteristics for around 2GHz

|  |  | Comments |
| --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |
| 1.1 | Antenna pattern | Refer to Recommendation ITU-R M.2101 |
| 1.2 | Element gain (dBi) | 5.8dB |
| 1.3 | Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 65º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30dB |
| 1.5 | Antenna polarization | Dual polarized: Linear ±45º |
| 1.6 | Antenna array configuration (Row × Column) | 8x4 |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.5λ for H 0.9λ for V |
| 1.8 | Array Ohmic loss (dB) | 2dB |
| 1.9 | Conducted power (before Ohmic loss) per antenna element | 43 to 46 dBm TRP |
| 1.10 | Base station maximum coverage angle in the horizontal plane (degrees) | 3 sector |

  - Above 2.5 GHz

Higher frequency operating bands which have been analysed since the original AAS work, the higher frequency (and hence smaller antenna) and the fact they are TDD has made them interesting for AAS. In general analysis has assumed larger antennas geometries with up to 8 columns.

Initial estimations used similar vertical definitions to the 2GHz antenna where the vertical gain was based on approx. 8 to 10 elements with a 0.9λ spacing. The wide spacing in the vertical dimensions leads to grating lobes that do not greatly degrade the mobile network, but can cause excessive interference to satellite if they are close in frequency. As such later responses have favoured maintaining the total vertical dimensions but using more elements at closer spacing to achieve it. For the case without coexistence with satellite, the parameters in LS to ITU on 3300-3400 MHz (RP-182783) can be used.

Table 2-2 (Option 1): Beamforming antenna characteristics for above 2.5 GHz below 5 GHz

|  |  | Comments |
| --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |
| 1.1 | Antenna pattern | Refer to Recommendation ITU-R M.2101 |
| 1.2 | Element gain (dBi) | 5.8dB |
| 1.3 | Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 65º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30dB |
| 1.5 | Antenna polarization | Dual polarized: Linear ±45º |
| 1.6 | Antenna array configuration (Row × Column) | 8x8 |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.5λ for H 0.9λ for V |
| 1.8 | Array Ohmic loss (dB) | 2dB |
| 1.9 | Conducted power (before Ohmic loss) per antenna element | 43 to 46 dBm TRP |
| 1.10 | Base station maximum coverage angle in the horizontal plane (degrees) | 3 sector |

Table 2-3 (Option 2): Beamforming antenna characteristics for above 2.5 GHz below 5 GHz

|  |  | Comments |
| --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |
| 1.1 | Antenna pattern | Refer to Recommendation ITU-R M.2101 |
| 1.2 | Element gain (dBi) | 3.5dB |
| 1.3 | Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 90º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30dB |
| 1.5 | Antenna polarization | Dual polarized: Linear ±45º |
| 1.6 | Antenna array configuration (Row × Column) | 16x8 |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.5λ for H 0.5λ for V |
| 1.8 | Array Ohmic loss (dB) | 2dB |
| 1.9 | Conducted power (before Ohmic loss) per antenna element | 43 to 46 dBm TRP |
| 1.10 | Base station maximum coverage angle in the horizontal plane (degrees) | 3 sector |

- Conclusion

This paper provides further discussion on the remaining issues.

Proposal 1：The two parameters (BS average output power and UE average output power) are not needed for the reply to WP5D

Proposal 2: Beamforming antenna characteristics for around 2GHz is shown in Table 2-1.

Proposal 3: Beamforming antenna characteristics for above 2.5 GHz is shown in Table 2-2 and 2-3.

References

[1]	RP-200513, New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz	Ericsson, Huawei, HiSilicon

[2] R4-2005170, WF on LS Reply to ITU-R WP5D for the frequency ranges below 6 GHz, Huawei

[3] R4-2005733, WF on BS and UE IMT parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz, CATT