3GPP TSG-RAN WG4 #95-e	R4- 2007361

Electronic Meeting, 25th May – 5th June, 2020

Title:	LTE/NR spectrum sharing in Band 38/n38

Source:	Vodafone

Agenda: 	14.2.4

For:		Discussion

# Introduction

This document provides some views on some remaining details of V2X SL/Uu concurrent operation, in order to progress beyond what was agreed in R4-2002787 and R4-2002789 in the RAN4#94e meeting.

# Discussion

Dynamic Spectrum Sharing is is useful to allow operators to perform a soft re-farming of LTE spectrum towards NR. For Dynamic Spectrum Sharing support between LTE and NR in TDD bands, there are specific RAN4 core requirements that need to be specified for NR UEs. So far, these requirements have only been completed for the n41 spectrum range as n90. We also see a need to enable DSS for band 38 spectrum.

## DSS requirements for n38 NR band

For resource block level spectrum sharing, frequency alignment between LTE and NR is necessary to avoid inter-subcarrier interference. As mentioned in [1], the following three features are needed for band 38 spectrum sharing:

### 100kHz channel raster support

As shown in the following table, 100 kHz channel raster is supported for n38 in Rel-15:

| NR Operating Band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| ~ omitted ~ |  |  |  |
| n38 | 100 | 514000 – <20> – 524000 | 514000 – <20> – 524000 |
| ~ omitted ~ |  |  |  |

Table 1. Referred from TS38.101-1 Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

The NR band n38 already complies with this requirement.

### 30 KHz SS block SCS support

In the case of NR band n38 the 30 KHz SS block support is missing as shown in the following table. It is required to achieve UL-DL coexistence and for avoiding LTE CRS REs allocation. Therefore, changes are needed in Table 5.4.3.3-1 of TS38.101-1 on band n38 to support 30 KHz SS block SCS.

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| ~ omitted ~ |  |  |  |
| n38 | 15kHz | Case A | 6431 – <1> – 6544 |
| ~ omitted ~ |  |  |  |

Table 2. Referred from TS38.101-1 Table 5.4.3.3-1: Applicable SS raster entries per operating band

Proposal 1: 30 kHz SS SCS support should be added for n38 in Rel-16.

### UL 7.5kHz frequency shift

This feature is required for the UL sub-carrier alignment of UL spectrum sharing, described in detail in [2]. In section 5.4.2.1 of TS38.101-1, inclusion of band n38 for the list of bands supporting UL 7.5 kHz raster is required.

Proposal 2: 7.5 kHz raster support should be added for n38 in Rel-16.

# Conclusion

Vodafone plans to submit the attached WID containing the aforementioned proposals to the RAN#88e meeting.