3GPP RAN4#95-e	R4-2007487

Online Meeting, May 25th – June 5th, 2020

Title: 	Discussion regarding BFD and CBD requirements of IAB-MTs

Source: 	Qualcomm Incorporated

Agenda item:	6.5.3.5

Document for:	Discussion

# Introduction

During the last two meeting [1][2], RAN4 made the following agreement regarding beam failure detection requirements in IAB networks:

RAN4 94e - BFD/CBD requirements for IAB-MTs are defined for no-DRX mode only. RAN4 94bis-e - Sharing factor P is also necessary since measurement gap can be configured for IAB-MTs. - The following CBD related requirements, that have been defined in 38.133 for Rel-15 UEs, can be reused for IAB-MTs. - Measurement restrictions for SSB based CBD and CSI-RS based CBD - Scheduling availability during CBD - For IAB CBD requirement, down-select beam sweeping factor N for FR2 from following options during the next meeting - Option 1: N = 8 - Option 2: N = 4 - Option 3: N = 6

There are still quite a few open issues regarding BFD in IAB networks. Beam sweeping factor for CBD is still unresolved. We discuss those in our work.

# Discussion

## FR2 Beam Sweeping Factor

Rel-16 IAB-MTs are fixed nodes. Once they lose their BFD-RS, they may have better understanding about the angle of arrival for the CBD-RS. But, IAB-MTs may need to use narrower RX beams to obtain higher link budget. Besides, the beam sweeping factor for fixed UEs (power class 1) was also decided to be 8 in Rel-15. Hence, same beam sweeping factor (N = 8) should be used to evaluate CBD-RS for IAB-MTs.

Observation 1: IAB-MTs may need to use narrower RX beams to obtain higher link budget.

Observation 2: the beam sweeping factor for fixed UEs (power class 1) was also decided to be 8 in Rel-15.

Proposal 1: For IAB CBD requirement, use beam sweeping factor N=8 in evaluation period calculation.

In Rel-15, beam sweeping factor N was defined to be 8 for SSB based BFD evaluation period because UE would have to try for all eight RX beams to monitor SSB. For CSI-RS based BFD evaluation period, N = 1 was defined if the QCL relationship of CSI-RS were known and if CSI-RS were configured with repetition = OFF. On the other hand, if CSI-RS were configured with repetition = ON, no requirement was defined.

Proposal 2: For IAB BFD requirement, reuse the beam sweeping factors that were defined for UEs in Rel-15.

## Measurement Restriction and Minimum Requirement for L1 Indication

The requirements of Rel-15 can be reused in IAB networks.

Proposal 3: Re-use the measurement restriction requirements and minimum requirements for L1 indication, that were defined for UEs in Rel-15, in IAB networks.

# Conclusion

Rel-16 IAB-MTs are fixed nodes. Once they lose their BFD-RS, they may have better understanding about the angle of arrival for the CBD-RS. But, IAB-MTs may need to use narrower RX beams to obtain higher link budget. Besides, the beam sweeping factor for fixed UEs (power class 1) was also decided to be 8 in Rel-15. Hence, same beam sweeping factor (N = 8) should be used to evaluate CBD-RS for IAB-MTs.

Observation 1: IAB-MTs may need to use narrower RX beams to obtain higher link budget.

Observation 2: the beam sweeping factor for fixed UEs (power class 1) was also decided to be 8 in Rel-15.

Proposal 1: For IAB CBD requirement, use beam sweeping factor N=8 in evaluation period calculation.

Proposal 2: For IAB BFD requirement, reuse the beam sweeping factors that were defined for UEs in Rel-15.

Proposal 3: Re-use the measurement restriction requirements and minimum requirements for L1 indication, that were defined for UEs in Rel-15, in IAB networks.

# References

- R4-2005318, WF on NR IAB RRM requirements.

- R4-2002233, WF on NR IAB RRM requirements.

- R4-2005322, WF on RLM requirements and sharing factor in RLM/BFD/CBD evaluation for IAB-MTs.