3GPP TSG-RAN WG4 Meeting # 95-e 														R4-2007579 Electronic Meeting, 25 May – 5 June, 2020

Agenda Item:	6.5.2.1.3

Source: 	Ericsson

Title:	TP to TR 38.809: Conducted RX spurious

Document for:	Approval

- Introduction

Section 2 discuss around the RX spurious requirement and Section 3 provides text proposal on chapter 8 for Rx spurious for the IAB TR 38.809.

# Disucssion

Accoring to WF [1], the RX spurious is agreed to follow the BS spec:

| Requirement | WF on FR2 IAB-MT | WF on FR1 IAB-MT |
| --- | --- | --- |
| OTA Receiver spurious emission | Reuse BS RX spurious emission requirement for IAB |  |

Text proposal is provided in section 3 to justify the above decision.

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

## 8.6	Receiver spurious emissions

### 8.6.1 IAB-MT Reciever spurious for IAB type 1-H

To proect the coexsitng service, the spurious level generated from receiver should be limited. For example, for the synchronized protected service, this could be the case of protecting IAB-MT receiving from IAB node#1 as shown in Figure 8.6-1. The RX spurious generated from IAB-MT reciver at IAB node #2 will impact on the UE receiving camped to the parent IAB node#1 cell but not on the UE camped to IAB-DU cell of IAB node#2. This is because IAB-DU and IAB-MT will be operated in TDM manner and thus there is no IAB-DU transmitting in IAB node#2 when IAB-MT receiver is enabled. Compared with the traditional coexisting distance of 5m for outdoor deployment for UE to UE interference analysis, the distance between IAB-MT to victim UE at parent IAB cell is larger as it will be installed at site similar to BS. Therefore, there is not necessary to reuse the UE Rx spurious requirement. The IAB-MT Rx spurious requirement in this case can be relaxed.

For the unsynchronized co-existing service, IAB-MT spurious level does not need to be tighter than IAB-DU receiver spurious requirement.

From protection of both synchronized or unsynchorinzed coexisting service, the IAB-DU receiver spurious for receiver spurious could be reused for IAB-MT.

Figure 8.6-1: coexisting service interference when IAB-MT receiving from parent IAB

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-2002483, WF on IAB RX RF requirement, Samsung

- 37.843, RF requirement background for AAS BS radiated requirements.