3GPP TSG-RAN WG4 Meeting # 95-e 	R4-2007610

Electronic Meeting, 25 May – 5 June, 2020

Source:	Ericsson, T-Mobile US, MediaTek

Title:	TP for TR 37.716-11-11 to correct MSD for DC_2_n46

Agenda item:			6.1.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-11-11 v1.2.0 to correct MSD for DC_2_n46.

# 2. Text Proposal

# ---Start of changes---

### 6.5.1	DC_2_n46

#### 6.5.1.1	Operating bands for DC

Table 6.5.1.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2_n46 | 2 | n46 | No |

#### 6.5.1.2	Configuration for DC

Table 6.5.1.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A_n46A | DC_2A_n46A | 2A | n46A |

#### 6.5.1.3	Maximum output power for DC

Table 6.5.1.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_2A_n46A | 23 | +2/-3 |

#### 6.5.1.4	Spurious emission band UE co-existence for DC

Does not need to be defined for this combination.

#### 6.5.1.5	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.5.1.5-1

Table 6.5.1.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 5150 | 5925 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 10300 | 11850 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 15450 | 17775 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 20600 | 23700 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 25750 | 29625 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 11100 | 11460 | 30900 | 35550 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 12950 | 13370 | 36050 | 41475 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4075 | 3240 | 7000 | 7835 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2225 | 1330 | 8390 | 10000 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8850 | 9745 | 12150 | 13760 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 8150 | 6480 | 14000 | 15670 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 375 | 580 | 13540 | 15925 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10700 | 11655 | 17300 | 19685 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 21850 | 18690 | 2490 | 1475 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 22450 | 25610 | 12550 | 13565 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 14075 | 11630 | 4570 | 6300 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19150 | 21595 | 15850 | 17580 |

Based on Table 6.5.1.5-1, it can be seen that

- 2rd order harmonic may fall into Rx frequencies of band 43 48 49, n77 and n78.

- 3rd order harmonic may fall into own Rx frequencies of band n46.

- 2nd order IMD products may fall into Rx frequencies of bands 22, 42, 43, 48, 49, n77 and n78.

- 3rd order IMD products may fall into Rx frequencies of bands 1, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 50, 51, 65, 66, 70, 74, 75 and 76 and also own Rx frequencies of band 2.

- 4th order IMD products may fall into Rx frequencies of bands 31 and 72.

- 5th order IMD products may fall into Rx frequencies of bands 1, 3, 4, 9, 10, 11, 21, 23, 24, 25, 30, 32, 33, 34, 35, 36, 37, 39, 40, 47, 50, 65, 66, 70, 74, 75, n79 and n258 and own Rx frequencies of band 2 and n46.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.5.1.5-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.5.1.5-2: Band 2 and Band n46 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 3rd, 5th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 3rd, 5th IMD |
| GLONASS | 1591 | - | 1610 | Yes |  | 3rd, 5th IMD |
| GPS | 1563 | - | 1587 | Yes |  | 3rd, 5th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes |  | 5th IMD |
|  | 2400 | - | 2494 | Yes |  | 5th IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 5th IMD 3rd harmonics |
|  | 5150 | - | 5350 | Yes | Europe | 5th IMD |
|  | 5470 | - | 5725 | Yes |  | 5th IMD 3rd harmonics |
|  | 5150 | - | 5825 | Yes | Asia | 5th IMD 3rd harmonics |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

#### 6.5.1.6	∆TIB and ∆RIB values

For DC_2_n46, the TIB,c and RIB,c values are derived from LTE combination CA_2-46 and are given in the tables below.

Table 6.5.1.6-1: ΔTIB,c

| EN-DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n46 | 2 | 0 |
|  | n46 | 0 |

Table 6.5.1.6-2: ΔRIB,c

| EN-DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2_n46 | 2 | 0 |
|  | n46 | 0 |

#### 6.5.1.7	MSD

MSD for DC_2_n46 is proposed to be defined like in table below, where Note 1 in Table 6.5.1.7-1 is derived from Note 5 in TS 36.101 Table 7.3.1A-0eA and Note 2 in Table 6.5.1.7-1 is derived from Note 16 in TS36.101 in Table 7.3.1A-0a. The Table 6.5.1.7-2 is derived from TS36.101 Table 7.3.1A-0eC.

Table 6.5.1.7-1: Reference sensitivity QPSK PREFSENS (EN-DC with n46)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 2 | n46 |  |  |  | N/A |  | N/A |  | N/A | N/A |  |  |
| n46 | 2 | 28 | 28 | 28 | 28 |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the downlink (victim) transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 6.x.1.7-2 and Table 6.x.1.7-3. NOTE 2: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the downlink transmission bandwidth of a victim (lower) band. NOTE 3: The requirements for a victim (lower) band apply for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz with the DL carrier frequency in the lower band and the UL carrier frequency in the higher band, both in MHz. . |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5.1.7-2: n46 Reference sensitivity measurement exclusion region in MHz

| Licensed Component Carriers / E-UTRA Band / Harmonic order / Channel BW in UL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Band | Harmonic order | 5MHz | 10MHz | 15MHz | 20MHz |
| 2 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| NOTE 1: Even though UL harmonic does not fall directly into n46 the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the uplink channel center frequency by the harmonic order. |  |  |  |  |  |

Table 6.5.1.7-3: Downlink Reference sensitivity measurement exclusion region in MHz for harmonic mixing when paring with n46 uplink

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n46 | 2 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |

Table 6.5.1.7-4: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_2A_n46A | 2 | 1880 | 5 | 25 | 1960 | 12.0 | FDD TDD | IMD3 |
|  | n46 | 5720 | 20 | 100 | 5720 | N/A |  | N/A |

---End of changes---