3GPP TSG-RAN WG4 Meeting # 95-e 	R4-2007616

Electronic Meeting, 25 May – 5 June, 2020

Source:	Ericsson, AT&T

Title:	TP for TR 37.716-21-11: Including band combinations 14-66_n2

Agenda item:			8.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.716-21-11 to add DC_14A-66A_n2A and 14A-66A-66A-n2A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_14-66_n2

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 14-66_n2 | CA_14-66 | n2 |  |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_14A-66A_n2A | DC_14A_n2A DC_66A_n2A | CA_14A-66A | n2A |
| DC_14A-66A-66A_n2A | DC_14A_n2A DC_66A_n2A | CA_14A-66A-66A | n2A |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_14-66_n2, the TIB,c and RIB,c values are reused from LTE combination CA_2-14-66, and are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-66_n2 DC_14-66-66_n2 | 14 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-66_n2 DC_14-66-66_n2 | 14 | 0 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

### 5.1.x.4	REFSENS requirements

There is IMD4 impact from DC_14_n2 UL affecting band 66 DL. MSD value is derived from CA_2-4-13 which have IMD4 impact to band 4 from UL_2_13.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_14A-66A_n2A DC_14A-66A-66A_n2A | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1762 | 5 | 25 | 2162 | 7.6 | IMD4 |
|  | n2 | 1874 | 5 | 25 | 1954 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-200271, “Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)” Huawei, HiSilicon