3GPP TSG-RAN WG4 Meeting # 95-e 	R4-2007618

Electronic Meeting, 25 May – 5 June, 2020

Source:	Ericsson, AT&T

Title:	TP for TR 37.716-21-11: Including band combinations 14-66_n66

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.716-21-11 to add DC_14A-66A_n66A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_14-66_n66

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 14-66_n66 | CA_14-66 | n66 |  |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_14A-66A_n66A | DC_14A_n66A DC_66A_n66A1 | CA_14A-66A | n66 |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_14-66_n66, the TIB,c and RIB,c values are reused from LTE combination CA_14-66, and are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-66_n66 | 14 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-66_n66 | 14 | 0 |
|  | 66 | 0 |
|  | n66 | 0 |

### 5.1.x.4	REFSENS requirements

There is no additional requirement for this band combination.

---End of changes---