3GPP TSG-RAN WG4 Meeting # 95-e 	R4-2007622

Electronic Meeting, 25 May – 5 June, 2020

Source:	Ericsson, AT&T

Title:	TP for TR 37.716-31-11: Addition of EN-DC configurations containing bands DC_2-14-66_n2.

Agenda item:	9.5.2

Document for:	Approval

# Background

This contribution provides text proposal to TR 37.716-31-11 for addition of 2A-14A-66A-n2A and 2A-14A-66A-66A-n2A as defined in [1].

# Text Proposal

##### ---Start of changes---

## 5.1.x	DC_2A-14A-66A_n2A

#### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-14-66_n2 | CA_2-14-66 | n2 |  |

#### 5.1.x.2	Configuration for DC

Table 5.1.x.2-1:  Inter-band EN-DC configurations of 3 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-14A_66A_n2A DC_2A-14A_66A-66A_n2A | DC_2A_n2A1 DC_14A_n2A DC_66A_n2A | CA_2A-14A-66A CA_2A-14A-66A-66A | n2A |
| NOTE 1: Only single switched UL is supported |  |  |  |

#### 5.1.x.3	∆TIB and ∆RIB values

For DC_2-14-66_n2 the TIB,c and RIB,c values are reused from CA_2-14-66 and are given in the tables below.

Table 5.1.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-14-66_n2 DC_2-14-66-66_n2 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.x.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-14-66_n2 DC_2-14-66-66_n2 | 2 | 0.3 |
|  | 14 | 0 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

### 5.1.x.4	REFSENS requirements

No further MSD are needed to be specified.

##### ---End of changes---

# Reference

[1]	RP-200102, “Revised WID on Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL)”, Ericsson