3GPP TSG-RAN WG4 Meeting #95-e	R4-2007642

# Electronic Meeting, 25th May – 5th June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0740 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 on interruption requirements for CGI reading |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh-Core |  |  |  |  |  | Date: |  |  | 2020-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In WID RP-191601, CGI reading requirements with autonomous gap for NR capable UE should be specified for various scenarios. - CGI reading for NR cell (NR capable UE in LTE SA, NR SA UE, EN-DC UE, NE-DC UE, and NR-DC) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Interruption requirements for CGI identification of NR cell are introduced for EN-DC, NR SA, NE-DC and NR-DC. - Interruption requirements for CGI identification of LTE cell are introduced for EN-DC and NE-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No interruption requirments for CGI identification of an NR cell. No interruption requirments for CGI identification of an LTE cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2.8, 8.2.2.2.7, 8.2.3.2.8, 8.2.4.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change #1 >

8.2	Interruption

8.2.1	EN-DC Interruption

8.2.1.1	Introduction

This clause contains the requirements related to the interruptions on PSCell, and SCell, when

E-UTRA PCell transitions between active and non-active during DRX, or

E-UTRA PCell transitions from non-DRX to DRX, or

E-UTRA SCell in MCG or SCell in SCG is added or released, or

E-UTRA SCell in MCG or SCell in SCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either E-UTRA MCG or NR SCG, or

a supplementary UL carrier or an UL carrier is configured or de-configured, or

UL/DL BWP is switched on PSCell or SCell in SCG, or

CGI reading of an NR neighbour cell with autonomous gaps, or

CGI reading of an E-UTRA neighbour cell with autonomous gaps.

The requirements shall apply for E-UTRA-NR DC with an E-UTRA PCell.

This clause contains interruptions where victim cell is PSCell or SCell belonging to SCG. Requirements for interruptions requirements when the victim cell is E-UTRA PCell or E-UTRA SCell belonging to MCG are specified in TS 36.133 [15].

For a UE which does not support per-FR measurement gaps, interruptions to the PSCell or activated SCG SCells may be caused by EUTRA PCell, EUTRA SCells or SCells on any frequency range. For UE which support per-FR gaps, interruptions to the PSCell or activated SCG SCells may be caused by EUTRA PCell, EUTRA SCells or SCells on the same frequency range as the victim cell.

8.2.1.2	Requirements

< Unchanged text omitted >

### 8.2.1.2.X	 Interruptions when identifying CGI of an NR cell with autonomous gaps

When a UE is identifying CGI of an NR cell with autonomous gaps, the UE is allowed interruptions on PCell, PSCell or any activated SCell:

-	with up to 6 interruptions with interrupted slots up to interruption length X1 specified in Table 8.2.1.2.X-1 for each interruption during MIB decoding time period TMIB_intra specified in clause [9.2.7],  or TMIB_inter specified in clause [9.3.9].

-	with up to 6 interruptions with interrupted slots up to interruption length Y1 specified in Table 8.2.1.2.X-1 during SIB1 decoding time period TSIB1_intra specified in clause [9.2.7] or TSIB1_inter specified in clause [9.3.9], for SSB and CORESET for RMSI scheduling multiplexing patterns 1.

-	with up to 6 interruptions with interrupted slots up to interruption length Y2 specified in Table 8.2.1.2.X-1 during SIB1 decoding time period TSIB1_intra specified in clause [9.2.7] or TSIB1_inter specified in clause [9.3.9], for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3.

Table 8.2.1.2.X-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | [4] | [3] |
| 1 | 0.5 | 6 | [5] | [4] |
| 2 | 0.25 Note1 | 6 | [7] | [6] |
|  | 0.25 Note2 | 6 | [5] | [4] |
| 3 | 0.125 | 6 | [7] | [6] |
| NOTE 1: Victim cell is on FR1. NOTE 2: Victim cell is on FR2. |  |  |  |  |

### 8.2.1.2.Y	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps

When a UE is identifying CGI of an E-UTRA FDD cell or E-UTRA FDD cell with autonomous gaps, within time period [Tidentify_CGI_interRAT] in clause TBD in TS36.133 [15], the UE shall be able to transmit at least the number of ACK/NACKs specified in Table 8.2.1.2.Y-1 on PCell, PSCell or any activated SCell in the frequency range where autonomous gaps are used, provided that:

-	there is continuous DL data allocation,

-	no DRX cycle is used,

-	no measurement gaps are configured,

-	only one code word is transmitted in each slot,

-	2 slot ACK/NACK feedback is configured,

-	20 ms SMTC period is configured.

Table 8.2.1.2.Y-1: Minimum number of ACK/NACKs transmitted by the UE during [Tidentify_CGI_interRAT]

| Minimum number of transmitted ACK/NACKs | Configuration of the serving cell in which the transmitted ACK/NACKs are counted |  |
| --- | --- | --- |
|  | Duplex mode configuration | SCS |
| [84] | FDD | 15 kHz |
| [193] | FDD | 30 kHz |
| [402] | FDD | 60 kHz |
| [28] | TDD Note 1 | 15 kHz |
| [81] | TDD Note 1 | 30 kHz |
| [159] | TDD Note 1 | 60 kHz |
| [233] | TDD Note 2 | 60 kHz |
| [491] | TDD Note 2 | 120 kHz |
| NOTE 1: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-1 [18]. NOTE 2: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-2 [19]. |  |  |

< End of change #1 >

< Start of change #2 >

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation

8.2.2.1	Introduction

This clause contains the requirements related to the interruptions on PCell and activated SCell if configured, when

up to 7 SCells are configured, de-configured, activated or deactivated, or

a supplementary UL carrier or an UL carrier is configured or de-configured, or

measurements on SCC with deactivated SCell in NR SCG, or

UL/DL BWP is switched on PCell or SCell, or

CGI reading of an NR neighbour cell with autonomous gaps.

CGI reading of an E-UTRA neighbour cell with autonomous gaps.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

Editor’s Note:	The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition/release or MAC control signalling [17] for SCell activation/deactivation command.

This clause additionally contains requirements related to interruptions at inter-frequency SFTD between PCell in FR1 and neighbour cell in FR2.

For a UE which does not support per-FR measurement gaps, interruptions to the PCell and activated SCell may be caused by SCells on any frequency range. For UE which support per-FR gaps, interruptions to PCell and activated SCell may be caused by SCells on the same frequency range as the victim cell.

In addition to standalone NR carrier aggregation when no CCA is configured, the requirements in clause 8.2.2. and all subclauses of 8.2.2 apply when the UE is configured with

-A PCell not using CCA in downlink and one or more SCells using CCA in downlink  or

-A PCell and one or more SCells using CCA in downlink

8.2.2.2	Requirements

< Unchanged text omitted >

### 8.2.2.2.X	 Interruptions when identifying CGI of an NR cell with autonomous gaps

When a UE is identifying CGI of an NR cell with autonomous gaps, the UE is allowed interruptions on PCell, PSCell or any activated SCell:

-	with up to 6 interruptions with interrupted slots up to interruption length X1 specified in Table 8.2.2.2.X-1 for each interruption during MIB decoding time period TMIB_intra specified in clause [9.2.7],  or TMIB_inter specified in clause [9.3.9].

-	with up to 6 interruptions with interrupted slots up to interruption length Y1 specified in Table 8.2.2.2.X-1 during SIB1 decoding time period TSIB1_intra specified in clause [9.2.7] or TSIB1_inter specified in clause [9.3.9], for SSB and CORESET for RMSI scheduling multiplexing patterns 1.

-	with up to 6 interruptions with interrupted slots up to interruption length Y2 specified in Table 8.2.2.2.X-1 during SIB1 decoding time period TSIB1_intra specified in clause [9.2.7] or TSIB1_inter specified in clause [9.3.9], for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3.

Table 8.2.2.2.X-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | [4] | [3] |
| 1 | 0.5 | 6 | [5] | [4] |
| 2 | 0.25 Note1 | 6 | [7] | [6] |
|  | 0.25 Note2 | 6 | [5] | [4] |
| 3 | 0.125 | 6 | [7] | [6] |
| NOTE 1: Victim cell is on FR1. NOTE 2: Victim cell is on FR2. |  |  |  |  |

### 8.2.2.2.Y	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps

When a UE is identifying CGI of an E-UTRA FDD cell or E-UTRA FDD cell with autonomous gaps, within time period [Tidentify_CGI_interRAT] in clause TBD in TS36.133 [15], the UE shall be able to transmit at least the number of ACK/NACKs specified in Table 8.2.2.2.Y-1 on PCell, PSCell or any activated SCell in the frequency range where autonomous gaps are used, provided that:

-	there is continuous DL data allocation,

-	no DRX cycle is used,

-	no measurement gaps are configured,

-	only one code word is transmitted in each slot,

-	2 slot ACK/NACK feedback is configured,

-	20 ms SMTC period is configured.

Table 8.2.2.2.Y-1: Minimum number of ACK/NACKs transmitted by the UE during [Tidentify_CGI_interRAT]

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 3 | 4 | 3 |
| 1 | 0.5 | 4 | 5 | 4 |
| 2 | 0.25 Note1 | 6 | 7 | 6 |
|  | 0.25 Note2 | 4 | 5 | 4 |
| 3 | 0.125 | 6 | 7 | 6 |
| NOTE 1: Victim cell is on FR1. NOTE 2: Victim cell is on FR2. |  |  |  |  |

< End of change #2 >

< Start of change #3 >

8.2.3	NE-DC Interruptions

8.2.3.1	Introduction

This clause contains the requirements related to the interruptions on PCell and SCell, when

E-UTRA PSCell transitions between active and non-active during DRX, or

E-UTRA PSCell transitions from non-DRX to DRX, or

E-UTRA PSCell/SCell in SCG or SCell in MCG is added or released, or

E-UTRA PSCell/SCell in SCG or SCell in MCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either E-UTRA SCG or NR MCG or

PUSCH/PUCCH carrier configuration and deconfiguration in NR MCG, or

UL/DL BWP is switched on PCell or SCell in MCG, or

CGI reading of an NR neighbour cell with autonomous gaps, or

CGI reading of an E-UTRA neighbour cell with autonomous gaps.

The requirements shall apply for NE-DC with an NR PCell.

This clause contains interruptions where victim cell is PCell or SCell belonging to MCG. Requirements for interruptions requirements when the victim cell is E-UTRA PSCell or E-UTRA SCell belonging to SCG are specified in TS 36.133 [15].

For a UE which does not support per-FR measurement gaps, interruptions to the PCell, E-UTRA PSCell or activated MCG SCells may be caused by EUTRA PSCell, EUTRA SCells or SCells on any frequency range. For UE which support per-FR gaps, interruptions to the PCell, E-UTRA PSCell or activated MCG SCells may be caused by EUTRA PSCell, EUTRA SCells or SCells on the same frequency range as the victim cell.

8.2.3.2	Requirements

< Unchanged text omitted >

### 8.2.3.2.X	 Interruptions when identifying CGI of an NR cell with autonomous gaps

When a UE is identifying CGI of an NR cell with autonomous gaps, the UE is allowed interruptions on PCell, PSCell or any activated SCell:

-	with up to 6 interruptions with interrupted slots up to interruption length X1 specified in Table 8.2.3.2.X-1 for each interruption during MIB decoding time period TMIB_intra specified in clause [9.2.7],  or TMIB_inter specified in clause [9.3.9].

-	with up to 6 interruptions with interrupted slots up to interruption length Y1 specified in Table 8.2.3.2.X-1 during SIB1 decoding time period TSIB1_intra specified in clause [9.2.7] or TSIB1_inter specified in clause [9.3.9], for SSB and CORESET for RMSI scheduling multiplexing patterns 1.

-	with up to 6 interruptions with interrupted slots up to interruption length Y2 specified in Table 8.2.3.2.X-1 during SIB1 decoding time period TSIB1_intra specified in clause [9.2.7] or TSIB1_inter specified in clause [9.3.9], for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3.

Table 8.2.3.2.X-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | [4] | [3] |
| 1 | 0.5 | 6 | [5] | [4] |
| 2 | 0.25 Note1 | 6 | [7] | [6] |
|  | 0.25 Note2 | 6 | [5] | [4] |
| 3 | 0.125 | 6 | [7] | [6] |
| NOTE 1: Victim cell is on FR1. NOTE 2: Victim cell is on FR2. |  |  |  |  |

### 8.2.3.2.Y	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps

When a UE is identifying CGI of an E-UTRA FDD cell or E-UTRA FDD cell with autonomous gaps, within time period [Tidentify_CGI_interRAT] in clause TBD in TS36.133 [15], the UE shall be able to transmit at least the number of ACK/NACKs specified in Table 8.2.3.2.Y-1 on PCell, PSCell or any activated SCell in the frequency range where autonomous gaps are used, provided that:

-	there is continuous DL data allocation,

-	no DRX cycle is used,

-	no measurement gaps are configured,

-	only one code word is transmitted in each slot,

-	2 slot ACK/NACK feedback is configured,

-	20 ms SMTC period is configured.

Table 8.2.3.2.Y-1: Minimum number of ACK/NACKs transmitted by the UE during [Tidentify_CGI_interRAT]

| Minimum number of transmitted ACK/NACKs | Configuration of the serving cell in which the transmitted ACK/NACKs are counted |  |
| --- | --- | --- |
|  | Duplex mode configuration | SCS |
| [84] | FDD | 15 kHz |
| [193] | FDD | 30 kHz |
| [402] | FDD | 60 kHz |
| [28] | TDD Note 1 | 15 kHz |
| [81] | TDD Note 1 | 30 kHz |
| [159] | TDD Note 1 | 60 kHz |
| [233] | TDD Note 2 | 60 kHz |
| [491] | TDD Note 2 | 120 kHz |
| NOTE 1: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-1 [18]. NOTE 2: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-2 [19]. |  |  |

< End of change #3 >

< Start of change #4 >

8.2.4	NR-DC: Interruptions

8.2.4.1	Introduction

This clause contains the requirements related to the interruptions on PCell, PSCell and activated SCell if configured, when

up to TBD SCells are configured, deconfigured, activated or deactivated or,

a supplementary UL carrier or an UL carrier is configured or de-configured, or

measurements on SCC with deactivated SCell in NR SCG, or

UL/DL BWP is switched on PCell, PSCell or SCell.

transitions between active and non-active during DRX, or

transitions from non-DRX to DRX, or

CGI reading of an NR neighbour cell with autonomous gaps.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

Editor’s Note:	The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition/release or MAC control signalling [17] for SCell activation/deactivation command. How to specify this is FFS.

The requirements shall apply for NR-DC with an NR PCell, PSCell or SCell.

For a UE which does not support per-FR measurement gaps, interruptions to the PCell and activated SCell may be caused by SCells on any frequency range. For UE which support per-FR gaps, interruptions to PCell, PSCell and activated SCell may be caused by SCells on the same frequency range as the victim cell.

8.2.4.2	Requirements

< Unchanged text omitted >

### 8.2.4.2.X	 Interruptions when identifying CGI of an NR cell with autonomous gaps

When a UE is identifying CGI of an NR cell with autonomous gaps, the UE is allowed interruptions on PCell, PSCell or any activated SCell:

-	with up to 6 interruptions with interrupted slots up to interruption length X1 specified in Table 8.2.4.2.X-1 for each interruption during MIB decoding time period TMIB_intra specified in clause [9.2.7],  or TMIB_inter specified in clause [9.3.9].

-	with up to 6 interruptions with interrupted slots up to interruption length Y1 specified in Table 8.2.4.2.X-1 during SIB1 decoding time period TSIB1_intra specified in clause [9.2.7] or TSIB1_inter specified in clause [9.3.9], for SSB and CORESET for RMSI scheduling multiplexing patterns 1.

-	with up to 6 interruptions with interrupted slots up to interruption length Y2 specified in Table 8.2.4.2.X-1 during SIB1 decoding time period TSIB1_intra specified in clause [9.2.7] or TSIB1_inter specified in clause [9.3.9], for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3.

Table 8.2.4.2.X-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | [4] | [3] |
| 1 | 0.5 | 6 | [5] | [4] |
| 2 | 0.25 Note1 | 6 | [7] | [6] |
|  | 0.25 Note2 | 6 | [5] | [4] |
| 3 | 0.125 | 6 | [7] | [6] |
| NOTE 1: Victim cell is on FR1. NOTE 2: Victim cell is on FR2. |  |  |  |  |

### 8.2.4.2.Y	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps

When a UE is identifying CGI of an E-UTRA FDD cell or E-UTRA FDD cell with autonomous gaps, within time period [Tidentify_CGI_interRAT] in clause TBD in TS36.133 [15], the UE shall be able to transmit at least the number of ACK/NACKs specified in Table 8.2.4.2.Y-1 on PCell, PSCell or any activated SCell in the frequency range where autonomous gaps are used, provided that:

-	there is continuous DL data allocation,

-	no DRX cycle is used,

-	no measurement gaps are configured,

-	only one code word is transmitted in each slot,

-	2 slot ACK/NACK feedback is configured,

-	20 ms SMTC period is configured.

Table 8.2.4.2.Y-1: Minimum number of ACK/NACKs transmitted by the UE during [Tidentify_CGI_interRAT]

| Minimum number of transmitted ACK/NACKs | Configuration of the serving cell in which the transmitted ACK/NACKs are counted |  |
| --- | --- | --- |
|  | Duplex mode configuration | SCS |
| [84] | FDD | 15 kHz |
| [193] | FDD | 30 kHz |
| [402] | FDD | 60 kHz |
| [28] | TDD Note 1 | 15 kHz |
| [81] | TDD Note 1 | 30 kHz |
| [159] | TDD Note 1 | 60 kHz |
| [233] | TDD Note 2 | 60 kHz |
| [491] | TDD Note 2 | 120 kHz |
| NOTE 1: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-1 [18]. NOTE 2: TDD UL-DL configuration is as specified in Table A.3.3.1-1 of TS 38.101-2 [19]. |  |  |

< End of change #4 >