3GPP TSG-RAN WG4 Meeting #95-e	R4-2007649

Electronic Meeting, 25 May – 5 June, 2020

Source:	ZTE

Title:	Further discussion on configuration of CSI-RS based L3 measurement

Agenda item:	6.16.1.1

Document for:	Discussion

1. Introduction

In the RAN4#94-e-bis meeting, there were discussions on bandwidth and density configuration for defining CSI-RS based measurement requirements. It was agreed only define one set of RRM requirements for CSI-RS based L3 measurement. However whether to define requirements for {D=1 with PRBs ≥ 96} are still FFS. It is captured in WF [1].

- Define requirements for CSI-RS configuration with {D=3 with PRBs ≥ 48}. FFS whether to define requirements for {D=1 with PRBs ≥ 96}. - Not define additional CSI-RS based measurement requirement

In this contribution we further provide our views on the bandwidth and density for defining CSI-RS based RRM measurement requirements.

2. Discussion

It is important that density 1 is supported from RRM requirements perspective. When larger BW is interested, e.g. PRBs ≥ 96, the overhead is very high when CSI-RS resources are configured with density 3. The similar measurement accuracy can be achieved for both density 1 and density 3 is bandwidth is larger enough.

Moreover the CSI-RS resources configured for L3 measurements will be transmitted periodically. Since no CSI-IM resources are available to be configured for CSI-RS based L3 measurement, the periodical CSI-RS will be consistent interference to PDSCH channel which will highly degrade the system performance. Therefore it is important to reduce the overhead of CSI-RS based L3 measurement.

It was agreed only define one set of RRM requirements for CSI-RS based L3 measurement. However whether to define requirements for {D=1 with PRBs ≥ 96}

Observation 1: Density 1 should be supported from RRM requirements perspective.

Then as one set requirements are specified for both density 1 and density 3 CSI-RS resource configuration, further study is needed to define the delay and accuracy requirements.

Proposal 1: Define one set of requirements for the cases of both 48 PRBs with density 3 and 96 PRBs with density 1.

3. Conclusion

In this contribution, we further provided our views on the bandwidth and density for defining CSI-RS based RRM measurement requirements. Based on the observations following proposals are present.

Proposal 1: Define one set of requirements for the cases of both 48 PRBs with density 3 and 96 PRBs with density 1.

4. References

- R4-2005355	WF on CSI-RS configuration and intra/inter-frequency measurements definition for CSI-RS based L3 measurement, CATT